3GPP TSG RAN WG1 #109-e								        R1-2204049

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.4.2

Source:	InterDigital, Inc.

Title:	Co-channel coexistence for LTE sidelink and NR sidelink

Document for:	Discussion and Decision

# Introduction

R18 SL evolution WI was approved in RAN plenary #94 and one of the objectives is to study and specify solutions for co-channel coexistence between LTE and NR sidelink [1]. It is not expected that the solutions will impact LTE V2X specifications.

Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible

In this contribution, we discuss our views on the potential solutions for co-channel coexistence.

# Discussion

## Not-co-channel and Co-channel LTE and NR SL coexistence

During R16 V2X SI, LTE and NR SL coexistence was discussed and not-co-channel in-device coexistence was identified as a WI objective. In a not-co-channel coexistence scenario, LTE and NR SL coexist in different channels, e.g., adjacent channels or channels separated sufficiently far away from each other. In this scenario, a LTE SL transmission and a NR SL transmission can overlap in time domain and simultaneous transmissions can result in unwanted spurious emissions. Also, a LTE SL transmission also can interfere with a NR SL reception and vice versa [2].

The solutions adopted in R16 NR SL include e.g., long-term and short-term TDM. In the long-term TDM solution, dedicated resource pools are configured for LTE SL and NR SL, e.g., resource pools non-overlapping in time domain. In the short-term TDM solution, when an overlapping occurs, a UE determines dynamically which of LTE and NR SL to operate based on the priority information available to the UE T ms prior to the start of the earliest transmission or reception as specified in TS 38.213 [3]. The time period (T) is defined as less than 4 ms and based on UE implementation.

In R18 SL evolution WI planning phase, co-channel coexistence between LTE and NR SL was recognized as an important objective by operators and OEMs, which can be used to enable spectrum sharing and maximum deployment flexibility.

In co-channel coexistence scenario, a LTE SL transmission and NR SL transmission can overlap in both time and frequency domain. For example, UEs can be (pre)configured with overlapping resources in a LTE and NR SL resource pool. A UE performing a NR SL resource selection may not possess any information regarding which NR SL resource overlaps with a LTE SL resource and whether this LTE SL resource is reserved for a LTE SL transmission. As a result, this UE can select a NR SL resource which overlaps with a LTE SL resource reserved by another UE for LTE SL transmission, which will cause a collision. The R16 NR SL short-term TDM solution can be used to address co-channel in-device coexistence, but not the collision due to the incoordination between LTE and NR SL resource selection.

Observation 1: R16 NR SL short-term TDM does not address collisions between LTE and NR SL transmissions in co-channel coexistence scenario.

## Resource selection coordination between LTE and NR SL

It is thus desirable that the solution for R18 co-channel coexistence address both in-device and collision issues. As indicated in TS 38.213, a UE determines whether a LTE SL and NR SL transmission or reception overlap in time and the priority information only T ms prior to the start of the earliest transmission. In our view, a resource selection coordination between LTE and NR SL can be studied to avoid such overlapping situation in the first place.

One option for this coordination can be NR SL resource (re)selection including LTE SL sensing information. We think it is conceivable a UE is able to perform LTE SL sensing for a triggered NR SL resource selection. Whether or not a UE is able to include LTE SL sensing information in a NR SL resource selection within the defined processing time (e.g., T1) can be considered as a UE capability.

For a UE with such capability, the LTE SL resources reserved for the UE’s own LTE SL transmission and reception or other UEs’ LTE SL transmissions (based on LTE SL sensing) can be excluded prior to the processing of NR SL sensing information. In case the number of NR SL resources after exclusion of LTE SL reserved resources is below a threshold, the UE can request another NR SL resource pool for the transmission. We think further discussions are required to develop details on such a NR SL resource (re)selection procedure including LTE SL sensing information.

Proposal 1: Study NR SL resource (re)selection including information based on

    - UE’s scheduled LTE SL transmission and reception

    - LTE SL sensing performed by the UE

Another option can be based on the R17 IUC framework. In IUC Scheme 2, a UE can indicate a potential/expected conflict in a PSFCH due to the overlapping between two NR SL reservations. If LTE SL sensing information is available in determining such a conflict, a UE can indicate a conflict between a reserved NR SL resource and LTE SL resource. Since no impact is expected on LTE SL, in this case, the UE transmits a PSFCH to the UE reserving the NR SL resource. The indication procedure may consider further details, e.g., a LTE SL single-subframe resource can overlap one or more NR SL single-slot resource depending on the NR SL numerology and should be studied more closely.

Proposal 2: Study NR SL IUC conflict indication based on overlapping LTE SL and NR SL resource reservation.

# Conclusion

In this contribution, we provided our views on the potential solutions for co-channel coexistence. Our proposals and observation are as follows:

Observation 1: R16 NR SL short-term TDM does not address collisions between LTE and NR SL transmissions in co-channel coexistence scenario.

Proposal 1: Study NR SL resource (re)selection including information based on

    - UE’s scheduled LTE SL transmission and reception

    - LTE SL sensing performed by the UE

Proposal 2: Study NR SL IUC conflict indication based on overlapping LTE SL and NR SL resource reservation.

# References

- RP-220300, “WID revision: NR sidelink evolution”, Oppo, 3GPP TSG RAN meeting #95e, March, 2022

- TR 38.889, “Study on NR-based access to unlicensed spectrum”, December 2018

- TS 38.213, “Physical layer procedure for control”, March 2022