3GPP TSG-RAN WG1 #109-e	R1-2204050

e-Meeting, May 9th - 20th, 2022

Agenda Item:	9.2.2.1

Source:	InterDigital, Inc.

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#94e, the Rel-18 study item (SI) [1] on AI/ML for NR Air Interface was approved.

In particular, the SI description (SID) includes the “study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.”

One objective of the SI from the perspective of RAN1 is to focus on an initial set of use cases, such as CSI feedback enhancement, Beam management and Positioning accuracy enhancement, and finalize representative sub-use cases for each use case, to characterize and provide baseline performance evaluations.

This contribution discusses evaluation aspects for CSI feedback enhancement and provides simulation results for auto-encoder based CSI compression approaches. The evaluations are performed for a variety of configurations (such as training datasets, channel models, delay spread, and Doppler) and comparisons are provided to the CSI Type I baseline, as well as the ideal SVD.

# Autoencoder (AE) based CSI Compression for NR

This section provides details on the dataset generation, ML model assumptions and training, as well as link level simulation results for the ML-based CSI compression under the selected simulation configurations.

## Dataset generation

This section describes the synthetic datasets used for model training and provides the respective parameter configuration.

To assess how the AE model for the CSI compression generalizes, several scenarios are considered for the training datasets, as follows:

- Single-channel synthetic training dataset (CDL-A). The ML model trained with the single-channel CDL-A data will be referred to as “Model-CDLA” in this contribution.

- Multi-channel synthetic training dataset (combination of CDL-A, CDL-B, CDL-C). The ML model trained with this multi-channel combination will be referred to as “Model-Mixed”

Table 1 Common parameters of the training dataset

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| BWP | 52 RBs |
| Subcarrier spacing | 30 kHz |
| Number of transmit antennas (Nt) | 8 |
| Number of receive antennas (Nr) | 2 |
| UE speed | [3-30] km/hr |
| Channel estimation | Ideal |
| Number of Drops | 120 |
| Number of Slots | 100 |

Table 2 Specific parameters for the training datasets

| Parameter | Model-CDLA | Model-Mixed |
| --- | --- | --- |
| Channel model | CDL-A only | CDL-A, CDL-B, CDL-C |
| Delay spread | 30 ns | [30 (CDL-A) 300 (CDL-B, CDL-C) ns |
| Number of samples | 120 drops / 100 slots each | 120 drops/channel model 100 slots/drop |

In all the generated datasets, the speed per drop was drawn at random between 3 and 30 km/hr, which in turn yields a Doppler range between 5 and 55 Hz approximately, within the same dataset. The dimension of each generated data sample is Nsc×Nsymbols×Nr×Nt, where the number of subcarriers (Nsc) is 624, the number of OFDM symbols (Nsymbols) is 14, and Nr and Nt are as specified in Table 1.

## Model assumption and training

A CSI-Net [2] based autoencoder model has been utilized where both the encoder and decoder consist of convolutional, fully connected and batch normalization layers.

- Preprocessing

To effectively compare the performance of the existing CSI feedback and precoding methods with the deep learning-based methods, we utilize the following pre-processing strategy:

  - Since the implementation of CSI Type-I method computes a single precoding matrix by averaging over a set of RBs and across the entire time slot, we follow a similar strategy and average the channel matrix across 2 resource blocks and across 1 time slot. Therefore, if the size of the raw channel is 624x14x2x8, representing 52 RBs, 14 OFDM symbols in 1 time slot and Nt = 8 and Nr = 2, The pre-processed channel will have a dimension of 26x1x2x8.

  - Additionally, we normalize the channels to [0,1] range as mentioned in [2].

- Quantization

The output of the encoder is quantized using a uniform quantizer. The encoder output is passed through a tanh layer to restrict the range of the encoder output for uniform quantization. The quantization operation is included during the training so that the encoder and decoder can learn appropriate weights while taking into account the quantization impact.

- Model Information

  - Base Model: For our evaluation we use the CSI-Net [2] autoencoder model. The model has 3 main functional blocks. Encoder block, Quantization, Decoder block

    - Encoder architecture : CNN->BN->FC

    - Quantizer : Linear quantization

    - Decoder architecture: FC -> RN->RN->CNN

    - Where.

      - FC: Fully connected layer,

      - CNN: Convolutional Neural Network

      - BN: Batch Normalization

      - RN: CNN->BN->CNN->BN->CNN->BN with a skip connection from the input to the RN block

  - Training dataset

    - For the 3 datasets scenarios discussed in the previous section, 3 different models are trained.

    - For each dataset, 80% of the dataset is utilized for training and 20 % is utilized for validation.

  - Loss function

    - We utilize the mean squared error loss function for training. The mean squared error is calculated between the input to the encoder and the output of the decoder.

  - Additional information:

    - Optimizer: ADAM with learning rate = 0.0001

# Evaluation results of AE based CSI compression

### Simulation assumptions

This section describes the simulation assumptions used to assess the ML models' performance. For CDL-A only data, three different separately-trained models with feedback sizes 64 bits, 32 bits, and 16 bits are imported and evaluated in the test bench. For the mixed data (CDL-A, CDL-B, CDL-C), two different models with feedback sizes 64 bits and 32 bits are evaluated. For all of the aforementioned models, the encoder and decoder networks are imported separately. The encoder network is placed under the receiver (UE side) block to compress the channel while the decoder network is placed under the transmitter (gNB side) block to decompress the channel. As mentioned in Section 2.2, the sub-band size associated with the ML approach is 24 sub-carries (2 RBs). In particular, the decompressed channel at the gNB is of size 26x2x8, where the first dimension represents the number of subbands, where the 2x8 channel associated with each sub-band is obtained through averaging out the channel coefficients across 24 sub-carries (2 RBs).

The parameters used for the considered suite of simulations are summarized in Table 3. The MCS index is kept fixed throughout the simulations. The sub-band size associated with the considered baselines is set to 4 RBs. This is different from the sub-band size used by the ML approach which uses 2 RBs as mentioned before. To assess how well the ML approach works, two baseline are used, i) the optimal SVD based precoders, where SVD-based precoders are derived for all subbands and then sent back by the UE, and ii) CSI Type I codebook, where the wideband and sub-band precoders’ indices are selected and sent back by the UE. For the ML approach, SVD precoders are derived based on the decompressed channel at the gNB side. Finally, to evaluate the performance, the block error rate (BLER) is used as performance metric.

Table 3 Common simulation parameters for model evaluation.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| BWP | 52 RBs |
| Subcarrier spacing | 30 kHz |
| Subband/PRG size | 4 RBs |
| Number of transmit antennas (Nt) | 8 |
| Number of receive antennas (Nr) | 2 |
| Number of layers | 1 |
| UE speed | 3 km/hr |
| Channel estimation | Ideal |
| SNR | [-10,10] dB |
| MCS index | 13 |
| Number of drops | 500 |
| Number of slots per drop | 500 |

### Testing under the same channel model and delay spread (In-distribution)

This section provides an initial baseline of the ML-based CSI compression for in-distribution channels, where the “Model-CDLA” and “Model-Mixed” are tested in the same channel models and the same delay spread as used for their respective training. For each model, the BLER of the ML-based CSI compression is compared to the BLER for the ideal SVD as well as to the CSI Type I performance.

Figure 1 shows the in-distribution performance of Model-CDLA (trained on a single-channel model synthetic dataset), tested in the CDL-A 30 ns channel.

Figure 1. In-distribution (CDL-A 30ns) performance of Model-CDLA; various feedback sizes evaluated

It can be seen that the ML-based CSI compression outperforms the CSI Type I baseline; for the same feedback overhead as CSI Type I, the ML-based CSI compression approaches the ideal SVD performance. As the compression ratio is increased and the feedback overhead is reduced to 32 or 16 bits, the performance of the ML-based CSI compression degrades somewhat, but still outperforms the baseline CSI Type I.

Observation 1: The AI/ML model outperforms CSI Type I when tested in the same channel model and delay spread (CDL-A 30ns) as used for training.

To further evaluate the in-distribution performance, the ML-model trained on the mixed CDL-A, CDL-B and CDL-C channel (“Model-Mixed”) is tested separately in the CDL-A 30 ns, CDL-B 300 ns, and CDL-C 300 ns. The simulation results are shown in Figure 2, Figure 3 and Figure 4, respectively.

Figure 2. In-distribution (CDL-A 30ns) performance of Model-Mixed

Figure 3. In-distribution (CDL-B 300ns) performance of Model-Mixed

Figure 4. In-distribution (CDL-C 300ns) performance of Model-Mixed

Figure 3 and Figure 4 show that the in-distribution performance of the ML-based CSI compression for CDL-B 300 ns and CDL-C 300 ns is significantly better than the CSI Type I baseline (for both the same feedback size of 64 bits as CSI Type I, and for lower feedback size).

Observation 2: The multi-channel (combination of CDL-A, CDL-B and CDL-C) trained AI/ML CSI compression model tested in-distribution, outperforms CSI Type I, even with smaller feedback overhead. With the same feedback overhead, the AI/ML mixed model performs better than CSI Type I.

At the same time, the in-distribution performance of “Model-Mixed” for CDL-A 30 ns in Figure 2 shows only a marginal improvement of the 64-bit solution/model compared to the CSI Type I baseline.

The relative performance of the single-channel trained (Model-CDLA) versus multi-channel trained (Model-Mixed) CSI compression model is evaluated in-distribution for CDL-A, and shown in Figure 5 below.

Figure 5. Relative performance of Model-CDLA and Model-Mixed for in-distribution CDL-A 30 ns

While both the single-channel trained (Model-CDLA) and multi-channel trained (Model-Mixed) outperform the CSI Type I, the performance of the Model-Mixed is slightly worse than the performance of the Model-CDLA. This is likely due to the generalization loss associated with training the model on a more diverse dataset.

Observation 3: The “Model-Mixed” exhibits a small performance degradation compared to “Model-CDLA” when tested in the CDL-A 30 ns channel (possibly due the generalization loss). However, overall, the “Model-Mixed” is a better model (compared to “Model-CDLA”) that can cover more cases.

### Testing under the different channel model and/or delay spread (Out-of-distribution) – delay spread impact

This section presents evaluation results for out-of-distribution testing, with focus on the impact of the channel delay spread. The term out-of-distribution is used to indicate the case where the testing setting is different from the training setting.

One example is the single-channel CDL-A 30 ns trained ML Model (Model-CDLA), and tested on CDL-A 300 ns. The simulation results are shown in Figure 6.

Figure 6. Out-of-distribution (CDL-A 300ns – higher delay spread) performance of Model-CDLA

The simulation results from Figure 6 show a significant degradation of Model-CDLA in the out-of-distribution increased delay spread scenario, where the performance becomes worse than the CSI Type I baseline.

Observation 4: The AI/ML model trained in single-channel CDL-A 30 ns exhibits significant performance degradation when tested out-of-distribution using CDL-A with a larger delay spread (300 ns).

To further investigate the out-of-distribution (larger delay spread) performance, another example is considered, specifically the Model-Mixed; this is tested in CDL-A 300 ns, with the simulation results shown in Figure 7.

Figure 7. Out-of-distribution (CDL-A 300ns – higher delay spread) performance of Model-Mixed

Compared to the in-distribution performance of Model-Mixed in CDL-A 30 (Figure 2), the mixed model experiences some performance degradation for out-of-distribution (larger delay spread); however, that relative degradation is significantly smaller compared to the degradation experienced by the single-channel trained model in Figure 6. This is likely due to the large variability in the mixed-channel training relative to the CDL-A only training.

Observation 5: For AI/ML models trained on mixed-channel models (including multiple delay spreads), the performance is more resilient to out-of-distribution (larger delay spread in CDL-A) testing.

The impact of delay spread is further evaluated, by simulating out-of-distribution channels with smaller delay spread than used for training. For example, simulation results for Model-Mixed in CDL-B 30 ns and CDL-C 30 ns are shown in Figure 8 and Figure 9, respectively.

Figure 8. Out-of-distribution (CDL-B 30ns – lower delay spread) performance of Model-Mixed

Figure 9. Out-of-distribution (CDL-C 30ns – lower delay spread) performance of Model-Mixed

The simulation results from Figure 8 and Figure 9 show that for the Model-Mixed, testing in the out-of-distribution smaller delay spread channel has only a small impact on performance. Both models still significantly outperform CSI Type I.

Observation 6: For the simulated configuration, testing mixed-channel trained models (“Model-Mixed”) out-of-distribution with smaller delay spread has only a minor impact on performance.

### Testing under the different channel model and/or delay spread (Out-of-distribution) – channel model impact

This section presents evaluation results for out-of-distribution testing, with focus on the impact of the channel model.

For example, the single-channel CDL-A 30 ns trained ML Model (Model-CDLA) is tested in CDL-B 30 ns, and CDL-C 30 ns, and the simulation results are shown in Figure 10 and Figure 11, respectively.

Figure 10. Out-of-distribution – channel model impact (CDL-A 30 train; CDL-B 30 test)

Figure 11. Out-of-distribution – channel model impact (CDL-A 30 train; CDL-C 30 test)

The results indicate that the model trained on CDL-A performs well on CDL-B and CDL-C for the same delay spread as in training (30 ns).

Observation 7: The AI/ML model trained on CDL-A performs well on CDL-B and CDL-C for the same delay spread as in training (30 ns); (out-of-distribution from a channel model standpoint)

Observation 8: The AI/ML performance is sensitive to the training dataset. It is critical that the dataset includes several channel models, with multiple different parameters for each of the channel models.

### Impact of feedback size

This section evaluates the impact of the CSI feedback size on the link performance. An example is shown in Figure 12, for Model-CDLA (trained on a single-channel, specifically CDL-A 30 ns) and tested in CDL-A 30 ns (in-distribution).

Figure 12. Impact of CSI feedback size (tested in CDL-A 30ns)

It can be seen that reducing the CSI feedback size from 64 bits to 32 bits and 16 bits results in a gradual performance degradation of the ML model while still outperforming CSI Type I.  This suggests that it is possible to configure the ML-based CSI compression to reach a target trade-off between link performance and feedback overhead.

Observation 9: A factor of 4 reduction in the feedback size is achievable with the AI/ML model (trained in single-channel CDL-A 30 ns and tested in CDL-A 30 ns), as compared to CSI Type I.

### Impact of Doppler

The impact of UE speed / Doppler is evaluated for Model-Mixed, via testing on CDL-A 30 ns, for a UE speed of 30 km/h. The simulation results for this scenario are shown in Figure 13.

Figure 13. Doppler impact for Model-Mixed, tested with CDL-A 30 ns and 30 km/hr speed

Observation 10: The AI/ML model trained in multi-channel (“Model-Mixed”) outperforms CSI Type I when tested in CDL-A 30 ns and UE speed 30 km/h (55 Hz Doppler).

# Conclusion

In this contribution, we discussed a ML model for CSI compression and evaluated the ML model and compare the performance over Type I codebook. From the evaluation results, we observed the following:

Observation 1: The AI/ML model outperforms CSI Type I when tested in the same channel model and delay spread (CDL-A 30ns) as used for training.

Observation 2: The multi-channel (combination of CDL-A, CDL-B and CDL-C) trained AI/ML CSI compression model tested in-distribution, outperforms CSI Type I, even with smaller feedback overhead. With the same feedback overhead, the AI/ML mixed model performs better than CSI Type I.

Observation 3: The “Model-Mixed” exhibits a small performance degradation compared to “Model-CDLA” when tested in the CDL-A 30 ns channel (possibly due the generalization loss). However, overall, the “Model-Mixed” is a better model (compared to “Model-CDLA”) that can cover more cases.

Observation 4: The AI/ML model trained in single-channel CDL-A 30 ns exhibits significant performance degradation when tested out-of-distribution using CDL-A with a larger delay spread (300 ns).

Observation 5: For AI/ML models trained on mixed-channel models (including multiple delay spreads), the performance is more resilient to out-of-distribution (larger delay spread in CDL-A) testing.

Observation 6: For the simulated configuration, testing mixed-channel trained models (“Model-Mixed”) out-of-distribution with smaller delay spread has only a minor impact on performance.

Observation 7: The AI/ML model trained on CDL-A (“Model-CDLA”) performs well on CDL-B and CDL-C for the same delay spread as in training (30 ns); (out-of-distribution from a channel model standpoint)

Observation 8: The AI/ML performance is sensitive to the training dataset. It is critical that the dataset includes several channel models, with multiple different parameters for each of the channel models.

Observation 9: A factor of 4 reduction in the feedback size is achievable with the AI/ML model (trained in single-channel CDL-A 30 ns and tested in CDL-A 30 ns), as compared to CSI Type I.

Observation 10: The AI/ML model trained in multi-channel (“Model-Mixed”) outperforms CSI Type I when tested in CDL-A 30 ns and UE speed 30 km/h (55 Hz Doppler).

Based on the observations, we propose the following:

Proposal 1:	Consider AI/ML-based CSI compression as a representative sub-use case of the CSI feedback enhancement use case.

Proposal 2: 	Consider to use a common dataset for model training; similarly, use a common dataset for model performance evaluation.

Proposal 3: 	Consider the synthetic dataset based on existing 3GPP channel model for model training and evaluation. Existing codebook Type I and Type II can be use as baseline for performance evaluation.

Proposal 4:	For CSI enhancements, RAN1 to identify set of relevant statistics for in-distribution and out-of-distribution evaluation of the model performance.

Proposal 5: 	Evaluation methodology should include the configuration assumptions for the datasets (training, validation, and testing).

# References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

C. Wen, W. Shih and S. Jin, "Deep Learning for Massive MIMO CSI Feedback," in IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748-751, Oct. 2018, doi: 10.1109/LWC.2018.2818160.