3GPP TSG RAN WG1 #109-e	R1-2204054

e-Meeting, May 9th – May 20th, 2022

Source:	Sharp

Title:	Subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

# Introduction

We present our view on subband non-overlapping full duplex based on the following assumptions in SID [1].

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

# Interference issues

# Intra-cell interference

Figure 1 is an example of a serving cell operation for subband non-overlapping full duplex. As in Figure 1, the gNB transmits channels in the DL resource, and receives channels in the UL resource, within a serving cell resource. Due to the small gap between the DL resource and the UL resource, the received channels in the UL resource suffer from strong self-interference caused by channels transmitted by the gNB itself (i.e., gNB self-interference).

Although tolerable interference level should be studied/provided by RAN4, RAN1 can discuss candidate solutions to mitigate gNB self-interference.

Proposal 1: RAN1 further studies candidate solutions to mitigate gNB self-interference for subband non-overlapping full duplex.

On the other hand, from UE point of view, intra-cell inter-subband CLI causes an issue for receiving UEs (i.e., UE-to-UE intra-cell inter-subband CLI). This is because the UE#2’s spectrum emission outside the UL resource is received by the UE#1.

Although tolerable interference level should be studied/provided by RAN4, RAN1 can discuss candidate solutions to mitigate UE-to-UE intra-cell inter-subband CLI.

Proposal 2: RAN1 further studies candidate solutions to mitigate UE-to-UE intra-cell inter-subband CLI for subband non-overlapping full duplex.

Figure 1: Example of a serving cell for subband non-overlapping full duplex

Figure 2 is another example of a serving cell operation for subband non-overlapping full duplex. As in Figure 2, two TRPs are deployed within the same serving cell resource, the TRP#1 receives channels in the UL resource, and the TRP#2 transmits channels in the DL resource. Due to the small gap between the DL resource and the UL resource, the received channels in the UL resource suffer from strong intra-cell inter-subband CLI (i.e., TRP-to-TRP intra-cell inter-subband CLI).

Although tolerable interference level should be studied/provided by RAN4, RAN1 can discuss candidate solutions to mitigate TRP-to-TRP intra-cell inter-subband CLI.

Proposal 3: RAN1 further studies candidate solutions to mitigate TRP-to-TRP intra-cell inter-subband CLI for subband non-overlapping full duplex.

On the other hand, from UE point of view, intra-cell inter-subband CLI causes an issue for receiving UEs (i.e., UE-to-UE intra-cell inter-subband CLI). However, the situation is quite similar to what is suggested in Proposal 2.

Figure 2: Example of a serving cell for subband non-overlapping full duplex

# Inter-cell interference

Figure 3 is an example of inter-cell operation for subband non-overlapping full duplex. In Figure 3, the gNB#1 receives channels in the UL resource from the UE#1 and the gNB#2 transmits channels in the DL resource to the UE#2. Due to the small gap between the DL resource and the UL resource, the gNB#1 receives strong inter-cell inter-subband CLI from the gNB#2 (i.e., gNB-to-gNB inter-cell inter-subband CLI).

On the other hand, the UE#2 receives strong inter-cell inter-subband CLI from the UE#1 (i.e., UE-to-UE inter-cell inter-subband CLI).

Although tolerable interference level should be studied/provided by RAN4, RAN1 can discuss candidate solutions to mitigate gNB-to-gNB/UE-to-UE inter-cell inter-subband CLI.

Proposal 4: RAN1 further studies candidate solutions to mitigate gNB-to-gNB/UE-to-UE inter-cell inter-subband CLI for subband non-overlapping full duplex.

Figure 3: Example of inter-cell operation for subband non-overlapping full duplex

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: RAN1 further studies candidate solutions to mitigate gNB self-interference for subband non-overlapping full duplex.

Proposal 2: RAN1 further studies candidate solutions to mitigate UE-to-UE intra-cell inter-subband CLI for subband non-overlapping full duplex.

Proposal 3: RAN1 further studies candidate solutions to mitigate TRP-to-TRP intra-cell inter-subband CLI for subband non-overlapping full duplex.

Proposal 4: RAN1 further studies candidate solutions to mitigate gNB-to-gNB/UE-to-UE inter-cell inter-subband CLI for subband non-overlapping full duplex.

# References

- [1]	RP-213591, 3GPP TSG RAN#94-e, December 2021.