3GPP TSG RAN WG1 #109-e			R1-2204055

e-Meeting, May 9th -20th, 2022

Agenda item:	9.2.2.1

Title:	Evaluation of CSI compression with AI/ML

Source:	Beijing Jiaotong University

Document for:	Discussion

1. Introduction

In RAN 94-e meeting, new study item on AI/ML for NR Air Interface was approved [1]. In the study item scope, the initial set of use cases were identified:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this paper, we discuss the evaluation methodology and provide preliminary evaluation result for two sub use cases in another paper [2] for CSI compression.

2. Evaluation methodology and KPIs

2.1 Simulation scenarios

For the research of CSI compression with AI/ML, it is necessary to calibrate its simulation and experiment before we evaluate DL-based CSI feedback methods. In the current pre-research stage, it is mainly to observe whether the AI-based technology has a gain compared with the existing technology, and how much the gain is. It is completely possible to use the existing channel model for performance evaluation. We can obtain the simulation scenarios dataset based on various existing channel models in TR 38.901 [4], such as TDL, CDL and other channel models of link-level platforms, and UMa, UMi, Indoor and other channel models of system-level platforms.

Proposal #1: The simulation scenarios dataset could be constructed both on the link level and system level simulation platforms based on various existing channel models in TR 38.901.

2.2 Benchmark

Most of the schemes used in existing communication systems are CSI feedback based on codebooks, mainly Type I codebook and Type II/eType II codebook. Among them, the Rel-16 Type II/eType II codebook has higher precision and is suitable for MU scheduling. The Rel-16 Type II/eType II codebook mainly projects channel information or precoding information into the angle and delay domains, compresses the information through the sparsity of the angle domain and the delay domain, removes coefficients with small proportions, and retains angles with large proportions and delay coefficients, so as to express channel characteristics with less information. It should be a great benchmark for CSI compression with AI/ML.

Proposal #2: Rel-16 Type II/eType II codebook could be considered as a benchmark for evaluation for CSI compression.

2.3 KPIs

KPIs are critical for the evaluation of AI-based CSI compression. Two categories of KPIs for performance evaluation are discussed below. One is the KPIs for AI model, and another is the KPIs for communication.

KPIs for AI model include model complexity, inference delay, memory storage, model generalization, hardware requirements, etc. A KPI to evaluate the computational volume of an AI model is the number of multiplication and addition operations, which are usually expressed as multiplication or addition operations (FLOP: floating operation), multiplication and addition operations (MAC: multiply–accumulate, 1MAC= 1 times + 1 plus = 2OPs); a common KPI of AI computing power is the number of (floating point/multiply or add) operations performed per second (FLOPS: floating point operations per second). The ​​model complexity can be measured by FLOP, and the ​​hardware requirement can be measured by FLOPS. The inference delay of the AI ​​model is related to the hardware computing power and model complexity, so the measurement needs to be performed under the same hardware. Generalization can be achieved by verifying the performance of AI models in different test sets. Through the verification of generalization, the applicable scenarios of an AI model can be obtained.

Proposal #3: Model complexity, inference delay, memory storage, model generalization, hardware requirements could be considered as a KPI for evaluation for CSI compression. Specifically, model complexity can be measured by FLOP, hardware requirement can be measured by FLOPS, model generalization can be measured by verifying the performance of AI models in different test sets.

As for the KPIs for communication, almost NMSE and cosine similarity in the current research, which is intermediate variable affecting the system performance, and its corresponding relationship with spectral efficiency is not yet clear. And traditional communication KPI can be considered, for example, the throughput and spectral efficiency can be used to evaluate the sub use case of CSI feedback, but these are only applying to the system-level simulation evaluations.

Proposal #4: NMSE, cosine similarity, throughput and spectral efficiency could be considered as performance KPIs for evaluation for CSI compression.

In addition, the channel capacity with imperfect CSI can also be used as KPI for evaluation for CSI compression. Unlike throughput and spectral efficiency, system capacity can be used both at the link level simulation evaluations and at the system level simulation evaluations. The channel capacity with imperfect CSI can be measured with the known feedback CSI and CSI error. During the data transmission phase, the receive signal  y ∈CM×1 can be recast as:

y=γHx+n,

where H∈CM×N, x ∈CN×1, n ∈CM×1 represent the ground truth channel, transmission data and noise, respectively, γ is the receive SNR at the user. Assume that H=H+H,  where H is the feedback CSI matrix, H is the CSI error matrix, whose elements are independent and identically distributed and zero-mean σH2-variance complex-Gassian distribution, where σH2=PeMN and Pe=HF2. We also assume that x∽CN(0, PsIN), n∽CN(0, σn2IM), where a∽CN(μ,Σ) denotes a complex Gaussian vector a with mean μ and covariance Σ, IM represents an M×M identity matrix, and x, n and H are uncorrelated. According to [6], the capacity can be proposed by:

C=Ε|H，Pe{log2det(IN+γPsγPsPeM+σn2HHH)} .

Proposal #5: Channel capacity (bit/s) with imperfect CSI could be considered as performance KPIs for evaluation for CSI compression.

3. Preliminary results

3.1 Simulation assumptions

To simulate two sub use cases in another paper [2], the bit-level DL-based CSI feedback (BC) and he symbol-level DL-based CSI feedback (SC). We consider two simulation scenarios (UMi, UMa) to generate the uplink CSI and downlink CSI which has detailed description in TR38.901[4]. Table 1 describes the detailed parameter configuration of our simulation.

The uplink CSI and downlink CSI are generated by QuaDRiGa [5] according to the 3GPP TR 38.901 [4] in which the downlink center frequency is 2.4 GHz and the uplink center frequency is 2.6 GHz. The gNB is positioned at the center of a square area with the size of 200m × 200m for UMi and 400m× 400m for UMa. A uniform linear array (ULA) with half-wavelength antenna space is deployed at the gNB. Both the antennas in the gNB and in the UE are omnidirectional. The number of the antennas at the gNB is  Nt Nt = 32 and the number of the antennas at the UE is Nr =1. The uplink and the downlink both have Nc = 256 subcarriers. When transforming the channel matrix into the angular-delay domain, we retain the first 32 rows of the channel matrix. That is, F is 32×32 in size. The training, validation, and testing sets contain 100,000, 30,000, and 20,000 samples, respectively. A sample pair contains a downlink CSI sample and an uplink CSI sample. Following the setting in the existing SC-CSI methods [3].

Table 1: Simulation assumptions for UMi and UMa

| Parameter | Values |  |
| --- | --- | --- |
| Scenarios | UMi，UMa |  |
| BS antenna configurations | (M,N,P) = (32, 1, 1), dV = 0.5λ |  |
| UE mobility (movement in horizontal plane) | 3 km/h |  |
| Bandwidth | 10MHz |  |
| UT antenna configurations | 1 element (vertically polarized) |  |
| Center Frequency | 2.4GHZ for uplink; 2.6 GHz for downlink |  |
| Carrier number | 256 |  |
| ISD | 200m for UMi, 400m for UMa |  |
| BS antenna height | 10m for UMi, 35m for UMa |  |
| UE height (hUT) in meters | general equation | hUT=3(nfl – 1) + 1.5 |
|  | nfl for outdoor UEs | 1 |
|  | nfl for indoor UEs | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |

The training strategy of the bit-level DL-based CSI feedback method contains the following three steps:

- The CsiNet+ without quantization is trained by an end-to-end approach in the truncated angular-delay domain;

- The CsiNet+ encoder, non-uniform quantizer, and the non-uniform dequantizer are combined to generate the training dataset, which is then used to train the offset network for quantization compensation by minimizing the mean square error (MSE) between the training dataset and the output of the offset network;

- The parameters of the CsiNet+ encoder are fixed and the parameters of the offset network and the CsiNet+ decoder are fine tuned to further improve the reconstruction performance. More training Details about bit-level CsiNet+ can be found in [3].

The training strategy of the symbol-level DL-based CSI feedback method following the setting in the existing methods [7].

The channel capacity is employed as the channel coding rate to simplify the evaluation process for comparing the symbol-level CSI feedback method with the bit-level CSI feedback method. According to the experiment results provided by [3], the performance of the quantized version with B = 6 is near the performance of the unquantized version represented by 32-bit float values. Here, we assume the performance of the quantized version with B = 5 equals the performance of the unquantized version, which simplifies the quantization process and improves the upper bound of the performance of the bit-level DL-based CSI feedback method.

The symbol-level DL-based CSI feedback method assumes that any complex value can be transmitted over the channel. However, in the real scenarios the hardware or protocol can only admit certain sets of channel inputs, such as the use of a digital constellation. To make the symbol-level DL-based CSI feedback method compatible with the existing digital modulation module, we quantize the output of the symbol-level DL-based CSI feedback to the digital constellation (e.g., 64QAM, 256QAM, 1024QAM).

3.2 Simulation results

Fig.1-Fig.3 show the performance of the bit-level DL-based CSI feedback in two simulation scenarios (UMi, UMa) with NMSE, cosine similarity and Channel capacity with imperfect CSI, respectively. With the increase of the number of feedback bits, the performance of the BC_UMi and the BC_UMa increased. Specifically, With the increase of the feedback bits from 1024 bits to 4096 bits, the NMSE decreased more than 5 dB. The comparison between the cosine similarity and Channel capacity reveals similar results. When we compress the truncated CSI to different dimensional vector (512/1024) and use different quantization schemes, the performance of the bit-level DL-based CSI feedback method will show different model gains.

Observation #1: The channel recovery capability of the bit-level DL-based CSI feedback model is ideal.

Observation #2: Different air interface feedback and quantization schemes will show different model gains.

Figure 1: Evaluation for the bit-level DL-based CSI feedback in NMSE. The label “BC_ed512_UMi” means the CsiNet+ compresses the truncated CSI to a 512-dimensional vector, and the simulation scenarios is UMi. Note that the number of quantization bits corresponding to “ed512” is 2, 4, 6, 8; And the number of quantization bits corresponding to “ed1024” is 1, 2, 3, and 4, respectively.

Figure 2: Evaluation for the bit-level DL-based CSI feedback in cosine similarity.

Figure 3: Evaluation for the bit-level DL-based CSI feedback in Channel capacity with imperfect CSI. The label “BC_snr0_UMi” means the CsiNet+ compresses the truncated CSI to a 512-dimensional vector, the receive SNR is 0 dB and the simulation scenarios is UMi.

Fig.4-Fig.6 show the performance of the symbol-level DL-based CSI feedback in two simulation scenarios (UMi, UMa) with NMSE, cosine similarity and Channel capacity with imperfect CSI, respectively. The performance of the BC_UMi and the BC_UMa are worse than the performance of the SC_UMi and the SC_UMa, respectively. Specifically, as for NMSE, the performance of the BC_UMa is 3.83 dB worse than the performance of the SC_UMa at SNRtest = -10 dB. With the increase of the SNRtest from −10 dB to -6 dB, the SC_UMa brings a gradually increased performance, outperforming the BC_UMa by a margin at almost 3.34 dB. With the further increase of the SNRtest from -6 dB to 10 dB, the gap between the BC_UMa and the SC_UMa slowly decreases, while is still larger than 4 dB. In other words, if NMSE = −7 dB is required at the BS to promise the reconstruction quality, the UE adopting the SC_UMa can save at least 20 dB transmission power than that adopting the BC_UMa. The comparison between the BC_UMi and the SC_UMi reveals similar results.

Observation #3: The performance of the symbol-level DL-based CSI feedback better than the bit-level DL-based CSI feedback.

Figure 4: Evaluation for the symbol-level DL-based CSI feedback in NMSE.

Figure 5: Evaluation for the symbol-level DL-based CSI feedback in cosine similarity.

Figure 6: Evaluation for the symbol-level DL-based CSI feedback in Channel capacity with imperfect CSI.

Fig. 7-Fig.8 compare the performance of the quantized symbol-level DL-based CSI feedback in NMSE and cosine similarity. We use three modulation scheme (64QAM, 256QAM and 1024QAM) to quantize the symbol-level DL-based CSI feedback. With the increase of the modulation order, the performance of the SC_UMi and the SC_UMa increased. Specifically, the performance of the SC_64QAM_UMa is 0.2 dB worse than the performance of the SC_1024QAM_UMa at SNRtest = -10 dB. The comparison between the cosine similarity reveals similar results.

Observation #4: The performance of the quantized symbol-level DL-based CSI feedback increases with the improvement of the quantization level.

Figure 7: Evaluation for quantizing the symbol-level DL-based CSI feedback in NMSE.

Figure 8: Evaluation for quantizing the symbol-level DL-based CSI feedback in cosine similarity.

Based on the above observations, we have the following proposal:

Proposal #6: The channel recovery capability of the symbol-level DL-based CSI feedback better than the bit-level DL-based CSI feedback.

4. Conclusion

In this paper, the evaluation of CSI compression with AI/ML are discussed, and provide preliminary evaluation result for two sub use cases in another paper [2] for CSI compression.

We have the following observations:

Observation #1: The channel recovery capability of the bit-level DL-based CSI feedback model is ideal.

Observation #2: Different air interface feedback and quantization schemes will show different model gains.

Observation #3: The performance of the symbol-level DL-based CSI feedback better than the bit-level DL-based CSI feedback.

Observation #4: The performance of the quantized symbol-level DL-based CSI feedback increases with the improvement of the quantization level.

We have the following proposals:

Proposal #1: The simulation scenarios dataset could be constructed both on the link level and system level simulation platforms based on various existing channel models in TR 38.901.

Proposal #2: Rel-16 Type II/eType II codebook could be considered as a benchmark for evaluation for CSI compression.

Proposal #3: Model complexity, inference delay, memory storage, model generalization, hardware requirements could be considered as a KPI for evaluation for CSI compression. Specifically, model complexity can be measured by FLOP, hardware requirement can be measured by FLOPS, model generalization can be measured by verifying the performance of AI models in different test sets.

Proposal #4: NMSE, cosine similarity, throughput and spectral efficiency could be considered as performance KPIs for evaluation for CSI compression.

Proposal #5: Channel capacity (bit/s) with imperfect CSI could be considered as performance KPIs for evaluation for CSI compression.

Proposal #6: The channel recovery capability of the symbol-level DL-based CSI feedback better than the bit-level DL-based CSI feedback.

5. References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] R1-2204057, CSI compression with AI/ML

[3] J. Guo, C.-K. Wen, S. Jin, and G. Y. Li, “Convolutional neural network- based multiple-rate compressive sensing for massive mimo csi feedback: Design, simulation, and analysis,” IEEE Transactions on Wireless Communications, vol. 19, no. 4, pp. 2827–2840, 2020.

[4] TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, 3GPP TR 38.901 V17.0.0, Mar. 2022

[5] Jaeckel S ,  Raschkowski L ,  Borner K , et al. QuaDRiGa: A 3-D Multi-Cell Channel Model With Time Evolution for Enabling Virtual Field Trials[J]. IEEE Transactions on Antennas and Propagation, 2014, 62(6):3242-3256.

[6] B. Hassibi and B. M. Hochwald, "How much training is needed in multiple-antenna wireless links?," in IEEE Transactions on Information Theory, vol. 49, no. 4, pp. 951-963, April 2003, doi: 10.1109/TIT.2003.809594.

[7] J. Xu, B. Ai, W. Chen, A. Yang, P. Sun and M. Rodrigues, “Wireless Image Transmission Using Deep Source Channel Coding With Attention Modules,” IEEE Transactions on Circuits and Systems for Video Technology, vol. 32, no. 4, pp. 2315-2328, April 2022,.