3GPP TSG RAN WG1  #109-e			R1-2204057

e-Meeting, May 9th -20th, 2022

Agenda item:	9.2.2.2

Title:	CSI compression with AI/ML

Source:	Beijing Jiaotong University

Document for:	Discussion

1. Introduction

In RAN 94-e meeting, new study item on AI/ML for NR Air Interface was approved [1]. In the study item scope, the initial set of use cases were identified:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this paper, two sub use cases on CSI feedback enhancement are proposed, and some potential specification impacts are discussed.

2. CSI compression with AI/ML

In the 5G system, the acquisition of channel state information (CSI) is a very critical step. Through CSI, you can achieve the number of information streams carried by the channel, the quality or signal-to-noise ratio of the channel, and the channel matrix. As the number of antennas increases, the overhead of CSI feedback also increases. At present, whether in academia or industry, CSI overhead reduction is ​​one of the key research directions.

3GPP currently standardizes the Type I codebook and the Type II codebook for feedback of the channel matrix. At present, CSI feedback mechanisms such as Type I/II codebooks have the following problems and shortcomings:

1) Type I/II codebooks are all based on DFT vectors, and the prerequisite is that the antennas are uniformly arranged in horizontal and vertical dimensions. That is to say, the current type I/II codebook is not suitable for 3G base station antennas, special-shaped antennas, and large-scale distributed antennas. It has great restrictions on the subsequent antenna hardware design.

2) The design of Type I/II codebook is based on the assumption of uniform distribution of signal arrival angle and departure angle. However, in the actual environment, the statistic of arrival angle and departure angle are not uniformly distributed, which makes Type I/II codebook suboptimal.

3) Type I/II codebooks have their own application scenarios. The Type I codebook designed for single-user transmission is relatively simple, but has limited accuracy. The Type II codebook designed for multi-user transmission has high precision, while the overhead of the Type II codebook is too high.

The introduction of AI/ML technology for CSI compression can effectively solve these problems, as follows:

1) For the non-aligned base station antenna array, channel data is obtained through simulation modelling, channel measurement, etc. AI network is trained by the collected data to obtain a matching AI network.

2) For complex actual scenarios, it is also possible to optimize the AI network for a certain base station.

3) For different scenarios and requirements, different feedback resource can be used for AI network training.

2.1 Bit-level DL-based CSI feedback

Based on the modular design principle in modern wireless communications, the bit-level DL-based CSI feedback [2] focus on CSI compression (i.e., source coding) with the promise of reliable transmission provided by appropriate channel coding and modulation as shown in Fig. 1. We consider a single cell FDD massive MIMO-OFDM system with Nt≥1 antennas at a BS and a single antenna at a UE. Both the uplink and the downlink are over Nc subcarriers. The downlink CSI in the spatial-frequency domain is represented by Hd∈CNc×Nt.

By exploiting the sparsity of the CSI information in the angular-delay domain, a 2D discrete Fourier transform (DFT) is executed at the UE to convert the downlink CSI Hd from the spatial-frequency domain to the angular-delay domain, which is expressed as:

F= IdHdDa,

where F∈CNc×Nt is the CSI information in the angular-delay domain, Id is an Nc×Nc inverse DFT (IDFT) matrix, and Da is an Nt×Nt DFT matrix. Due to the fact that most of the power among multiple paths lies in a particularly limited period, the first Nc' rows which exhibit the distinct non-zero values are retained to construct the truncated CSI F∈CNc'×Nt. These two stages are integrated as the spatial-frequency domain to truncated angular-delay domain (SF2TAD) module in Fig. 1.

Figure 1: The bit-level DL-based CSI feedback framework

The SC-CSI encoder (SC-CSI-E) module at the UE encodes the truncated CSI F to the compressed vector c∈Rm. The quantizer at the UE quantizes each dimension of the compressed vector into B bits. The number of feedback bits is m×B. During the feedback process, the feedback bits is transmitted without error.

The dequantizer and the SC-CSI decoder (SC-CSI-D) at the BS dequantize the feedback bits and then decode it subsequently. The recovered truncated CSI in the angular-delay domain at the BS is represented by F∈CNc'×Nt.  Since the truncated operation is non-differentiable, The SC-CSI-E, the quantizer, the SC-CSI-D and the dequantizer are trained by optimizing the following loss function Lsc:

Lsc=1Ti=1T∥Fi-F(i)∥22,

Where T is the number of samples in the training dataset and F(i) represents the i-th sample in the training dataset in the truncated angular-delay domain.

In the inference stage, the recovered CSI in the truncated angular-delay domain at the BS is padded and converted to the CSI in the spatial-frequency domain for the subsequent process. We provide initial simulation results to show the performance of the bit-level DL-based CSI feedback in [4].

Proposal #1: Study Bit-level DL-based CSI feedback as a use case for CSI feedback enhancement.

2.2 Symbol-level DL-based CSI feedback

The use case Bit-level DL-based CSI feedback considers the CSI compression problem as a source coding problem, which is the mainstream in the classical separate source channel coding (SSCC) system. However, there are several drawbacks. Firstly, this SSCC scheme has been demonstrated inferior to the joint source-channel coding (JSCC) scheme in the finite block length regime in theory. Secondly, the SSCC scheme has “cliff effect” in the real wireless scenario. That means the reconstruction quality of the CSI drops drastically, if the real feedback channel condition is worse than expected, and beyond the capability of the applied channel coding scheme. In this case, the recovered CSI at the BS is useless for the subsequent process. The JSCC scheme can provide a graceful performance degradation even the real channel condition becomes worse than the expected channel condition, which makes the recovered CSI still valuable for the subsequent process. Lastly, even though the hybrid automatic repeat request (HARQ) mechanism can compensate for errors of channel decoding caused by channel condition mismatch, HARQ inevitably increases the additional feedback overhead and brings the latency problem for the CSI feedback task.

In this part, we propose the Symbol-level DL-based CSI feedback [4] built upon the JSCC strategy as shown in Fig. 2. We compare the architecture of the symbol-level DL-based CSI feedback with that of the bit-level DL-based CSI feedback for a better explanation.

According to the sparsity assumption of CSI in the angular-delay domain, the CSI information in the spatial-frequency domain Hd is first converted to the truncated angular-delay domain in the bit-level DL-based CSI feedback. However, some useful information about the CSI is discarded and cannot be compensated in the subsequent process, which will lead to some distortion of the recovered CSI in the bit-level DL-based CSI feedback. Here, we exploit nonlinear transforms for CSI source. The nonlinear transforms are implied by using the analysis transform network (ATN) at the UE:

T=AαHd∈CNc'×Nt,

and the synthesis transform network (STN) at the BS:

Hd=Sβ(T)∈CNc×Nt,

where T∈CNc'×Nt is the transformed CSI at the UE, T∈CNc'×Nt and Hd∈CNc×Ntare the recovered CSI in the transform domain and the recovered CSI in the spatial-frequency domain at the BS, respectively. To simplify the notation and enable fair comparison with the bit-level DL-based CSI feedback, the CSI in transform domain T and the recovered CSI in the transform domain T have the same size with the truncated CSI in the angular-delay domain F.

Figure 2: The symbol-level DL-based CSI feedback framework

To exploit the power of JSCC, the SC-CSI-E and the SC-CSI-D in the bit-level DL-based CSI feedback is improved by the following steps:

(1) the CSI in the transform domain T is inputted to the SC-CSI-E.

(2) the output of the SC-CSI-E at the UE is first converted to the complex values;

(3) according to the power constraint, a power normalization is applied at the UE;

(4) the symbols are mapped to OFDM subcarriers and transmitted by the UE;

(5) the MRC and OFDM demapping are executed at the BS;

(6) the received complex values is converted to real values;

(7) the SC-CSI-D decodes the real values to the recovered CSI in the transform domain T.

Based on these modifications, the architectures in the bit-level DL-based CSI feedback can be exploited in the Symbol-level DL-based CSI feedback. These conversions endow the encoding network and the decoding network ability to explore the knowledge from both the source and channel by the end-to- end training, which improves the reconstruction performance of the CSI and overcomes the problem of “cliff effect” in the SSCC scheme.

Lastly, an SNR adaption strategy [5] is introduced to match channel variations for the symbol-level DL-based CSI feedback. We also provide initial simulation results to show the performance of the symbol-level DL-based CSI feedback in [4].

Proposal #2: Study Symbol-level DL-based CSI feedback as a use case for CSI feedback enhancement.

2.3 Specification impact

Since the specification impact seems quite similar of two sub use, we identify the following points:

- The training dataset can be constructed by the simulation data, the field data, and both of them.

- The training stage of the two sub use cases can be executed offline.

- The trained AI model for the UE side can be: (1) pre-set in the UE; (2) transferred from the gNB to the UE.

- The output type (i.e., the bits or the symbols) should be reported from UE to gNB.

- The feedback bits of the bit-level DL-based CSI feedback can be carried on the current PUCCH.

- The feedback symbols of the symbol-level DL-based CSI feedback can be directly carried on the subcarriers in the OFDM system.

Proposal #3: Study the necessary specification change to support CSI compression, considering AI model transfer, enable/disable AI based CSI compression feedback, as well as the format of inference output as CSI compression feedback to gNB.

3. Conclusion

In this paper, the CSI feedback enhancement use case is discussed. Based on the discussion above, we propose the following proposal:

Proposal #1: Study Bit-level DL-based CSI feedback as a use case for CSI feedback enhancement.

Proposal #2: Study Symbol-level DL-based CSI feedback as a use case for CSI feedback enhancement.

Proposal #3: Study the necessary specification change to support CSI compression, considering AI model transfer, enable/disable AI based CSI compression feedback, as well as the format of inference output as CSI compression feedback to gNB.

4. References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] J. Guo, X. Li, M. Chen, P. Jiang, T.Yang, W. Duan, H. Wang, S. Jin and Q.n Yu, “AI Enabled Wireless Communications with Real Channel Measurements: Channel Feedback”, Journal of Communications and Information Networks, vol. 5, no. 3, pp. 310-317, Sep. 2020.

[3] R1-220XXX, Evaluation of CSI compression with AI/ML

[4] J. Xu, B. Ai, N. Wang N and W. Chen, “Deep Joint Source-Channel Coding for CSI Feedback: An End-to-End Approach”, arXiv preprint arXiv:2203.16005, 2022.

[5] J. Xu, B. Ai, W. Chen, A. Yang, P. Sun and M. Rodrigues, “Wireless Image Transmission Using Deep Source Channel Coding With Attention Modules,” IEEE Transactions on Circuits and Systems for Video Technology, vol. 32, no. 4, pp. 2315-2328, April 2022,.