3GPP TSG-RAN WG1 Meeting #109-e		R1-2204062

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.2.1

Source:	Charter Communications

Title:	Evaluating General aspects of AI-ML framework

Document for:	Discussion

# Introduction

In RAN#94, it was proposed to study the benefits of augmenting the air-interface with AI/ML algorithms for enhanced performance. Three main use cases are selected to examine such benefits, including CSI feedback, beamforming and positioning. Enhanced performance under each of these use cases could be different, including but not limited to reduced overhead, improved throughput, increased accuracy and higher reliability [1].

The goal is to assess the performance of ML methods in comparison with traditional methods and also consider the associated potential specification impacts that leverage ML methods. First step is to establish a general model characterization and description with proper notation for subsequent discussions. Next step is to acquire a better understanding of achievable gains and also associated complexity by running extensive Link level and system level simulations for each use case.

Last but not least, specification impact will be assessed in order to improve the understanding of requirements for enabling AI/ML techniques such as potential specification of new signalling, assistance information, measurement, and feedback, AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases.

In this contribution, the number of trainable parameters in various layer designs is studied and its effect in training complexity, exchange overhead and performance are evaluated.

# Background

Applying ML techniques includes two subsequent steps: first, training the model and second, the inference. The first step involves running iterative optimizers such as Batch Gradients Decent (BGD), Stochastic Gradients Decent (SGD), etc., over many iterations to find the locally optimal weights or coefficients for the ML model. After training the ML model, the next step is inference, which is applying the trained model on new sets of input to find the desired outputs.

Since the training phase includes large number of optimization iterations, it imposes much larger complexity compared with the inference phase. The complexity of training phase is proportional to the number of weights to optimize. In a NN, the number of parameters to train depends on various factors including the structure of NN, e.g., feedforward, recurrent, convolutional, the number of layers and number of neurons in each layer.

In some scenarios, the trained parameters need to be transferred from one node to another, e.g., from gNB to UE or vice versa. Example of such scenario is using autoencoders in processing CSI feedback where both encoder and decoder need to be trained jointly. If the training is performed at UE, then the trained parameters of decoder need to be transferred to gNB. On the other hand, if the training is performed at gNB, then, the trained parameters of encoder should be provided for UE.

In addition, in various use cases, it is beneficial to assign the task of training to gNB which has more computational power and transfer the trained parameters to UEs for inference. The exchange overhead in such scenarios is proportional to the number of trained parameters. In this contribution, we evaluate the complexity of various layers in term of their trainable parameters and also evaluate and compare various neural network (NN) architectures considering their imposed overhead.

# Training Complexity

Major complexity of NN applications stems from the training phase where large number of parameters are calculated through iterative optimization methods such as gradient decent. The complexity of training phase directly depends on the number of trainable parameters. Large number of trainable parameters not only imposes computational complexity but also requires larger memory space.  In addition, as the number of trainable parameters increase, the overhead for exchanging those parameters between gNB and UE also increases. Therefore, it is of great importance to evaluate the number of trainable parameters for different NN architectures.

There are three main types of layers that have been used in various ML applications: Dense, Convolutional and Recurrent. In dense layers, every input node is connected to every output node, therefore, the number of trainable parameters for a dense layer equal Nparam= NinNout+Nout where Nin and Nout represent the number of input and output nodes, respectively. The term  NinNout account for the multiplicative terms from input nodes to output nodes and Nout accounts for the additive bias terms at output nodes. Since, the number of parameters is proportional to the size of input vector, dense layer is not suitable for large data sets such as image, video, etc.

In convolutional layers, a kernel of desired size is cross correlated by the input to create the output. Each kernel corresponds to a single input channel and output channel. Denoting the number of input and output channels as Cin and Cout, respectively, the number of trainable parameters equal Nparam=CinCout K1K2+Cout where K1 and K2 represent the kernel size in vertical and horizontal dimensions, respectively. Therefore, the number of parameters in a convolutional layer is independent of input size. This property makes the convolutional layers a suitable candidate for processing and feature extraction of large data sets such as image and video.

In recurrent layers or cells, the output depends on both the input and also the previous state of the cell. Long Short-Term Memory (LSTM) and Gated Recurrent Unit (GRU) are important types of recurrent layers. Each cell contains few dense layers each with Nin+Nh input size where Nin represents the input units and Nh represent hidden or output units. Thus, the number of trainable parameters equal  Nparam=Nd (Nh Nin+Nh+Nh )  where  Nd represents the number of dense layers and equals 3 and 4 for GRU and LSTM, respectively. Recurrent layers are capable of processing sequential data; however, the number of trainable parameters can become large if the input size is large.

Choosing a NN architecture with minimal trainable parameters can greatly reduce the training complexity. In addition, the number of trainable parameters affects the overhead for exchanging the trained parameters. Due to capability of convolutional layers for extracting features from multi-dimensional data and also their low number of trainable parameters, convolutional layers are a suitable choice for designing ML techniques in given use cases.

Observation 1:  Convolutional NNs (CNNs) are suitable choice for designing ML techniques in given use cases. CNNs are capable of feature extracting from multi-dimensional data (spatial, frequency and time) and also enjoy low number of trainable parameters which can reduce the training complexity and also exchanging overhead.

# Exchange of Trained Parameters

Depending on whether the training is performed on gNB side or UE side, four modes of training can be described:

- The training is performed separately at gNB and UE, which does not require exchange of trained parameters.

- The training is performed solely at the gNB and the trained parameters are provided for UEs.

- The training is performed at UEs and the trained parameters are transferred to gNB.

- Hybrid mode including partial training at UE and gNB with use of partial exchange of trained parameters.

The number of trainable parameters substantially affects the exchange overhead. Particularly, for the use case of CSI feedback where the compression and decompression should be jointly optimized, the first option is excluded. If the training is performed at UE, then the trained parameters of decoder need to be transferred to gNB. On the other hand, if the training is performed at gNB, then, the trained parameters of encoder should be provided for UE. Therefore, we evaluate the performance of various NNs in enhancing CSI feedback considering their trainable parameters.

Dense Encoder, Dense Decoder:

A simple autoencoder is shown in Fig. 1 which includes three dense layers at encoder and decoder. The input is concatenation of best singular vectors for Ns subbands from CDL-C channel with Nt transmit antennas. The real and imaginary parts are concatenated to create a vector of size 2NtNs. The number of neurons/nodes in each dense layer is shown in Fig. 1.

Figure 1: An autoencoder with encoder and decoder using dense layers

The number of trainable parameters for this autoencoder is shown in Table 1. It can be seen that both encoder and decoder have around 1M parameters which cause immense exchange overhead. Thus, using such architecture is not viable for CSI feedback use case.

Table 1: Trainable parameters of the autoencoder with dense/dense layers

| Input size | Feedback bits (L) | Encoder Parameters |  |  | Decoder Parameters |  |  | Cosine similarity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 208 | 130 bits | Dense 1 | Dense 2 | Dense 3 | Dense 1 | Dense 2 | Dense 3 | 0.941 |
|  |  | 173,888 | 693,056 | 108,290 | 108,992 | 693,056 | 173,264 |  |
|  |  | 975,234 |  |  | 975,312 |  |  |  |

LSTM encoder, Dense decoder:

To reduce the number of trainable parameters in encoder, LSTM layers are used in Fig.2. LSTM layers are capable of analysing sequential data, e.g., singular vectors for subsequent subbands. The input is a matrix of 2Nt×Ns and the number of units for each LSTM is shown in Fig.2.

Figure 2: An autoencoder with encoder using LSTM layers and decoder using dense layers

In Table 2, the number of parameters for the autoencoder with LSTM/dense layers are shown. The number of encoder parameters are reduced by a factor of 16 compared with the previous design with all dense layers. Therefore, such design is more suitable for training mode 2 where the training is performed at gNB and the trained parameters of encoder is transferred to UE. LSTM layers not only reduce the number of trainable parameters but also improve the performance evaluated by cosine similarity between reconstructed versions and original samples.

Table 2: Trainable parameters of the autoencoder with LSTM/dense layers

| Input size | Feedback bits (L) | Encoder Parameters |  |  | Decoder Parameters |  |  | Cosine similarity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16× 13 | 130 bits | LSTM 1 | LSTM 2 | Dense 3 | Dense 1 | Dense 2 | Dense 3 | 0.981 |
|  |  | 41,472 | 6,000 | 17,030 | 108,992 | 693,056 | 173,264 |  |
|  |  | 59,102 |  |  | 975,312 |  |  |  |

CNN Encoder, Dense Decoder:

To further reduce the parameters, convolutional layers are used in Fig. 3. Conv layers reduce the trainable parameters of dense architecture and LSTM with factor of 65 and 4, respectively, as shown in Table 3.

Figure 3: An autoencoder with encoder using Conv layers and decoder using dense layers

For transferring the encoder’s trained parameters from gNB to UE, convolutional layers have the least number of trainable parameters while provide the best performance in terms of cosine similarity

Table 3: Trainable parameters of the autoencoder with Conv/dense layers

| Input size | Feedback bits (L) | Encoder Parameters |  |  | Decoder Parameters |  |  | Cosine similarity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16× 13 | 130 bits | Conv 1 | Conv 2 | Dense 3 | Dense 1 | Dense 2 | Dense 3 | 0.984 |
|  |  | 20 | 19 | 14,170 | 108,992 | 693,056 | 173,264 |  |
|  |  | 14,209 |  |  | 975,312 |  |  |  |

LSTM encoder, LSTM decoder:

To reduce the number of trainable parameters in decoder, LSTM layers can be used at both encoder and decoder, as shown in Fig. 4. It can be seen in Table 4 that the number of trainable parameters at decoder also reduced by a factor of 16 compared with dense design. As a result, such design can be more feasible for both training modes 2 and 3 at the expense of minor degradation in the performance.

Figure 4: An autoencoder with encoder using LSTM layers and decoder using LSTM layers

Table 4: Trainable parameters of the autoencoder with LSTM/LSTM layers

| Input size | Feedback bits (L) | Encoder Parameters |  |  | Decoder Parameters |  |  | Cosine similarity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16× 13 | 130 bits | LSTM 1 | LSTM 2 | Dense 3 | LSTM 1 | LSTM 2 | Dense 3 | 0.969 |
|  |  | 41,472 | 6,000 | 17,030 | 11,008 | 6,272 | 43,472 |  |
|  |  | 59,102 |  |  | 60,752 |  |  |  |

Conv Encoder, Conv Decoder:

Transposed convolutional layers can be used at decoder to further reduce the number of trainable parameters. Transposed convolutional layers perform the opposite operation of convolutional layers by up-sampling and adding redundancies. The number of trainable parameters for convolutional and transposed convolutional layers is significantly lower than similar LSTM and dense layers. The drawback is slight degradation is performance.

Figure 5: An autoencoder with encoder using Conv layers and decoder using TransConv layers

Table 5: Trainable parameters of the autoencoder with Conv/Conv layers

| Input size | Feedback bits (L) | Encoder Parameters |  |  | Decoder Parameters |  |  | Cosine similarity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16× 13 | 130 bits | Conv 1 | Conv 2 | Dense 3 | Conv 1 | Conv 2 | Dense 3 | 0.942 |
|  |  | 20 | 19 | 14,170 | 80 | 73 | 49,712 |  |
|  |  | 14,209 |  |  | 49,865 |  |  |  |

Summary of described autoencoder designs is listed in Table 6. The convolutional encoder and dense decoder provide the best performance in terms of cosine similarity.  This design can be feasible for training mode 2 where only encoder’s trained parameters need to be exchanged. However, the decoder has large number of trainable parameters which make it infeasible for training mode 3.  On the other hand, Conv/Conv design provides the least number of trainable parameters both at encoder and decoder at the expense of slight performance degradation.

Table 6: Summary of suggested autoencoder designs: performance vs trainable parameters

| Autoencoder Structure | Encoder Parameters | Decoder Parameters | Cosine similarity |
| --- | --- | --- | --- |
| Dense Encoder, Dense Decoder | 975,234 | 975,312 | 0.941 |
| LSTM Encoder, Dense Decoder | 59,102 | 975,312 | 0.981 |
| Conv Encoder, Dense Decoder | 14,209 | 975,312 | 0.984 |
| LSTM Encoder, LSTM Decoder | 59,102 | 60,752 | 0.969 |
| Conv Encoder, Conv Decoder | 14,209 | 49,865 | 0.942 |

Observation 2: An autoencoder with convolutional encoder and dense decoder provides the best performance in terms of cosine similarity. However, large number of trainable parameters at decoder impose large overhead.

Observation 3: An autoencoder with convolutional encoder and transposed convolutional decoder has the least number of trainable parameters which makes it suitable for use cases that require exchanging parameters.

# Conclusions

In this contribution, we evaluated and compared various NN layers considering the number of trainable parameters. The number of trainable parameters not only affect the training complexity but also the exchange overhead for use cases that trained parameters need to be transferred from gNB to UE or vice versa. Particularly, for CSI feedback use case where the encoder and decoder should be optimized jointly, evaluating the exchange overhead has more significance. We showed that using convolutional layers at encoder and transposed convolutional layers at decoder provide least number of trainable parameters while enjoying comparable performance compared with other suggested designs.

Observation 1:  Convolutional NNs (CNNs) are a suitable choice for designing ML techniques in given use cases. CNNs are capable of feature extracting from multi-dimensional data (spatial, frequency and time) and enjoy low number of trainable parameters which can reduce the training complexity and also exchanging overhead.

Observation 2: An autoencoder with convolutional encoder and dense decoder provides the best performance in terms of cosine similarity. However, large number of trainable parameters at decoder impose large overhead.

Observation 3: An autoencoder with convolutional encoder and transposed convolutional decoder has the least number of trainable parameters which makes it suitable for use cases that require exchanging parameters.

# References

[1] RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e