3GPP TSG RAN WG1 #109-e			R1-2204068

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.3.1

Source:	InterDigital, Inc.

Title:	Evaluation assumption and methodology for study on NR-duplex

Document for:	Discussion and Decision

# Introduction

For Rel-18, a study item on evolution of NR duplex operation has been approved [1], where the objectives identified for the study item are as follows:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss evaluation assumption and methodology for study on NR-duplex.

# Discussions

## Deployment scenarios and evaluation methodology

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-17, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-18, enhancements to support full duplex (FD) at least for gNB have been proposed and endorsed as the study item, see Figure 1. Moreover, subband non-overlapping FD (SNFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on subband non-overlapping FD-gNB and HD-UEs in a cell

In subband non-overlapping FD, considering a single UE perspective, a UE in a cell illustrated in Figure 2 can be informed of the schedule mixed D/U transmission patterns across RBs per symbol/slot, where the UE can perform an UL Tx when the scheduled UL Tx is matched with an UL region of the mixed D/U transmission patterns on a given symbol/slot. For example, the UE can perform UL Tx on scheduled RBs across the adjacent 3 slots, as shown in Figure 2, which results in an UL coverage/capacity enhancement. As for the cell perspective, parallel DL transmissions toward other UEs over scheduled non-overlapping RBs can be performed by the FD-gNB.

In subband non-overlapping FD, the UEs that are close may experience inter-subband interference leakage. As such, it is necessary to investigate subband non-overlapping scenarios to examine the potential benefits of SNFD. On the other hand, the studies have shown that scenarios based on fully-overlapping or partial-overlapping subband FD can be beneficial and achieve higher gains in specific traffic topologies, e.g., in small cells. Thus, the evaluations can be based on subband non-overlapping, subband partial overlapping, as well as subband overlapping scenarios.

Observation 1. Scenarios on subband non-overlapping (as for inter-subband CLI), subband partial overlapping and subband overlapping (as for intra-subband CLI) may achieve different gains based on at least traffic and/or cell sizes.

Proposal 1. Consider evaluating achieved gain and performance in subband non-overlapping scenario based on inter-subband CLI, and also in subband partial overlapping and subband overlapping scenarios based on intra-subband CLI.

Moreover, Rel-18 NR-duplex study should consider both FR1 and FR2 based on the above deployment scenarios taking both intra- and inter-subband CLI effects into account for performance evaluation. Especially for the inter-subband CLI analysis, it should consider TX leakage power and effects on the rest of the RBs especially for adjacent RBs, depending on the actual resource allocation of the Tx in SNFD scenario.

Proposal 2. Consider evaluating intra- and inter-subband CLI in both FR1 and FR2 frequency ranges.

Considering that the performance of the subband overlapping and subband non-overlapping scenarios depends on the traffic, cell size, etc., the evaluation can be performed on urban macro, micro, small cell, and indoor layouts. However, studies have shown [2] that the indoor scenarios where dropped UEs are in the same building can be the most important scenario which best presents the UE-to-UE CLI effects.

Proposal 3. Urban macro and indoor scenarios can be considered for evaluations in this study, and the indoor scenarios should be prioritized which have the most significant UE-to-UE CLI effects.

In the following sections, simulation assumptions and methodology based on our initial link-level and system-level simulation results are provided. In link-level simulations, we have addressed the impact of inter-subband and intra-subband CLI on the performance of the victim UE. The results show the importance of such evaluations, as the results can be used to identify use cases and future target scenarios.

In system-level simulations, we have addressed the overall performance trade-off between DL and UL, in the presence of dynamic inter-cell CLI from neighbour cell(s) per slot basis. The results are provided in our companion contribution [4]. It is observed that while the performance depends on considered traffic model and DL/UL resource allocation scheme, application of CLI mitigation scheme is necessary for reliable communication in the presence of CLI.

## LLS simulation assumptions

In this subsection, we discuss LLS simulation assumptions and methodology based on our initial LLS results for study on NR-duplex. We conduct the LLS to address the potential negative impacts on DL reception on a subband (e.g., RBs), when intra-subband CLI on the same overlapped subband or inter-subband CLI on adjacent or non-overlapped subband exists due to nearby other UE’s uplink transmissions. Simulation assumptions for the LLS are summarized in Table 1.

We consider a duplexing scheme where the DL signal can be considered to be a couple of dBs less than, greater than or equal to the UL (CLI) signal. For a given UL (CLI) RB allocation, we evaluate the DL receiver performance for various RB allocations over the system bandwidth considering several degrees of intra and inter subband CLI overlaps as shown in Figure 3. For the initial results presented, no signal impairments due to PA nonlinearity or other imperfections are considered, where such impairments may further increase the adjacent inter-subband CLI level.

Table 1. LLS simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 |
| Antenna Configuration | 1Tx-2Rx |
| Carrier Frequency | 4GHz |
| SCS | 15kHz |
| MCS | 4, 10, 19 and 27 |
| System BW | 20MHz |
| Allocated no. of RBs for UL and DL signals | DL (Victim): 1RB, 2RBs, 5RBs, 10 RBs, 25RBs, 50RBs |
|  | UL (Aggressor): 25RBs |
| DL to UL power ratio (dB) | -6dB, 0dB, 6dB, 18dB |
| UL timing advance over DL | 0 , 1/2 of a symbol, 1/4 of a symbol |
| Propagation condition | TDL-A |
| Delay Spread | 30ns |
| UE Velocity | 3km/h |
| DMRS | DM-RS type-1, # of DMRS 1 + 1 |
| Channel estimation | Realistic |
| Receiver Type | MMSE |
| PA nonlinearity | None |

The DL throughput performance is compared with a corresponding DL throughput performance with no intra/inter-subband CLI.

Figure 3. Normalized signal spectrum of UL and DL signals

Figure 3 shows the normalized power spectrum of 5MHz (25-RBs) DL signal when the signal is scheduled at different subband locations (shown in pink, sky blue, red and blue colors) in a 20MHz channel bandwidth relative to a 5MHz UL signal (green color). The UL signal introduces intra-subband CLI on the DL signal when it is fully or partially overlapped with the UL signal as shown by the pink and sky blue colors respectively. On the other hand, an inter-band CLI is introduced on the DL signal, shown by the red and blue colors, which are adjacent to a UL signal.

Figure 4. DL Throughput performance under various intra-subband CLI

Figure 4 shows throughput performance curves of a DL signal for various degrees (100%, 50% and 25%) of overlap with an UL signal introducing intra-subband CLI. It can be seen that DL throughput performance suffers considerably, below 40% of maximum throughput value and may even be zero, as a result of intra-subband CLI when there is an overlap in DL and UL subbands. Further results for this evaluation can be found in our companion contribution [3].

In addition, a realistic scenario would be to consider a timing advance (TA) on the UL signal originating from a neighboring cell. The timing advance can be a fraction of an OFDM symbol as demonstrated in Figure 5. The resulting effect of the TA on the UL signal may be to introduce inter-carrier interference on the UL signal and as a result increase the level of the inter-subband CLI.

TA

Slot [1-ms]

Figure 5. An example demonstrating TA on a UL signal

Observation 2: DL throughput performance suffers considerably as a result of intra-subband CLI when there is an overlap in DL and UL subbands.

## SLS simulation assumptions

In this subsection, we discuss SLS simulation assumptions and methodology based on our initial SLS results for study on NR-duplex. We conduct the SLS to see how much of a negative impact on DL and UL reception in a cell can be observed, when inter-cell CLI from neighbour cell(s) may be present dynamically per slot basis. We consider an Indoor office scenario, details of which are provided in Table 2. For the evaluation, we compare performance of a baseline TDD scheme, where all gNBs/cells in the deployment use an aligned (common) TDD configuration, with a flexible duplex scheme where each gNB can independently and dynamically select the TDD configuration that will best serve its traffic needs.

Details regarding the TDD configurations can be found in Table 3. No CLI mitigation schemes are employed for these simulations. In addition, there is no UL power control and all UEs transmit at the maximum transmit power. The results for this evaluation can be found in our companion contribution [4]. Our results highlight the detrimental effects of CLI on both DL and UL UPT, highlighting the need for effective CLI management schemes.

Table 2. Indoor deployment scenario

|  | Indoor Sub-7GHz |
| --- | --- |
| Layout | (a,b,c,d)=(20,40,20,40) |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | 10 per gNB per 20MHz |
| SCS | 30 KHz |
| Channel Model | NR InH Mixed Office model |
| BS Tx Power | 23dBm (total across all TX antennas) |
| UE Tx Power | 23dBm (total across all TX antennas) |
| BS Antenna gain | 0dBi |
| UE Antenna gain | 0 dBi |
| BS Noise Figure | 5dB |
| UE Receiver Noise Figure | 9dB |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | According to 36.889 Table A.1.1. DL/UL FTP traffic – DL/UL traffic ratio: 1/1 and 2/1 FTP |
| UE to UE link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Table 3: TDD configuration parameters

|  | Baseline (static TDD) | Flexible duplex |
| --- | --- | --- |
| DL/UL subframe ratio | Semi-static subframe allocation - For DL:UL traffic ratio = 1:1, DL/UL subframe = 6:4 - For DL:UL traffic ratio = 2:1, DL/UL subframe = 6:4 | Flexible UL/DL subframe ratio allocation. gNB can choose between following additional DL/UL subframe options: DL heavy: 9:1, 8:2, 7:3 UL heavy: 4:6 |

# Summary

In this contribution, we discussed link-level and system-level evaluation assumption and methodology for study on NR-duplex. From the discussions, we made following observations and proposals:

Observation 1. Scenarios on subband non-overlapping (as for inter-subband CLI), subband partial overlapping and subband overlapping (as for intra-subband CLI) may achieve different gains based on at least traffic and/or cell sizes.

Observation 2: DL throughput performance suffers considerably as a result of intra-subband CLI when there is an overlap in DL and UL subbands.

Proposal 1. Consider evaluating achieved gain and performance in subband non-overlapping scenario based on inter-subband CLI, and also in subband partial overlapping and subband overlapping scenarios based on intra-subband CLI.

Proposal 2. Consider evaluating intra- and inter-subband CLI in both FR1 and FR2 frequency ranges.

Proposal 3. Urban macro and indoor scenarios can be considered for evaluations in this study, and the indoor scenarios should be prioritized which have the most significant UE-to-UE CLI effects.

# References

- RP-220633, “Revised SID: Study on evolution of NR duplex operation”, CMCC.

- R1-1611679, “Evaluation method of flexible duplex”, Huawei, HiSilicon.

- R1-2204069, “Discussion on subband non-overlapping full duplex”, InterDigital.

- R1-2204070, “Discussion on enhancements of dynamic TDD operations”, InterDigital.