3GPP TSG RAN WG1 Meeting #109-e					            R1-2204071

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.7.1

Source:	Panasonic

Title:	Remaining issues on Enhancements for idle/inactive-mode UE power saving

Document for:	Discussion/Decision

# Introduction

On the enhancements for IDLE/INACTVE UEs, including both paging and TRS/CSI-RS occasion(s), the conclusions/agreements in the appendix were achieved in RAN1#108-e meeting [1]:

This contribution provides our views particularly on the remaining details of enhancement for TRS/CSI-RS occasion(s) for idle/inactive UEs.

# Discussion

On UE default assumption of TRS availability

In the previous meeting, it was discussed whether/how to specify UE default assumption of TRS availability in the below cases:

- C1: before UE receives L1 availability indication after camping on a cell

- C2: the TRS resource configuration is changed by SI update procedure

- C3: During the system modification period and before the new SIBs become valid

In our view, although the TRS resource should be regarded not reliable for AGC and T/F synchronization, it is not necessary to specify the UE behaviour on whether to assume the TRS is available or not. The reasons are:

- The above cases take small portion of the whole UE operation time and thus the gain for power saving is neglectable.

- To mandatory specify the UE behaviour would prevent UE from possible smart implementation to handle these cases to achieve better performance. For example, if gNB shares the TRS in a more static or long term way, UE may detect by using previous configuration/information and estimate the TRS actually availability. Such possibility should be up to UE implementation. UE may choose whether to assume the TRS is available by its own strategy.

Therefore,

Proposal 1: The UE can assume the configured group(s) of TRS resource set(s) are unavailable in below cases:

- C1: before UE receives L1 availability indication after camping on a cell

- C2: the TRS resource configuration is changed by SI update procedure

- C3: During the system modification period and before the new SIBs become valid

On whether/how to align the reference point for paging and PEI based TRS availability indication

Regarding the reference time of the start of validity duration for TRS availability indication, the below agreement ruled out using the reference time of the MO of paging DCI.

Conclusion No consensus to support - For PEI based availability indication, the reference time of the start of validity duration follows the reference time for associated monitoring occasion for the paging DCI.

Two options to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication are left for further down selection:

- starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets,

- starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets,

Basically the first option is more aligned with previous agreement that the reference time of the start of validity duration follows the occasion of the L1 indication, which is either the PEI or paging DCI. So UE behaviour possibilities are:

- If UE detects only PEI, reference time follows PEI

- If UE detects both PEI and paging DCI, reference time firstly follows PEI before detecting paging DCI. Then validity duration may be updated with reference time following PO.

- If UE does not detect PEI but only detects paging DCI, reference time follows PO

The second option means UE follows the first occasion between PEI-O and PO, which also has three possibilities:

- If UE detects only PEI, reference time follows PEI

- If UE detects both PEI and paging DCI, reference time follows PEI

- If UE does not detect PEI but only detects paging DCI, reference time follows PO

Although the first option potential needs more UE operation, network has more opportunities to update the TRS availability. In addition, it was also agreed that the L1 indication in PEI and paging DCI are not necessarily expected by UE to be consistent. Also first option is more aligned with the previous agreement. Hence,

Proposal 2: Adopt the TP#1 in below.

============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: The UE can assume the configured group(s) of TRS resource set(s) are unavailable in below cases:

- C1: before UE receives L1 availability indication after camping on a cell

- C2: the TRS resource configuration is changed by SI update procedure

- C3: During the system modification period and before the new SIBs become valid

Proposal 2: Adopt the TP#1 in below.

============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================

# Reference

[1] RAN1 Chair’s Notes of 3GPP RAN1#108-e

# Appendix

Agreements achieved in RAN1#108-e meeting

Conclusion

For Rel-17, the following for paging early indication design are not supported:

- Multipe PEI-Os for one PO

- UE assistance information for preferred offset between PEI-O and PO

- Different PEI configurations for different devices types

Agreement

The following TP for 38.202-h00, subclause 6 to reflect reception type with PEI-RNTI is endorsed.

<Begin TP for 38.202-h00, subclause 6.2>

Table 6.2-1: Downlink "Reception Types"

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel | Comment |
| --- | --- | --- | --- | --- |
| <unchanged rows omitted for brevity> |  |  |  |  |
| P | PDCCH | CI-RNTI | N/A |  |
| Q | PDCCH | PEI-RNTI | N/A | Note1 |

Table 6.2-2: Downlink "Reception Type" combinations

| Supported Combinations |  |  | Comment |
| --- | --- | --- | --- |
| PCell | PSCell | SCell |  |
| 1. RRC_IDLE |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  | Note 1 |
| 2. RRC_INACTIVE |  |  |  |
| A + (B and/or (C1 or Q) and/or D0) + F0 |  |  | Note 1 |
| < unchanged rows omitted for brevity> |  |  |  |

<End TP for 38.202-h00, subclause 6.2>

Agreement

For the alignment with the following RAN2#116bis-e agreements,

Þ Both subgroupNumPerPO and Nsg-UEID range from 1 to 8. Þ If network supports PEI but not subgrouping, the wholeSubgroupConfig-r17 is absent. The parameter subgroupsNumPerPO is mandatory present if subgroupConfig-r17 is configured.

- Adopt following TP to TS38.213 Section 10.4:

TS38.213v17.0.0] 10.4 A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, , by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, , by PONumPerPEI A paging indication field of DCI format 2_7 includes segments of bits, where if and if is not providedconfigured or . For a subgroup index , , a UE determines a value for the bit in the paging indication field, where is a paging occasion index, and , , , and are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion.

- Adopt following TP to TS38.212 Section 7.3.1.3.8:

[TS38.212v17.0.0] 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – bit(s), where - is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - is the number of sub-groups of a paging occasion configured by higher layer parametersubgroupsNumPerPO, if subgroupsNumPerPO is configured and not set to 0; otherwise is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPOsubgroupsNumPerPO is configured and not set to 0; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.

Agreement

Adopt the following TP to Section 10 of TS 38.213

- 10 UE procedure for receiving control information *** Unchanged text is omitted *** If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2/2A-PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG - When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively. - When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG. *** Unchanged text is omitted ***

Conclusion

SFN of the first PF of the PF(s) associated with the PEI-O is provided by (SFN of UE’s PF) -

Agreement

- Confirm the following working assumption:

Working assumption SearchSpaceId = 0 can be configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3

- When SearchSpaceId = 0 is configured for peiSearchSpace, subject to SS/PBCH block and CORESET multiplexing pattern 2 or 3, the PDCCH monitoring occasions for PEI-O are same as for RMSI as defined in clause 13 in TS 38.213.

- UE determines first PDCCH MO for PEI-O based on PEI-F_offset and firstPDCCH-MonitoringOccasionOfPEI-O, as previously agreed for the case with SearchSpaceId > 0.

  - Note: UE expects the first PDCCH MO for PEI-O determined from PEI-F_offset and firstPDCCH-MonitoringOccasionOfPEI-O is aligned with the MOs for RMSI as defined in clause 13 in TS 38.213.

Agreement

The following TP to TS 38.213 is endorsed

10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 if configured or and a DCI format 1_0 with CRC scrambled by P-RNTI includes a the TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided bydefaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication fieldindicating the TRS resource sets, where is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE indicates that the UE keeps existing assumption of the availability or unavailability of associated TRS resource sets. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than themultiple of the number of frames. *** Unchanged text is omitted ***

Agreement

The following TP to TS 38.214 is endorsed

5.1.6.1.1 CSI-RS for tracking <omitted text> Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by numberOfresources - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomainin the TRS-ResourceSet and the second symbol location in a slot isfirstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. The frequencyDomainAllocation configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameterperiodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. - the UE does not expect the TRS-ResourceSet to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured byTRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more scramblingIDs where if a single scramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scramblingID. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID. <omitted text>

Conclusion

No consensus to support

- PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion

Conclusion

A UE can receive different TRS availability indication content from PEI and associated paging PDCCH.

- Note: It is up to UE implementation whether UE receives both PEI and the associated paging PDCCH to receive TRS availability indication

Conclusion

No consensus to support

- For PEI based availability indication, the reference time of the start of validity duration follows the reference time for associated monitoring occasion for the paging DCI.

Agreement

Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication

- Alt1: Adopt TP#1

- Alt2: Adopt TP#2

- Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same.

- Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value.

============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================

Agreement

TRS-ResourceSetConfig IE is not accounted in determination of available resource elements for PDSCH.