3GPP TSG-RAN WG1 #109-e 	R1-2204072

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.7.2

Source:	Panasonic

Title:	Remaining issues for extensions to Rel-16 DCI-based power saving adaptation during DRX Active Time

Document for:	Discussion/Decision

# Introduction

- In RAN1#108-e meeting, the agreements [1] in the appendix were achieved regarding extensions to Rel-16 DCI-based power saving adaptation.

In this contribution, we share our views on remaining issues of the common design of search space set group (SSSG) switching and PDCCH skipping.

# Discussion

## Potential additional cases for PDCCH skipping

Type 0/0A/1/2 CSS

It has been agreed that the PDCCH based monitoring adaptation is applied to USS and type-3 CSS. When PDCCH skipping is indicated, it is still FFS on whether to continue monitoring PDCCH scrambled by C-RNTI for Type 0/0A/1/2 CSS or not.

For UE power saving aspect, allowing UE to continue monitoring PDCCH will lower the power saving gain. However, as the skipping occasions are seldom compared with Type 3 CSS + USS, the compromised power saving gain is marginal. From the network side, some opportunity to schedule UE even in the skipping period has benefit to stability of system operation. Thus this can be up to gNB implementation and the level of monitoring activity should be sufficiently low. In addition, applying skipping only for Type 3 CSS + USS is aligned with the supported search spaces used for SSSG switching. This is more aligned with common framework for PDCCH skipping and SSSG switching. This is beneficial for clearer network implementation with lower complexity.

Proposal 1: PDCCH skipping is not applied to Type 0/0A/1/2 CSS.

CS-RNTI for PDCCH skipping

So far the PDCCH skipping applies to at least the PDCCH with CRC scrambled by C-RNTI and MCS-C-RNTI. On the other hand, whether CS-RNTI should also be skipped needs to be discussed and clarified for UE power saving gain. For UEs configured with SPS and/or CG type 2, monitoring PDCCH is always needed for activation and deactivation. If this is allowed when PDCCH skipped is being applied, the power saving gain can be degraded. Thus, to make the design simple and effective, we prefer the PDCCH skipping also apply to CS-RNTI.

Proposal 2: PDCCH skipping applies to C-RNTI, MCS-C-RNTI and CS-RNTI in Type 3 CSS and USS.

In case a certain SPS configuration is active and UE indicated with PDCCH skipping, the UE behaviour should also be clarified that whether UE shall skip monitoring the SPS PDSCH. This has not clarified. For more aligned behaviour of skipping receiving signal from gNB, SPS PDSCH should be skipped. If service continuity needs to be guaranteed, UE should continue receiving the SPS PDSCH of the active SPS configuration. As a comparable UE behavior impacting power saving gain, it has been clarified that RAN1 understanding is CSI/SRS is not affected by PDCCH skipping. Therefore, we may also follow similar principle that already active SPS configurations are not affected. The similar discussion would be applied for CG transmission of PUSCH.

Proposal 3: It needs to be clarified when PDCCH skipping is applied, whether SPS/CG is affected. For simpler UE behavior, active SPS/CG is not affected.

PDCCH skipping termination by SR

In the below agreement, it shows that PDCCH skipping is not applied when SR is pending. RAN1 needs to further study on how to capture the behaviour regarding PDCCH skipping after a positive SR is transmitted in PUCCH.

Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. o It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE can stops PDCCH skipping and monitor PDCCH for UL grant (i.e., PDCCH skipping is not activated ).

In our view, the RAN2 agreement only describes that the PDCCH skipping is not applied when SR is pending. So basically the following issues are still open:

- Whether the PDCCH skipping duration decrement is still ongoing when SR is pending.

  - If not, whether the PDCCH skipping should be terminated and the skipping duration is reset to 0. Or the PDCCH skipping is pending and will be resumed again after pending SR is released.

  - If yes, UE may still needs to skip PDCCH after the pending SR is released if the skipping duration is not yet decremented to zero.

Our current view is UE stops PDCCH skipping because UE side dealing with duration calculation based on SR may not be synchronized with gNB side if gNB miss detects the SR. So a simple handling as in the FFS bullet in the agreement is preferred.

Proposal 4: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping  (i.e., PDCCH skipping is not activated deactivated).

## On SSSG switching

There is a working assumption in the below agreement of RAN1#108e meeting regarding the condition to reset the searchSpaceSwitchTimer-r17.

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

In our view, the working assumption should be confirmed, especially including the PDCCH reception with CRC scrambled by CS-RNTI. Because it is able to cover all the possible cases that UE detects a PDCCH in any possible search space. This makes UE stay in the current SSSG. To include the CS-RNTI is also beneficial to have more aligned and simpler design for PDCCH skipping, for which we also propose to skip CS-RNTI.

Proposal 5: Confirm the working assumption that: If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: PDCCH skipping is not applied to Type 0/0A/1/2 CSS.

Proposal 2: PDCCH skipping applies to C-RNTI, MCS-C-RNTI and CS-RNTI in Type 3 CSS and USS.

Proposal 3: It needs to be clarified when PDCCH skipping is applied, whether SPS/CG is affected. For simpler UE behavior, active SPS/CG is not affected.

Proposal 4: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping  (i.e., PDCCH skipping is not activated deactivated).

# Reference

[1] RAN1 Chair’s Notes for 3GPP RAN1#108-e

# Appendix:

Agreements achieved in RAN1#108-e meeting:

Working Assumption (Excerpted from RAN#107bis-e)

- The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS

Agreement

-             The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation.

Agreement

- It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values.

Agreement

For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2,

- 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-     1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

Note: Same as in existing specifications, specifications allow gNB to configure a search-space set with zero PDCCH candidates.

Agreement

Answer 1 for R1-2200884 (R2-2201960):

Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e.

Agreement

Answer 2 for R1-2200884 (R2-2201960):

RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification.

o    It is RAN1 understanding  PDCCH skipping  is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending.

- FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE can stops PDCCH skipping and monitor PDCCH for UL grant (i.e., PDCCH skipping is not activated ).

Agreement

Answer 4 for R1-2200884 (R2-2201960):

·        It is RAN1’s understanding that transmission of CSI/SRS is not affected by PDCCH skipping i.e. CSI/SRS is still transmitted during the skipping duration.

Agreement

·         It is RAN1 understanding that for DRX operation,

  - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time.

Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

Agreement

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE

- Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by

- Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI

- Otherwise, decrease the timer value by one after each slot.

- Note: There is no timer for SSSG switching is running for UE in SSSG#0

Agreement

[Draft] LS reply on PDCCH skipping, R1-2202896 is endorsed in principle.

Agreement

Final LS reply on PDCCH skipping, R1-2202905 is endorsed

Working Assumption

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

Agreement

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes

  - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

    - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

Agreement

-     When the timer expires, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 in a slot

  - no earlier than [application delay] after the slot where timer expires and

  - if PDCCH skipping is being applied, no earlier than in slot where PDCCH skipping duration expires.

Working assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.