3GPP TSG-RAN WG1 #109-e 	R1-2204074

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.2

Source:	Panasonic

Title:	Discussion on potential network energy saving techniques

Document for:	Discussion/Decision

# Introduction

- In RAN#94-e meeting, the new study item on network energy savings for NR was updated [1].

In this contribution, we share our views on the potential network energy saving techniques.

# Discussion

## On backward compatibility

The below note is mentioned in the SID [1].

Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

As an important aspect for new scheme operation to achieve network energy saving, backward compatibility can be looked after in below possible cases:

Case 1: Some new schemes may not lead to any air interface change but only require new network implementation and/or new X2/S1 signaling. No modification for UE is required.

Case 2: New mechanism leads to some air interface specification impact.

- Case 2.1: The legacy UEs can still work normally even when the serving gNB operates with new mechanism for network energy saving, e.g. gNB adapts resource but the changing does not impact or not visible for legacy UEs.

- Case 2.2: New scheme of adaptation is only supposed to operate for Rel.18 UEs within a certain cell or BWP. Legacy UE may be impacted due to the gNB adaptation. So legacy UEs may be barred to the cell but would be operated in the other cell.

Our understanding of the SID is that, all above cases are within the scope. From RAN1 perspective, coexisted operation of Case 2.1 and 2.2 should be ensured.

Proposal 1: Network energy saving mechanism should either ensure backward compatibility or is able to coexist with scheme serving legacy UEs.

## On example scenarios for network energy saving and network energy saving techniques

SID provides below four typical example scenarios for investigation of network energy saving:

- Scenario 1: Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- Scenario 2: FR2 beam-based scenarios (note: this scenario can also model small cells)

- Scenario 3: Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- Scenario 4: EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

The network adaptation techniques may be applied in time, frequency, spatial and also power domain. Below table provides our high level view on potential options to achieve energy saving by network adaptation in different domains and potential applicability in each scenario. As power domain is the combination between frequency domain and PSD domain, for the orthogonality of the domains, PSD domain adaptation is described.

Table 1. High level views on potential options for network traffic adaption to save energy

| - Adaptation techniques |  | - Applicable scenarios |
| --- | --- | --- |
| - Time domain adaptation | - TDD configuration can be adapted to traffic load. Some ‘Idle symbols/slots’ can be achieved by gNB to step into sleep modes | - 1~4 |
| - Frequency domain adaptation | - Multiple carrier operation using DC/CA. CC activation/ deactivation and cell on/off can be employed for traffic adaptation. - BWP framework can also be used to adapt to traffic in a cell. | - 1~4 |
| - Antenna domain adaptation | - In general, the number of Tx/Rx chains can be adapted as per needed. In addition, active antenna elements/panels may also be changed. | - 1, 2, 4 |
|  | - When gNB operates beam sweeping, time domain adaptation can be combined with antenna domain adaptation so that beam sweeping patterns are adaptive to the traffic, UE distributions and coverage requirement. This can be achieved with or without adapting the number of Tx/Rx chains, antenna elements/panels. | - 2, 4 |
| - PSD domain adaptation | - Based on the traffic, UE loads and required coverage, the maximum number of assigned PRBs and maximum PSD may be reduced. This contributes to reduce the maximum transmission power of gNB and it may save the RF amplifier energy consumption. The adjustment of the required coverage needs some coordination with other carriers and/or neighbour cells in order to avoid the situation of "out of the coverage" for UEs. | - 1, 2, 4 |

- Therefore, based on the above table, our view is:

- Proposal 2: For network energy saving study at initial stage, the high level scheme for adaptation in time, frequency, antenna/spatial and power domain as shown in Table.1 should be investigated. The applicable scenarios for each adaptation technique should also be clarified.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: Network energy saving mechanism should either ensure backward compatibility or is able to coexist with scheme serving legacy UEs.

- Proposal 2: For network energy saving study at initial stage, the high level scheme for adaptation in time, frequency, antenna/spatial and power domain as shown in Table.1 should be investigated. The applicable scenarios for each adaptation technique should also be clarified.

# Reference

[1] RP-213554 New SI: Study on network energy savings for NR

[2] 3GPP TR 38.840 V16.0.0, Study on User Equipment (UE) power saving in NR (Release 16)

# Appendix:

The objectives of SID RP-213554 are quoted below:

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study.

The following example scenarios are listed in no particular order.

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

Note 2: the study of energy savings specifically for IAB is not part of the scope.

The study should coordinate with RAN4 as needed.