3GPP TSG-RAN WG1 #109-e 	R1-2204075

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.2

Source:	Panasonic

Title:	Remaining issues on PDCCH monitoring

Document for:	Discussion, Decision

# Introduction

In this paper, we discuss some remaining issues from previous meeting RAN1#108-e, as captured in Feature Lead Summary [1].

# Discussion

Issue A1-3: Determination of X in case of multiple supported X values for multi-slot monitoring

The following working assumption is the outcome from previous meeting:

Working assumption For the purpose of BD/CCE budget determination for PDCCH monitoring for μ=6, if more than one of the PDCCH monitoring (Xs,Ys) capability combinations supported by a UE comply with all configured search space sets, then the UE selects Xs and thereby MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values corresponding to the complying combination with the largest MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values. Note1: This determination of Xs may be independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration.

One controversial point that prevents the above WA to an agreement is that, companies seem to have different opinions on whether the “active” combination of (Xs,Ys) should be determined by “all configured search space sets”, especially when search space set group (SSSG) switching is configured/enabled. For example, it seems unreasonable to consider the SSSG that has been switched off, because those inactive SSSG would not contribute to the BD/CCE budget. From this logic, some companies suggested to change the above WA into “…by a UE comply with all configured active search space sets within an active BWP”. However, the other camp think the (Xs,Ys) combination should be determined by all configured search space sets, regardless of SSSG switching. In essence, these companies believe that the selection of (Xs,Ys) is fixed with a BWP.

Although we recognize the merit of the first view to fully utilize BD/CCE budget, we support the second view – the selection of (Xs,Ys) is fixed with a BWP. The reason is that, if BD/CCE budget is influenced by the SSSG switching operation as the consequence of the first view, the current specified SSSG switching time needs to be revisited in order to take such process time into account. It is not desirable at this late maintenance stage. Furthermore, it is also unfavourable to increase the SSSG switching delay too much compared to existing values, because SSSG switching is aimed at a fast/simple operation (compared to BWP switching). Instead, switching between different combination of (Xs,Ys) can be done via BWP switching.

Therefore, we propose to confirm the above WA.

Proposal 1: Confirm the WA on Determination of X in case of multiple supported X values for multi-slot monitoring.

Issue A2-1: Search space configuration and enhancement

The agreement from previous meeting is as follows.

Agreement Revise the RAN1#107bis-e agreement/working assumption: For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values that can be configured for SCS 480kHz and 960kHz: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1} slots Offset O values that can be configured: {0, 4, 8, …, 4(Xp-1)4} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of XsL slots - FFS: details of offset The configured offset is restricted to be an integer multiple of L slots - For duration-r17 - The values represent slots - The value range is {8, 12, …, 20476} - If duration-r17 is absent, the UE assumes the duration in slots is equal to L - The UE ignores duration-r17 for DCI format 2_0 - The configured duration is restricted to be an integer multiple of XsL slots at least for Group (1) SSs - The maximum duration is one less slot group of L slots than the configured periodicity - FFS: need to revise the definition of durationduration-r17 is the total number of slots in consecutive groups of L slots in which a Search Space can exist in every period as given by monitoringPeriodicityAndOffset-r17 - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bitsTwo sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot each group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - For Group (1) SSs - The slots indicated in the bitmap should be consecutive per group of L slots - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE - Working assumption: For Group (2) SSs - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L - For Type1 CSS without dedicated RRC - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

The last bullet point (i.e. WA) provides additional restriction on Group (2) SS. Some concern was raised in the previous meeting regarding different handling between Type 1 and Type 0/0A/2 CSSs. In our understanding, such handling might be necessary. The reason is that, for the Type 0/0A/2 CSSs, UE only needs to monitor one slot corresponding to its serving beam although the monitoring occasions (for all beams) can be configured up to L slots, but this is not the case for Type-1 CSS. So the additional constraint (using M parameter in the WA) might be needed to reduce the UE monitoring effort.

Therefore, we support to confirm the WA.

Proposal 2: Confirm the WA on Group(2) SS configuration.

Issue A2-6: SS set group switching configuration with same or different (Xs, Ys)

In the previous discussion, companies supporting SSSG switching with different (Xs,Ys) argue the merit of flexibility, whereas some other companies brought up concerns on the switching timeline and corresponding P_switch values in case the switch involves different combinations of (Xs,Ys).

From our understanding, we need to first clarify whether “active” (Xs,Ys) combination is determined from all configured search space sets per BWP, or from search space sets of the active SSSG. We support the first view – the active (Xs,Ys) combination is fixed per BWP, regardless SS set is active or not. (Please see the reason above to address the issue A1-3). Therefore, SSSG switching within one BWP does not result in the change of (Xs,Ys), and hence with no impact on BD/CCE determination. If SSSG switching occurs between different BWPs (e.g. scheduling DCI indicates a different BWP), it could result in different (Xs,Ys) .

Proposal 3: In case that UE supports multiple combinations of (Xs,Ys), the active (Xs,Ys) combination is determined by all configured search space sets of a BWP.

- SSSG switching within a BWP does not change the active combination of (Xs,Ys)

- SSSG switching across different BWPs may change the active combination of (Xs,Ys)

# Conclusion

In this paper, the following proposals are made:

Proposal 1: Confirm the WA on Determination of X in case of multiple supported X values for multi-slot monitoring.

Working assumption For the purpose of BD/CCE budget determination for PDCCH monitoring for μ=6, if more than one of the PDCCH monitoring (Xs,Ys) capability combinations supported by a UE comply with all configured search space sets, then the UE selects Xs and thereby MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values corresponding to the complying combination with the largest MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ values. Note1: This determination of Xs may be independent of the size of monitoringSlotsWithinSlotGroup discussed in the context of search space configuration.

Proposal 2: Confirm the WA on Group(2) SS configuration.

- Working assumption: For Group (2) SSs - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L - For Type1 CSS without dedicated RRC - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

Proposal 3: In case that UE supports multiple combinations of (Xs,Ys), the active (Xs,Ys) combination is determined by all configured search space sets of a BWP.

- SSSG switching within a BWP does not change the active combination of (Xs,Ys)

- SSSG switching across different BWPs may change the active combination of (Xs,Ys)

# Reference

[1] R1-2202713, Feature lead summary #4 for B52.6 GHz PDCCH monitoring enhancements, Moderator (Lenovo)