3GPP TSG-RAN WG1 #109-e 	R1-2204077

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.1

Source:	Panasonic

Title:	General aspects of AI/ML framework

Document for:	Discussion, Decision

# Introduction

A new Study Item “Study on Artificial Intelligence and Machine Learning for NR Air Interface” has recently approved for Rel-18 [1]. This paper is to discuss some general aspects of AI/ML, including functional framework and categorization of network-UE collaboration.

# Functional framework

In our opinion, the basic framework and principles agreed for FS_NR_ENDC_data_collect previously studied by RAN3, as captured in section 4 of TR 37.817, is applicable for the study of AI/ML for NR air interface as well. Such framework should be considered as baseline for further discussion.

The following is the basic functional framework described in section 4 of TR 37.817.

Figure 4.2-1 (from TR 37.817): Functional Framework for RAN Intelligence

- Data Collection is a function that provides input data to Model training and Model inference functions. AI/ML algorithm specific data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, feedback from Actor, output from an AI/ML model.

  - Training Data: Data needed as input for the AI/ML Model Training function.

  - Inference Data: Data needed as input for the AI/ML Model Inference function.

- Model Training is a function that performs the ML model training, validation, and testing which may generate model performance metrics as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

  - Model Deployment/Update: Used to initially deploy a trained, validated, and tested AI/ML model to the Model Inference function or to deliver an updated model to the Model Inference function.

- Model Inference is a function that provides AI/ML model inference output (e.g., predictions or decisions) . Model Inference function may provide model performance feedback to Model Training function when applicable. The Model inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

  - Output: The inference output of the AI/ML model produced by a Model Inference function.

    - Note: Details of inference output are use case specific.

  - Model Performance Feedback:  It may be used for monitoring the performance of the AI/ML model.

    - Note: Details of the Model Performance Feedback process are out of RAN3 scope.

- Actor is a function that receives the output from the Model inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

  - Feedback: Information that may be needed to derive training or inference data or performance feedback.

Proposal 1: The basic functional framework defined in section 4 of TR 37.817 can be considered as baseline for study of AI/ML for NR air interface.

# Categorization of network-UE collaboration

Since the location of the AI/ML related functionality plays a crucial role of determining the collaboration between UE and network, we suggest to categorize the network-UE collaboration level based on the location of related functionalities which include Data Collection, Model Training, Model Inference, and Actor (see figure in section 2). Within the network, the exact location can be separated like gNB, OAM, and so on, but we think these network internal splits can be left to RAN3 and no need to spend the time in RAN1.

We have identified the following categorization:

- Cat-1: All AI/ML related functionalities are located at one side of network or UE. It can be further referred to as Cat-1-UE if all functionalities are located at UE, and otherwise Cat-1-Network.

  - Cat-1a: The other side is not aware of the usage of AI/ML (i.e., transparent to other side). No specification impact in this case.

  - Cat-1b: The other side might be aware of the usage of AI/ML, because additional signaling or procedure enhancements beyond the current specification is required between two side. Additional information is not directly related to training and inference, e.g., capability, new patterns, etc. Training or inference is done by implementation and not explicitly seen in the specification.

- Cat-2: Data Collection, Model Training and Model Inference are located at network, and UE acts as the Actor.

UE performs action corresponding to the network configuration or indication (i.e., output of the Model Inference) and provides feedback to network in terms of, e.g., performance feedback or measurement report.  This category may or may not have spec impact, depending on whether new signaling/procedure needs to be introduced between UE and network.

- Cat-3: Date Collection is located at network. The Model Training, Model Inference and Actor are located at UE.

UE can obtain data from the network for its own Model Training or Inference. Then UE decides its own action according to the output of UE’s Model Inference. Certain spec impact is envisioned, e.g., signaling and procedure for transmitting assistance information related to model training and inference.

- Cat-4: Date collection and Model training are at network, but Model Inference and Actor are located at UE.

In order to perform Inference at UE side, model can be updated according to the feedback provided from UE to the network and ML model (parameters, etc.) should be configured to the UE via network.

- Cat-5: Both network and UE have Model Training and Model Inference functionalities.

Network acts as centralized node for general model training, and training/inference models at UE is downloaded or updated from the network. UE can also provide model refinement according to its own environment and feedback to the network for general model updates. In this category, both UE and network can take actions based on its AI/ML module.

For Cat-5, the relation to SID text "Note 1: specific AI/ML models are not expected to be specified and are left to implementation" might need some discussion, e.g., how the training/inference models at UE is downloaded or updated from the network without specifying the AI/ML models.

In the above categorization, network-UE collaboration level generally is in an increasing order from the lower to higher categories, as well as the specification impact.

Generally speaking, there could be other possible location of AI/ML functionalities that does not fit into the above categorization. However, we think the above five categories can be representative in characterizing the levels of network-UE collaboration when studying the use cases such as beam management and CSI feedback enhancement. The use cases of beam management and CSI feedback are discussed in our accompany papers [2] and [3], respectively.

In summary, we propose the following:

Proposal 2: Network-UE collaboration can be categorized based on the location of AI/ML functionalities. The following categories can be considered:

- Cat-1: All AI/ML related functionalities are located at one side of network or UE.

- Cat-2: Data Collection, Model Training and Model Inference are located at network, and UE acts as the Actor.

- Cat-3: Date Collection is located at network. The Model Training, Model Inference and Actor are located at UE.

- Cat-4: Date collection and Model training are at network, but Model Inference and Actor are located at UE.

- Cat-5: Both network and UE have Model Training and Model Inference functionalities.

Proposal 3: AI/ML functionality mapping within the network (such as gNB or OAM) is up to RAN2/3 discussion. RAN1 discussion should focuses network-UE relation.

# Conclusion

In this paper, we discussed some general aspects of AI/ML. The proposals are as follows.

Proposal 1: The basic functional framework defined in section 4 of TR 37.817 can be considered as baseline for study of AI/ML for NR air interface.

Proposal 2: Network-UE collaboration can be categorized based on the location of AI/ML functionalities. The following categories can be considered:

- Cat-1: All AI/ML related functionalities are located at one side of network or UE.

- Cat-2: Data Collection, Model Training and Model Inference are located at network, and UE acts as the Actor.

- Cat-3: Date Collection is located at network. The Model Training, Model Inference and Actor are located at UE.

- Cat-4: Date collection and Model training are at network, but Model Inference and Actor are located at UE.

- Cat-5: Both network and UE have Model Training and Model Inference functionalities.

Proposal 3: AI/ML functionality mapping within the network (such as gNB or OAM) is up to RAN2/3 discussion. RAN1 discussion should focuses network-UE relation.

# Reference

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Moderator(Qualcomm)

[2] R1-2204078, Discussion on sub use cases of beam management, Panasonic

[3] R1-2204659, Discussion on AI/ML for CSI feedback enhancement, Panasonic