3GPP TSG-RAN WG1 #109-e 	R1-2204078

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.2

Source:	Panasonic

Title:	Discussion on sub use cases of beam management

Document for:	Discussion, Decision

# Introduction

A new Study Item “Study on Artificial Intelligence and Machine Learning for NR Air Interface” has recently approved for Rel-18. The following is the Objective from SID (see RP-213599).

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

As highlighted in the above objective, beam management has been identified as one of the use cases for study. This paper is targeted at further analyzing the sub use cases of beam management and various network-UE collaboration levels for the sub use cases.

# Discussion

## 2.1 Use case description

Generally speaking, beam management can be categorized into 3 main procedures: 1) initial beam establishment, 2) beam tracking and refining, and 3) beam failure recovery. The sub use cases can be discussed under these 3 categories below. The selection of the representative of sub use cases will be discussed in the next section.

1) Initial beam establishment

Initial beam establishment includes the procedures and functions by which a beam pair is initially established in the downlink and uplink transmission directions, for example, when a connection is established. In current NR, this is done via beam sweeping, where different SSBs that are associated with different Tx beams are transmitted in different OFDM symbols. The UE can then search for the strongest DL Tx beam (by adjusting the corresponding DL Rx beam at UE side as well) and report the selection via a corresponding RACH occasion and preamble to the gNB. In this way, the initial beam pair is established, which would remain valid after connection is set up until new beam indication is received by UE in CONNECTED mode.

One SSB burst containing all SSB beams is confined in time within 5 ms. It can be argued that even in the eMBB scenario, the initial beam establishment time overhead of 5 ms may be too long, and therefore AI/ML approaches can be used to speed up the initial beam selection. For example, the AI/ML algorithms can learn the relationship between the best SSB and a given location. With such knowledge, low latency beam searching or acquisition can be achieved.

When the initial beam establishment happen at the very first time to access the network after UE power on, AI/ML approach would basically be up to the UE implementation, because no information from the network can be leveraged before the initial connection is established. On the other hand, the initial beam establishment can also happen, e.g. after handover to another cell, for a newly added SCell, or when re-connecting to PCell after IDLE. In such cases, learning can be achieved by UE-gNB collaboration. Generally speaking, AI/ML enhanced initial beam establishment is a valid sub use case for consideration.

Observation 1: Initial beam establishment is one sub use case.

2) Beam tracking and refining

After the initial beam pair has been established, beam pair needs to be constantly adjusted due to UE movement or the change of radio environment. This can be referred to as beam tracking. In addition, beam management can also include beam refining, for example using a narrower CSI-RS beam compared to a relatively wider SSB beam used for initial beam establishment.

In current NR, for DL Tx beam adjustment, gNB can rely on the UE reporting on beam measurement. Different RS can be configured for beam measurement, such as CSI-RS or SSB. For the adjustment of UL Tx beam at UE side, SRS can be configured for UL beam management or sounding based management.

The above-mentioned beam adjustment procedures are associated with measurement and reporting overhead. On one hand, if the measurement/reporting interval is configured sufficiently short in order to cope with fast changing beam environment (e.g. fast UE movement), overhead and UE power consumption become large. On the other hand, if measurement/reporting interval is too long, the beam may become obsolete and beam failure could happen.

There are different ways to exploit AI/ML to tackle the above issue. One approach is to use AI/ML algorithms to adjust the time interval for measurement/reporting. This can be realized by either gNB or UE.

Another approach is the AI/ML based predictive beam switching. With help of AI/ML, beam can be proactively selected without waiting for the measurement/reporting. In such case, the measurement/reporting interval can be configured relatively long without causing beam failure, to reduce the overhead.

Further, instead of predicting the best beam for switching, AI/ML based beam prediction can also be used to narrow down the candidate beams for beam measurement/reporting, which can also contribute to the reduction of measurement overhead. More specifically, AI/ML is to determine a small set of beams to measure, and then estimate quality of unmeasured beams based on a small set of measured beams.

Observation 2: For beam tracking and refining, the following can be considered as sub use cases:

- Adjustment of measurement/reporting interval

- Predictive beam switching

- Partial beam set measurement

3) Beam failure recovery

Beam failure could happen if the beam adjustment cannot track the beam. This can be the case, for example, the current beam pair is suddenly blocked unexpectedly and the beam tracking function cannot react quick enough. Once beam failure happens, beam recovery typically requires the following steps:

- beam-failure detection,

- candidate-beam identification,

- recovery-request transmission, and

- network response to the beam-recovery request.

Among these steps, AI/ML can be useful at least for candidate-beam identification, e.g. based on past experience to learn the new set of candidate beams. Moreover, AI/ML can also help beam-failure detection/prevention. For example, if beam failure happens at some specific location or channel statistics with higher probability, UE or network can change the beam before the actual beam failure or trigger more measurement report. But from categorization point of view, it is more like beam tracking and refining as discussed above to “prevent” beam failure. Therefore, it can be argued that the AI/ML approaches would be similar to the beam tracking and refining use cases. There seems no need to have a separate sub use cases for beam failure recovery.

Observation 3: For beam failure recovery, the AI/ML approaches would be similar to beam tracking and refining sub use cases.

## 2.2 Network-UE collaboration for sub use cases

As highlighted in SID objective, the representative sub use cases need to be diverse enough to support various requirements on the network-UE collaboration levels. Within the network, the exact location can be separated like gNB, OAM, and so on but we think these network internal splits can be left to RAN3 and no need to spend the time in RAN1. As discussed in our accompany paper [1], the network-UE collaboration can be categorized by the location of the AI/ML related functionalities which include Data Collection, Model Training, Model Inference, and Actor. We identified the following categorization:

- Cat-1: All AI/ML related functionalities are located at one side of network or UE. All functionalities are located at UE is Cat-1-UE. All functionalities are located at network is Cat-1-Network.

- Cat-2: Data Collection, Model Training and Model Inference are located at network, and UE acts as the Actor.

UE performs action corresponding to the network configuration or indication (i.e. output of the Model Inference) and provides feedback to network in terms of, e.g. performance feedback or measurement report.  This category may or may not have spec impact, depending on whether new signaling/procedure needs to be introduced between UE and network.

- Cat-3: Date Collection is located at network. The Model Training, Model Inference and Actor are located at UE.

UE can obtain data from the network for its own Model Training or Inference. Then UE decides its own action according to the output of UE’s Model Inference. Certain spec impact is envisioned, e.g. signaling and procedure for transmitting assistance information related to model training and inference.

- Cat-4: Date collection and Model training are at network, but Model Inference and Actor are located at UE.

In order to perform Inference at UE side, model can be updated according to the feedback provided from UE to the network and ML model (parameters, etc.) should be configured to the UE via network.

- Cat-5: Both network and UE have Model Training and Model Inference functionalities.

Network acts as centralized node for general model training, and training/inference models at UE is downloaded or updated from the network. UE can also provide model refinement according to its own environment and feedback to the network for general model updates. For Cat-5, the relation to SID text " Note 1: specific AI/ML models are not expected to be specified and are left to implementation" might need some discussion, e.g. how training/inference models at UE is downloaded or updated from the network.

From the lower to higher categories, network-UE collaboration level generally is in an increasing order, as well as the specification impact. In the following, the sub use cases are studied with respect to these categories.

Sub use case 1: Initial beam establishment

For the initial beam establishment, the beam selection can be done by UE autonomously and then inform the decision to gNB via, e.g. the corresponding RACH resource. Therefore, Cat-1-UE seems to be a natural choice and considered as a baseline. On the other hand, in order to improve model training efficiency and accuracy, history data from other UEs can be utilized. In such case, Cat-3, Cat-4 or Cat-5 can be considered where data collection or both data collection and (general) model training is performed by network. From the specification impact point of view, further discussion is needed to standardize whether/how UE could obtain training data or inference model from network. Furthermore, the amount of signalling overhead for such case may also need to be carefully evaluated.

Observation 4: For sub use case of initial beam establishment, all AI/ML functionalities located at UE can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

Sub use case 2: Adjustment of measurement/reporting interval

For this sub use case, first of all, network based AI/ML such as Cat-1-Network or Cat-2 can be considered as baseline. In such case, network makes use of AI/ML to optimize the beam measurement interval and signalling the measurement configuration to the UE. Such configuration and indication can reuse the existing measurement configuration framework including periodic and aperiodic measurement. Depending on how to define the Actor, the same behaviour can be either modelled as Cat-1-network or Cat-2.

The adjustment of measurement/reporting interval can also be triggered by UE, if new event is defined. In such case, UE-based AI/ML such as Cat-1-UE and Cat-3 could be useful. As response to such event, network can configure new measurement/reporting interval for UE. Further, Cat-4 can be considered due to the similar reason. However, the additional advantage/complexity over the baseline should be further studied.

Another possibility could be Cat-5, where both network and UE can take actions based on AI/ML module. If the parameters of AI/ML module are synchronized between network and UE, there is no need for network to send DCI to trigger aperiodic CSI report. Measurement and reporting intervals are automatically adjusted thanks to the synchronized AI/ML from both sides. However, more significant spec impact is expected compared to the baseline.

Observation 5: For sub use case of adjustment of measurement/reporting interval, network based AI/ML can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

Sub use case 3: Predictive beam switching

Generally speaking, the AI/ML algorithms try to learn the relationship between the best serving beam of a UE and UE mobility information (such as location, speed and direction) in order to be able to predict the beam switching.

Similar to the sub use case of adjustment of measurement/reporting interval, network-based AI/ML (Cat-1-Network or Cat-2) can be considered as baseline, due to the fact that beam switching (for RRC_CONNECTED UE) should be under network’s control. For example, once the Model Inference at network predicts the best beam, network can switch UE serving beam by existing signaling framework such as MAC CE for PDCCH beam or scheduling DCI for PDSCH beam. Regarding the specification impact, it can be further discussed whether enhancement is needed. For example, similar to the discussion in Rel-17 NTN, network can configure beam switching pattern (i.e. a sequence of beams) with associated timing by RRC to save the MAC CE overhead. Another discussion point could be whether new signalling method is needed for UE to report mobility information related to location, speed and direction, beyond the existing method (e.g. CommonLocationInfo in the corresponding reportConfig).

On the other hand, the usage of UE-based AI/ML such Cat-1-UE and Cat-3 seemingly cannot provide obvious advantage over the baseline, considering the existing network-dominant beam switching framework for RRC_CONNECTED. The same reason is applied to Cat-4 and Cat-5 as well. Having said that, one potential use case could be that UE’s AI/ML module suggests the best beam and switching timing, and signal such suggestion to the network. But the final decision is still up to network. The advantage of such method and associated complexity including spec impact should be carefully investigated.

For the UE based mobility of RRC_IDLE and RRC_INACTIVE, similar discussion with Initial beam establishment would be applied.

Observation 6: For sub use case of predictive beam switching for RRC_CONNECTED, network based AI/ML can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

Sub use case 4: Partial beam set measurement

For this use case, AI/ML is to determine a small set of beams (called partial beam set) to measure for a given UE. Once the measurement results are obtained, AI/ML generates a new partial beam set for the given UE. Some algorithm may also need UE location information.

Since the selection of the partial beam set can either done by network or UE, both UE-based (Cat-1-UE, Cat-3) and network-based AI/ML (Cat-1-network, Cat-2) can be considered as baseline. It can be further study how Cat-4 and Cat-5 levels of network-UE collaboration can help.

Observation 7: For sub use case of partial beam measurement, both network based and UE based AI/ML can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

To summarize, we have the following proposal.

Proposal 1: AI/ML mapping within the network (such as gNB or OAM) is up to RAN2/3 discussion. RAN1 discussion should focuses network-UE relation.

Proposal 2: Consider the following mapping between sub use cases and network-UE collaboration levels for further study:

| Sub use cases | Cat-1-UE (all AI/ML functionalities at UE) | Cat-1-network (all AI/ML functionalities at network) | Cat-2 (Data Collection, Model Training and Model Inference at network; Actor at UE) | Cat-3 (Date Collection at network; Model Training, Model Inference and Actor at UE) | Cat-4 (Date collection and Model training at network; Model Inference and Actor at UE) | Cat-5 (Model Training and Model Inference at both network and UE ) |
| --- | --- | --- | --- | --- | --- | --- |
| Initial beam establishment | Baseline | Deprioritzed | Deprioritized | FFS | FFS | FFS |
| Adjustment of measurement/reporting interval | FFS | Baseline | Baseline | FFS | FFS | FFS |
| Predictive beam switching | FFS | Baseline | Baseline | FFS | FFS | FFS |
| Partial beam set measurement | Baseline | Baseline | Baseline | Baseline | FFS | FFS |

# Conclusion

In this paper, we have discussed the sub use cases of beam management and various network-UE collaboration levels for the sub use cases. We have the following observations:

Observation 1: Initial beam establishment is one sub use case.

Observation 2: For beam tracking and refining, the following can be considered as sub use cases:

- Adjustment of measurement/reporting interval

- Predictive beam switching

- Partial beam set measurement

Observation 3: For beam failure recovery, the AI/ML approaches would be similar to beam tracking and refining sub use cases.

Observation 4: For sub use case of initial beam establishment, all AI/ML functionalities located at UE can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

Observation 5: For sub use case of adjustment of measurement/reporting interval, network based AI/ML can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

Observation 6: For sub use case of predictive beam switching for RRC_CONNECTED, network based AI/ML can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

Observation 7: For sub use case of partial beam measurement, both network based and UE based AI/ML can be considered as baseline, and it can be FFS to spread AI/ML functionalities between UE and network.

The proposals are as follows:

Proposal 1: AI/ML mapping within the network (such as gNB or OAM) is up to RAN2/3 discussion. RAN1 discussion should focuses network-UE relation.

Proposal 2: Consider the following mapping between sub use cases and network-UE collaboration levels for further study:

| Sub use cases | Cat-1-UE (all AI/ML functionalities at UE) | Cat-1-network (all AI/ML functionalities at network) | Cat-2 (Data Collection, Model Training and Model Inference at network; Actor at UE) | Cat-3 (Date Collection at network; Model Training, Model Inference and Actor at UE) | Cat-4 (Date collection and Model training at network; Model Inference and Actor at UE) | Cat-5 (Model Training and Model Inference at both network and UE ) |
| --- | --- | --- | --- | --- | --- | --- |
| Initial beam establishment | Baseline | Deprioritzed | Deprioritized | FFS | FFS | FFS |
| Adjustment of measurement/reporting interval | FFS | Baseline | Baseline | FFS | FFS | FFS |
| Predictive beam switching | FFS | Baseline | Baseline | FFS | FFS | FFS |
| Partial beam set measurement | Baseline | Baseline | Baseline | Baseline | FFS | FFS |

# Reference

[1] R1- 2204077, General aspects of AI/ML framework, Panasonic