3GPP TSG-RAN WG1 Meeting #109-e	R1-2204082

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.9

Source:	Ericsson

Title:	Support of 16-QAM for unicast in UL and DL in NB-IoT

Document for:	Discussion and Decision

# 1	Introduction

The core-part of the Work Item (WI) on “Additional enhancements for NB-IoT and LTE-MTC” [1] was recently finalized including the specification of the following enhancement for NB-IoT:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

In this contribution we address capturing into the technical specification an agreement related to the support of 16-QAM for TDD NB-IoT.

# 2	Support of 16-QAM for unicast in DL

## 2.1	Support of 16-QAM for TDD NB-IoT

In RAN1# 108-e, there was a discussion on the support of 16-QAM for TDD NB-IoT which resulted in the following agreement:

Agreement - FDD/TDD differentiation is needed for FGs 1-1/1-2 - DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when 16QAM is configured.

Towards supporting 16-QAM for TDD NB-IoT, the part of the agreement stating “DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when 16QAM is configured” requires to be captured in TS 36.211 clause 10.0.1.2.

At RAN1# 108-e, special subframe configuration 9 was found not suitable to be used code-rate wise for 16-QAM in DL. Thus, for supporting 16-QAM for TDD NB-IoT, the following was agreed:

- “DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when 16QAM is configured”

To support 16-QAM for TDD NB-IoT, capture in TS 36.211 clause 10.0.1.2 through a TP that “DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM”:

------------------------------------------------------- Text Start ------------------------------------------------------------ #### 10.0.1.2 Frame structure type 2 Frame structure type 2 is applicable to TDD operation only. The following restrictions apply: - Uplink-downlink configuration 0 and 6 are not supported. - UpPTS is not used for NPUSCH or NPRACH. - DwPTS and UpPTS in special subframe configuration 10 is not used for transmissions. - On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission, and when npdsch-16QAM-Config-r17 is configured then DwPTS in special subframe configuration 9 for normal cyclic prefix is also not used for NPDSCH transmission. - Higher-layer parameter symbolBitmap does not apply to special subframes. ------------------------------------------------------- Text Ends ------------------------------------------------------------

# 5	Conclusion

In the previous sections we made the following observations for the support of 16-QAM for unicast in UL and DL for NB-IoT:

Observation 1	At RAN1# 108-e, special subframe configuration 9 was found not suitable to be used code-rate wise for 16-QAM in DL. Thus, for supporting 16-QAM for TDD NB-IoT, the following was agreed:

	“DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when 16QAM is configured”

Based on the discussion in the previous sections we propose the following:

Proposal 1	To support 16-QAM for TDD NB-IoT, capture in TS 36.211 clause 10.0.1.2 through a TP that “DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM”:

# 6	References

RP-201306, "WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

Session Notes for 8.16 (Rel-17 UE Features), Ad-Hoc Chair (NTT DOCOMO, INC.), 3GPP TSG RAN WG1 Meeting #107-e, e-Meeting, February 21st – March 3rd, 2022.