3GPP TSG RAN WG1 #109-e	R1-2204085

e-Meeting, May 9th – 20th, 2022

Source:	CableLabs

Title:	Discussion on resources on sidelink operation in unlicensed spectrum

Agenda Item:	9.4.1.2

Document for: 	Discussion and Decision

# Introduction

During RANP #95, a sidelink enhancement WI (RP-220300) was agreed. As part of the sidelink enhancement WI, an objective, to study and specify support sidelink operation in unlicensed spectrum, was included.

This contribution discusses a few issues on sidelink channel design on unlicensed spectrum. Firstly, the contribution discusses whether NR-U and SL-U coexist in a co-channel, and if so, how to determine sidelink resources in a time domain in consideration of Uu resources. Secondly, the contribution discusses whether and/or how to support interlaced sidelink transmission.

# SL-U Operation with co-channel NR-U

In a licensed spectrum, a sidelink transmission could occur in uplink resources in Uu. For example, in RAN1#99 and RAN1#100, the following agreements were made.

Agreements:

- NR supports SL transmissions at least in cell-specific UL resources in Uu.

Agreements:

For derivation of the set of slots to be included in the resource pool, the baseline is the derivation with bitmap and periodicity based on Subclause 14.1.5 of TS36.213 with the following modifications.

- FFS: Periodicity and L_bitmap value

- The slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if serving cell timing reference is in use, or DFN 0 otherwise

- The following procedure is used.

  - The set includes all the slots except the following slots:

    - Slots in which SLSS resource is configured,

    - (Working assumption) slots not having at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where

      - X is sl-LengthSymbols

      - Y is sl-StartSymbol

    - (Working assumption) reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213

A sidelink UE determines uplink resources based on TDD DL/UL configuration (e.g., tdd-UL-DL-ConfigurationCommon) or an indicated SL TDD configuration based on TDD DL/UL configuration (e.g., sl-TDD-Configuration). This would enable a network to control which resources would be used for sidelink and avoid sidelink transmission/reception overlapping with downlink reception.

In NR-U operation, however, cell-specific uplink resources may not be semi-statically signalled by the network, and resources may be used downlink or uplink transmissions (or reserved). So, if co-channel operation between NR-U and SL-U occurs, it should be discussed whether to limit a sidelink transmission in some Uu resources (e.g., uplink resources) or allow a sidelink transmission in any Uu resource.

Moreover, when a UE is configured with a NR-U serving cell and a SL-U in a co-channel, resource collisions between SL-U transmission/reception and NR-U transmission/reception would occur. For example, the UE is expected to monitor DCIs where monitoring occasions may overlap with SL-U resources. If the UE prioritizes SL operation (e.g., sensing, receiving or transmitting), it may impact on NR-U operation (e.g., missing group common or scheduling DCIs). If the UE prioritizes Uu operation, depending on periodicities of PDCCH monitoring occasions, SL-U operation may be frequently interrupted.

Given potential impacts on NR-U and/or SL-U operating in a co-channel, we propose to further discuss whether to support a co-channel operation between SL-U with NR-U, and if so, how to limit sidelink resources to certain Uu resources so that impacts on NR-U operation could be minimized.

Proposal 1: Discuss whether sidelink and Uu operation can occur simultaneously in the same carrier. If supported, potential enhancements to limit sidelink transmissions in some Uu resources (e.g., uplink resources) should be discussed.

To determine uplink resources (or resources allowed for a sidelink transmission) of the Uu operation in an unlicensed spectrum, a few options can be considered. For example, a semi-static TDD DL/UL configuration may be given by a base station and/or wireless device where sidelink operations are limited to semi-static UL resources based on the semi-static TDD DL/UL configuration. Such semi-static UL resources may not be used as downlink resources by the base station. This may limit flexibility of NR-U operation. Another example is that a base station (or pre-configuration) may indicate which resources could be used as sidelink resources (e.g., adding sidelink specific TDD DL/UL configuration or sidelink specific resource configuration in a sidelink BWP). This configuration can be used for a SL TDD configuration in PSBCH.

Alternatively, SL-U may be disabled when NR-U is enabled in a same carrier, at least in some conditions such as a stand-alone NR-U with mode 2 SL-U operation.

# Interlaced Sidelink Transmission

Resource allocation of a sidelink shared channel (PSSCH) is based on a subchannel consisting a number of consecutive RBs. A SL UE schedules one or more subchannels for a PSSCH. NR-U/LAA have adopted interlaced uplink transmissions such as PUSCH and PUCCH transmissions. It should be further discussed whether to introduce interlaced sidelink transmission as well.

Utilizing unlicensed spectrum such as 5 GHz would require satisfying regulatory requirements. For example, channel occupied bandwidth, according to ETSI EN 301 893 Section 4.2.2, should be greater than 80% of a nominal channel bandwidth. To support this and also potentially achieve performance benefits in some cases (e.g., in consideration of PSD limitation), NR-U has adopted interlaced resource blocks and allocation, where allocated resources are spread over an uplink BWP. When a sidelink operation occurs in such spectrum, the sidelink operation also needs to comply with the regulatory requirements.

In a sidelink configuration, a sidelink BWP may configure one or more sidelink resource pools, where a sidelink resource pool would be confined within the sidelink BWP. For frequency domain resources, a sidelink resource pool would consist of one or more subchannels where a subchannel comprises multiple consecutive RBs. The sidelink resource pool would include a list of sidelink slots for time domain resources. Figure 1 shows an example of a sidelink configuration.

Figure 1. Example of a sidelink configuration

For example, a minimal resource granularity for a sidelink data transmission is a subchannel in a sidelink slot. Resource sensing, selection and reservation may occur in one or more subchannels. Without interlaced transmission, each sidelink transmission may be required to allocate sufficiently large number of subchannels for example to satisfy regulatory requirements, which may be unnecessary or sub-optimal. This could lead resource congestion and increase failures in resource selection procedure. Thus, we propose to consider supporting interlaced transmissions for SL-U operation.

Proposal 2: Consider supporting interlaced sidelink transmissions.

In NR-U, a new resource allocation type (e.g., RA type 2) has been introduced to support interlaced PUSCH transmission. The RA type 2 assigns a first bitmap to indicate one or more interlaces (in a RB set) and a second bitmap to indicate one or more RB sets (e.g., one or more LBT bandwidths). A wireless device selects RBs indicated by both bitmaps. Similar resource allocation mechanism may be considered for a sidelink transmission for example extending RA type 2 for a sidelink transmission. If this applies, sensing and reservation may be done at interlace level instead of subchannel. For example, a subchannel in a sidelink operation may be replaced with an interlace.

This, however, may limit flexibility of a subchannel configuration of a sidelink resource pool (e.g., not able to configure separate subchannel size per each resource pool). For example, interlace resource blocks are common to all resource pools of a sidelink BWP and to one or more uplink BWPs of an uplink carrier sharing the same spectrum to the sidelink BWP (if supported).

In an example, with keeping subchannel configuration to allow resource allocation flexibility in each sidelink resource pool, a subchannel may be defined over a same interlace or adjacent interlaces, instead of consecutive PRBs. For example, a subchannel may correspond to one or more interlaces or a subchannel may comprise RBs belonging to a single interlace or one or more adjacent interlaces. This may enable to reuse subchannel based resource allocation/selection of sidelink operation while supporting interlaced transmission.

Proposal 3: If interlaced sidelink transmissions are supported, consider redefining a subchannel spanning one or adjacent interlaces. Interlaced resource blocks are common with uplink BWP(s) in the same carrier (if supported).

# Conclusion

This contribution proposes the followings:

Proposal 1: Discuss whether sidelink and Uu operation can occur simultaneously in the same carrier. If supported, potential enhancements to limit sidelink transmissions in some Uu resources (e.g., uplink resources) should be discussed.

Proposal 2: Consider supporting interlaced sidelink transmissions.

Proposal 3: If interlaced sidelink transmissions are supported, consider redefining a subchannel spanning one or adjacent interlaces. Interlaced resource blocks are common with uplink BWP(s) in the same carrier (if supported).

# References

[1] RP-220300, WID on NR sidelink evolution