3GPP TSG RAN WG1 #109-e		                          			   R1-2204087

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.10.1

Source:	InterDigital, Inc.

Title:	Multi-cell scheduling with a single DCI

Document for:	Discussion and Decision

# Introduction

RAN approved a new WI on multi-carrier enhancements [1]. One of the of the motivation to enhance the existing multi-carrier design is to support spectrum blocks that is more fragmented and scattered with narrow bandwidth. The current scheduling mechanism only allows scheduling of single cell PUSCH/PDSCH per a scheduling DCI. To reduce the overhead on control channels, the WID objective includes specifying solution to support single DCI scheduling multiple cells:

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In this contribution, we discuss bitfields of the single DCI for multiple cell scheduling, indication of the scheduled cells, two stage DCI support and the maximum number of simultaneously scheduled cells.

# Discussion

Single DCI scheduling multi-cell PDSCH/PUSCH was extensively studied in Rel-17 for the case of 2 scheduled cells. It was observed that single DCI scheduling multiple cells can provide CCE saving ratio about 10% to 40% depending on the DCI size [2].  Furthermore, it was shown that PDCCH blocking probability can be decreased and PDSCH throughput gain can be obtained in some cases. It is therefore recommended to select the DCI size carefully to benefit from the advantages of single DCI scheduling multiple cells.

DCI bitfield sharing among multiple cells:

Depending on the number of scheduled cells and the configured features of each cell, a large DCI overhead DCI may be required. To keep the DCI size low, a bitfield of the single DCI scheduling multiple cells can carry scheduling information of multiple cells scheduled cells. For example, single HARQ PID bitfield can be present in the DCI, and based on the indicated value the UE determines the HARQ PID for each scheduled cell. NR-U supports similar procedure for multi-TTI scheduling, where a single HARQ PID is indicated in the scheduling DCI.

For uplink and downlink scheduling, Table 1 and Table 2 provide our view on whether multiple bitfields are needed or not for different scheduled cells as well as sharing a bitfield among multiple cells to indicate scheduling information. For example, as shown in Table 1, we don’t think multiple bitfield for “identifier for DCI format” are needed and sharing this bitfield among multiple cells is not applicable as it does not carry scheduling information but rather helps the UE to identify the DCI format. For the “carrier indicator” bitfield, we think that only one bitfield is needed for the single DCI scheduling multiple cells and that bitfield can be used to indicate the scheduled cells (e.g., a code point can indicate a combination of cells that are being scheduled). For DFI flag and UL/SUL bitfields, a single bitfield is needed and sharing this information among multiple cells is not needed. For FDRA and TDRA bitfields, multiple bitfield can be present but also one bitfield can be shared among multiple scheduled cells. It is a tradeoff between scheduler flexibility and DCI overhead. For NDI bitfield, we think that multiple bitfield are needed and NDI cannot be shared among multiple scheduled cells. The complete list of bitfields for DCI format 0_1 and format 1_1 is listed Table 1 and Table 2 respectively.

Table 1 DCI format 0_1 bitfield sharing for multiple cells

| DCI format 0_1 bitfields | Bitfield size range | Multiple bitfields may be needed | Bitfield can be shared between multiple cells |
| --- | --- | --- | --- |
| Identifier for DCI format | 1 bit | No | Not applicable |
| Carrier indicator | 0 or 3 bits | No | Yes |
| DFI flag | 0 or 1 bit | No | No |
| UL/SUL | 0 or 1 bit | No | No |
| BWP | 0,1 or 2 bits | Yes | Yes |
| FDRA | Depends on the resource allocation type and the BWP size | Yes | Yes |
| TDRA | 0,1,2,3,4,5 or 6 bits | Yes | Yes |
| Freq hopping | 0 or 1 bit | Yes | Yes |
| MCS | 5 bits | Yes | No |
| NDI | 1,2,3,4,5,6,7 or 8 bits | Yes | No |
| RV | 2,3,4,5,6,7 or 8 bits | Yes | No |
| HARQ PID | 4 or 5 bits | Yes | Yes |
| 1st DAI | 1,2 or 4 bits | No | Yes |
| 2nd DAI | 0, 2 or 4 bits | No | Yes |
| TPC command | 2 bits | Yes | Yes |
| Second TPC | 0 or 2 bits | Yes | Yes |
| SRS resource set indicator | 0 or 2 bits | Yes | Yes |
| SRS resource indicator | or bits | Yes | Yes |
| Second SRS resource indicator | 0, log2(maxk∈{1,2,…,min⁡{Lmax,NSRS}}NSRSk)or log2(NSRS) bits | Yes | Yes |
| Precoding information | 0,1,2,3, 4, 5 or 6 bits | Yes | Yes |
| Second Precoding information | 0,1,2,3,4 or 5 bits | Yes | Yes |
| Antenna ports | 2,3,4 or 5 bits | Yes | Yes |
| SRS request | 2 or 3 bits | Yes | Yes |
| SRS offset indicator | 0,1 or 2 bits | Yes | Yes |
| CSI request | 0,1,2,3,4,5 or 6 bits | Yes | Yes |
| CBG transmission indication | 0,2,4,6 or 8 bits | Yes | No |
| PTRS-DMRS | 0 or 2 bits | Yes | Yes |
| Second PTRS-DMRS | 0 or 2 bits | Yes | Yes |
| Beta_offset indicator | 0 or 2 bits | Yes | Yes |
| DMRS sequence initialization | 0 or 1 bits | Yes | Yes |
| UL-SCH | 0 or 1 bit | Yes | Yes |
| CAPC | 0,1,2,3,4,5 or 6 bits | Yes | Yes |
| OLPC parameter set indication | 0,1 or 2 bits | Yes | Yes |
| Priority indicator | 0 or 1 bit | Yes | Yes |
| Invalid symbol pattern indicator | 0 or 1 bit | No | Yes |
| Minimum applicable scheduling offset indicator | 0 or 1 bit | No | Yes |
| SCell dormancy indication | 0,1,2,3,4 or 5 bits | No | Yes |
| Sidelink assignment index | 0,1 or 2 bits | No | Not applicable |
| PDCCH monitoring adaptation indication | 0,1 or 2 bits | No | Yes |

Table 2 DCI format 1_1 bitfield sharing for multiple cells

| DCI format 1_1 bitfields | Possible Bitfield sizes | Multiple bitfields may be needed | Bitfield can be shared between multiple cells |
| --- | --- | --- | --- |
| Identifier for DCI format | 1 bit | No | Yes |
| Carrier indicator | 0,3 bits | No | Yes |
| BWP | 0,2 bits | Yes | Yes |
| FDRA | Depends on the resource allocation type and the BWP size | Yes | Yes |
| TDRA | 0,1,2,3,4,5,6 bits | Yes | Yes |
| VRB-to-PRB mapping | 0,1 bit | Yes | Yes |
| PRB bundling size indicator | 0,1 bit | Yes | Yes |
| Rate matching indicator | 0,2 bits | Yes | Yes |
| ZP CSI-RS trigger | 0,2 bits | Yes | Yes |
| MCS for TB1 | 5 bits | Yes | No |
| NDI for TB1 | 1,2,3,4,5,6,7,8 bits | Yes | No |
| RV for TB1 | 2,3,4,5,6,7,8 bits | Yes | No |
| MCS for TB2 | 5 bits | Yes | No |
| NDI for TB2 | 1,2,3,4,5,6,7,8 bits | Yes | No |
| RV for TB2 | 2,3,4,5,6,7,8 bits | Yes | No |
| HARQ PID | 4,5 bits | Yes | Yes |
| DAI | 0,2,4,6 bits | No | Yes |
| TPC command for PUCCH | 2 bits | No | Yes |
| Second TPC for PUCCH | 0,2 bits | No | Yes |
| PUCCH resource indicator | 3 bits | No | Yes |
| PDSCH-to-HARQ_feedback timing indicator | 0,1,2,3 bits | No | Yes |
| One-shot HARQ-ACK request | 0,1 bit | No | Yes |
| Enhanced Type 3 codebook indicator | 0,1,2,3 bits | No | Yes |
| PDSCH group index | 0 or 1 bit | No | Yes |
| New feedback Indicator | 0,1 or 2 bits | No | Yes |
| Number of requested PDSCH groups | 0 or 1 bit | No | Yes |
| HARQ-ACK retransmission indicator | 0 or 1 bit | No | Yes |
| Antenna port(s) | 4,5 or 6 bits | Yes | Yes |
| Transmission configuration indication | 0,3 bits | Yes | Yes |
| SRS request | 2,3 bits | Yes | Yes |
| SRS offset indicator | 0,1,2 bits | Yes | Yes |
| CBGTI | 0,2,4,6,8 bits | Yes | No |
| CBGFI | 1 bit | Yes | No |
| DMRS sequence initialization | 1 bit | Yes | Yes |
| Priority indicator | 0 or 1 bit | Yes | Yes |
| CACP | 0,1,2,3 or 4 bits | Yes | Yes |
| Minimum applicable scheduling offset indicator | 0 or 1 bit | Yes | Yes |
| SCell dormancy indication | 0,1,2,3,4 or 5 bits | No | Yes |
| PDCCH monitoring adaptation indication | 0,1 or 2 bits | Yes | Yes |
| PUCCH cell indicator | 0,1 or 2 bits | No | Yes |

RAN1 should discuss the bitfield list and decide what information can be shared among multiple scheduled cells.

Proposal 1: Study indicating scheduling information for multiple cells using the same DCI bitfield.

DCI design for multi cell scheduling:

Another topic that needs to be discussed is how to indicate the scheduled cells using the single DCI. The number of scheduled cells can vary from one scheduling opportunity to another depending on the scheduler decision. Requiring the same number of scheduled cells when the single DCI scheduling multiple cells feature is enabled is not efficient. One approach is to design a DCI that is always capable of scheduling certain number of scheduled cells. This approach requires having multiple “single DCI” each one to schedule different number of cells.  The drawback of this approach is that it can increase the number of DCI formats and DCI sizes to be monitored by the UE. Another approach could be to have one “single DCI” that will be scheduling different number of scheduled cells. A bitfield in the DCI can indicate the group of scheduled cells to the UE.

Proposal 2: A bitfield in the DCI can indicate the scheduled cells.

During the study of 2 cells scheduled by single DCI in Rel-17 [2], it was shown that large DCI payload size can cause high PDCCH blocking probability. This goes against the motivation of supporting single DCI scheduling multiple cells that is reducing the load on the control channels. At the same time, reducing the DCI can severely impact the scheduling flexibility. One solution could be to have two stage DCI to carry scheduling information in two stages. For example, a first DCI can indicate the scheduled cells and part of the scheduling information for the scheduled cells. A second DCI can indicate the remaining scheduling information. Using two DCIs can reduce the PDCCH blocking in some scenarios, and it can be up to the scheduler whether to enable such feature or not. RRC can enable or disable two-stage DCI for multi-cell scheduling.

Proposal 3: Support two-stage DCI for multi-cell scheduling where the scheduling information are carried using two DCIs.

Maximum number of cells that can be scheduled simultaneously

Scheduling multiple cells from a single DCI is expected to bring benefits in terms of overhead of downlink control signaling when the data is scheduled in multiple carriers that may have limited bandwidth. However, supporting too large number of simultaneously scheduled cells without impacting the scheduling flexibility or PDCCH blocking probability may be challenging. It is expected that the number of cells over which single DCI is no longer beneficial depends on the scenario in terms of bandwidths of individual carriers. RAN1 should therefore agree on at least one baseline set of assumptions for this aspect to determine the right value for this aspect of the design. Considering that the objective of the WID is justified by deployment with scattered spectrum bands, a reasonable starting point for the maximum number of cells is that it should be more than 3 carriers.

Proposal 4: RAN1 to agree on at least one baseline scenario for the set of schedulable carriers and their bandwidths.

Proposal 5: The maximum number of simultaneously scheduled cells is not lower than 4.

# Conclusion

In this contribution, the following proposals are made:

Proposal 1: Study indicating multiple scheduling information for multiple cells using the same DCI bitfield.

Proposal 2: A bitfield in the DCI can indicate the scheduled cells.

Proposal 3: Support two-stage DCI for multi-cell scheduling where the scheduling information are carried using two DCIs.

Proposal 4: RAN1 to agree on at least one baseline scenario for the set of schedulable carriers and their bandwidths.

Proposal 5: The maximum number of simultaneously scheduled cells is not lower than 4.

# References

RP-220834, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO.

R1-2102138, “Feature lead summary #4 on multi-cell scheduling via a single DCI”, 3GPP TSG RAN WG1 Meeting #104-e, January 25th – February 5th, 2020.