3GPP TSG RAN WG1 #109-e		                          			   R1-2204089

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.8.1

Source:	InterDigital, Inc.

Title:	Remaining issues on PUSCH enhancements

Document for:	Discussion and Decision

# Introduction

This contribution discusses the following issues related to PUSCH enhancements:

- Available slot counting for inter-cell mTRP

- Support for CFRA PUSCH repetition

# Available slot counting for inter-cell mTRP

R17 supports multi-TRP inter-cell operation which requires the UE to monitor SSB associated with PCI other than the PCI of the serving cell. RAN1 discussed the potential impact of this feature on available slot counting in RAN1#107-e and RAN1#107b-e and decided to defer agreement until further progress on collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO AI.

In RAN1#108-e, RAN1 made the following agreement (in feMIMO AI):

Agreement For inter-cell mTRP, UE does not transmit PUCCH/PUSCH/PRACH in a slot or SRS in the symbols if in time domain the PUCCH/PUSCH/PRACH/SRS overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI.

In the above, the “active additional PCI” refers to the additional PCI associated with the active TCI states. At most 1 additional PCI can be associated with the active TCI states. The agreement means that the UE prioritizes reception of SSB associated to an active TCI state. This ensures robustness of PDCCH reception that may use such active TCI state.

In view of the above agreement, the most natural handling for available slot determination seems to consider the SSB associated with the active additional PCI in addition to the SSB associated with the serving cell PCI. This leads to the following proposal:

Proposal 1: Available slot determination considers both SSB of a serving cell PCI and SSB associated with an active TCI state.

If the above Proposal is agreed, the following text proposal to TS38.214 is suggested:

#### 6.1.2.1 Resource allocation in time domain <omitted text> For unpaired spectrum: - When AvailableSlotCounting is enabled, and in case K>1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or associated with an active TCI state. - Otherwise, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or associated with an active TCI state. - The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, and the TDRA information field value in the RAR UL grant. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or associated with an active TCI state. - The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst and the TDRA information field value in the DCI scheduling the PUSCH. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 scrambled by TC-RNTI, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or associated with an active TCI state. For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or associated with an active TCI state. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. <omitted text>

# Support for CFRA PUSCH repetition

In RAN1#108-e, RAN1 discussed whether to confirm the WA on CFRA PUSCH repetition and what information to send to RAN2 regarding this issue. RAN1 could not reach consensus on whether to confirm the WA as some companies believe this would have impact on RAN1 specification. RAN1 also did not reach consensus on an LS to RAN2.

In our understanding, the specification impact of supporting CFRA PUSCH repetition is restricted to RAN2 specifications. MAC specifies if MCS of the RAR is interpreted as legacy or repetition and provides the information to the physical layer.

Even if RAN1 cannot confirm the working assumption, RAN1 should provide the status to RAN2 to provide the possibility to RAN2 to decide whether the functionality is supported (without RAN1 impact) or not.

Proposal 2: Confirm working assumption to support repetition for PUSCH scheduled by RAR in CFRA with no additional RAN1 impact and inform RAN2.

Proposal 3: If Proposal 2 is not agreeable, inform RAN2 that RAN1 cannot reach consensus on whether to confirm working assumption and that RAN2 can decide if the functionality can be supported without RAN1 specification impact.

# Conclusion.

In this contribution, the following proposals are made:

Proposal 1: Available slot determination considers both SSB of a serving cell PCI and SSB associated with an active TCI state.

Proposal 2: Confirm working assumption to support repetition for PUSCH scheduled by RAR in CFRA with no additional RAN1 impact and inform RAN2.

Proposal 3: If Proposal 2 is not agreeable, inform RAN2 that RAN1 cannot reach consensus on whether to confirm working assumption and that RAN2 can decide if the functionality can be supported without RAN1 specification impact.

# Reference

[1] R1-2202569, “FL Summary #4 on Enhancements on PUSCH repetition type A”, Moderator (Sharp).

[2] R1-2202593, “Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3”, Moderator (ZTE Corporation).