3GPP TSG RAN WG1 #109-e		                          			   R1-2204090

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.8.2

Source:	InterDigital, Inc.

Title:	Joint channel estimation for PUSCH and PUCCH

Document for:	Discussion and Decision

# Introduction

This contribution discusses the following remaining issues for joint channel estimation:

- Treatment of group common TPC commands

- DM-RS bundling restart behaviour

# Remaining details for joint channel estimation

## Treatment of group common TPC commands

In RAN1#107e, RAN1 made a working assumption [2] that reception of a group common TPC command with format 2_2 does not constitute an event that violates power consistency and phase continuity. The working assumption further states that if the UE is configured to accumulate TPC commands and DMRS bundling is enabled, the UE is expected to accumulate TPC commands during a configured TDW (Time Domain Window) and apply the accumulated TDW at the next configured TDW such that power consistency can be preserved during a configured TDW.

Although the intention of the working assumption seems straightforward, different options are possible for its capture in the specification. RAN1 discussed these options extensively during RAN1#108-e and could not converge as issues would be identified in scenarios such as CG-DG interlacing.

Considering this situation and the fact that R17 is now in maintenance phase, it may be more reasonable to not confirm the working assumption such that reception of TPC command with format 2_2 is treated as an event that violates power consistency and phase continuity when the command indicates a change of power.

Proposal 1: Do not confirm following working assumption: “The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity”.

Proposal 2: Reception of group common TPC command with format 2_2 indicating non-zero power change is treated as an event that violates power consistency and phase continuity.

## DM-RS bundling restart behaviour

In RAN1#108-e, RAN1 discussed whether a UE that is not capable of restarting bundling after a dynamic event could restart bundling in a subsequent (Case 1) or overlapping (Case 2) semi-static event. RAN1 also discussed which behaviour is stipulated by existing specification and if there is need for clarification. No conclusion was reached.

On the issue of what existing specification is stipulating, careful reading leads to the conclusion that a non-capable UE never restarts bundling once power consistency and phase continuity is not maintained. The specification [38.214 v17.1.0] states that the end of an actual TDW is “The last symbol of the [PUSCH or PUCCH] transmission before the event, if an event occurs which causes power consistency and phase continuity not to be maintained […]”. In Case 1, a new actual TDW is started after the dynamic event, but the non-capable UE does not maintain power consistency and phase continuity within that actual TDW in the first place. Therefore, any subsequent dynamic or semi-static event does not cause power consistency and phase continuity not to be maintained, since such power consistency and phase continuity did not exist in the first place. This implies that the end of this actual TDW is the end of the nominal TDW. In Case 2, the situation is the same regardless of the order in which the overlapping events are processed, i.e. dynamic event first and semi-static event second (same as Case 1), or semi-static event first and dynamic event second (UE maintains bundling for an actual TDW of 0).

Observation 1: According to TS38.214 v17.1.0, a UE not capable of restarting bundling after a dynamic event does not restart bundling before the end of the nominal TDW.

The next question is whether it would be desirable to modify the specification such that a non-capable UE does restart bundling after a semi-static event. However, the relevance of defining such type of UE seems very questionable. The typical behaviour expected from a non-capable UE is that it could schedule in advance changes in powers and phases over the nominal TDW for bundling operation such that no recalculation is necessary later on. When a dynamic event occurs, previous calculations made for the subsequent semi-static event are no longer valid and the UE therefore would need to restart the calculations. Since the UE would need to be prepared to support this recalculation during nominal TDW anyway, in practice there would be little simplification from implementation perspective.

Observation 2: Relaxing the requirement to restart bundling after a dynamic event does not allow significant complexity reduction if the UE is anyway required to restart bundling after a subsequent semi-static event.

According to Observation 1, the desired behaviour is already captured in TS38.214 and no clarification seems necessary in the specification itself. However, it would be desirable that RAN1 confirms this behaviour in meeting minutes for reference purpose.

Proposal 3: RAN1 confirms that a UE not capable of restarting bundling after a dynamic event does not restart bundling before the end of the nominal TDW. No clarification to TS38.214 is required.

# Conclusion.

In this contribution, the following observations and proposals are made:

Proposal 1: Do not confirm following working assumption: “The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity”.

Proposal 2: Reception of group common TPC command with format 2_2 indicating non-zero power change is treated as an event that violates power consistency and phase continuity.

Observation 1: According to TS38.214 v17.1.0, a UE not capable of restarting bundling after a dynamic event does not restart bundling before the end of the nominal TDW.

Observation 2: Relaxing the requirement to restart bundling after a dynamic event does not allow significant complexity reduction if the UE is anyway required to restart bundling after a subsequent semi-static event.

Proposal 3: RAN1 confirms that a UE not capable of restarting bundling after a dynamic event does not restart bundling before the end of the nominal TDW. No clarification to TS38.214 is required.

# Reference

[1] 3GPP TS38.214, “Physical layer procedures for data”, v17.1.0.