3GPP TSG RAN WG1 Meeting #109-e	R1-2204092

e-Meeting, May 9th – 20th , 2022

Agenda Item:	9.5.1.3  Potential solutions for sidelink positioning

Source:	Locaila

Title:	Carrier phase measurement method for sidelink positioning

Document for:	Discussion & Decision

Introduction

Carrier phase positioning is one of the promising candidates to consider for sidelink positioning [1] because it provides relatively high accuracy with consuming fewer resources. According to our initial field experiment result[3], the subcarrier or carrier phase based measurement method is able to provide less than a meter accuracy within a 1Km area. In the experiment, we achieved the accuracy using only 10Mbps ~ 30Mbps bandwidth, which is the current allocated V2X spectrum in many countries.

In this contribution, we present a potential solution for sidelink positioning using the phase-based AoA measurement method. In conclusion, we propose it is necessary to design a new positioning symbol appropriate for sidelink environment.

AoA based Sidelink Positioning

Figure 1 below describes the concept of sidelink positioning using 2 uniform array antennas and UL-AoA measurement method.

Figure 1 Proposed sidelink positioning using phase-based AoA measurement

In figure 1 above, it is assumed all V2X vehicles periodically generate a positioning reference signal indicating their identity. The signal may either be UL SRS or other signals similar to PRS, however in this contribution, we assume a new type of continuous positioning reference signal (C-PRS) as presented in R1-2203635 [2]. The C-PRS signal is a continuous sinusoidal waveform in constant frequency on, a contiguous time axis, as shown in figure 2 (b). This type of reference signal arrangement is efficient for phase tracking and Doppler shift compensation in moving vehicles.

Figure 2 (a)  Rel-17 staggered PRS pattern               (b) Contiguous & continuous PRS

In order to avoid collisions with other vehicle signals, it is assumed that each vehicle is assigned a unique subcarrier set and a transmission time interval. The vehicles allocated in the same time slot are distinguished using different subcarrier numbers. The specific rules of subcarrier arrangement for vehicles are for further discussion.

Two or more phased array antennas installed on the roof of the vehicle may detect the direction and distance, velocity, etc. of incoming C-PRS signals. The signals collected through the phased array antennas can be separated into a set of subcarriers for each vehicle, for example by the FFT process. By calculating the covariance matrix of these separated subcarrier sets and applying well-known direction-finding algorithms such as MUSIC or ESPRIT, the relative directional angle information of other vehicles can be obtained.  In addition, it is possible to estimate the distance information of the signal source if the vehicles are synchronized using a common gNB signal (e.g. RSUs) or using satellite GPSDO. In this way, the location between vehicles can be determined even in the out-of-service area.

The sidelink positioning method proposed in this contribution is useful for locating other vehicles in the 1m~ 1Km range where radar or lidar does not affect. For example, when a large bus or truck is blocking the front of autonomous vehicles, overtaking can be dangerous because radar or lidar cannot detect other vehicles behind a truck. However, the proposed sidelink positioning method can provide information about other vehicles and improve the safety of autonomous driving.

- Block Type Continuous PRS arrangement

The proposed sidelink positioning method using the phased array antenna shown above can be implemented to some extent using the SRS or PRS variant defined in Rel-17. However, in situations where the vehicle is moving at high speed, a block type symbol arrangement of several symbol duration, as shown in figure 2 (b), is more efficient rather than SRS and PRS.  Figure 2 above compares the Rel-17 PRS pattern with the continuous PRS of the block type arrangement proposed in this article.

The above block symbol maintains the subcarrier frequency for a relatively long period of time, enabling high-speed vehicles to track and synchronize signals from other vehicles for precise distance measurements. In addition, since the sinusoidal waveform lasts for several symbol times, multiple snapshot vector collections and special and temporal smoothing of signal processing are possible during this interval, which allows for more precise multipath signal separation. For more information on other C-PRS symbol structures, refer to R1-2203635[2].

Conclusion

In conclusion, we propose following new aspects must be considered in priority during the ongoing study phase of 5G NR sidelink positioning.

Proposal 1: Study new sidelink positioning reference signal efficient for supporting phase-based measurement method

Proposal 2: Study use case, solution and impact to sidelink positioning when applying the phase-based AoA measurement method

Proposal 3: Consider using the carrier phase method for V2V synchronization and the necessary impact on gNodeB structure.

References

| [1] | RP-213588, Study on expanded and improved NR positioning, Intel, December 6-17, 2021 |
| --- | --- |
| [2] | 3GPP R1-2203635, "Continuous PRS for improved carrier phase measurement" , Dankook University , RAN WG1 #109 e-Meeting, May 9th – 20th, 2022 |
| [3] | 3GPP R1-2103634, "Use cases and applications on carrier phase based positioning for NR" , Locaila , RAN WG1 #109 e-Meeting, May 9th – 20th, 2022 |