3GPP TSG RAN WG1 #109-e	R1- 2204094

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.1
Source:	Toyota InfoTechnology Center
Title:	Discussion on V2X use cases, scenarios, and requirements for sidelink positioning

Document for:	Discussion and Decision

# Introduction

In RAN #94-e meeting [1], the SID for Rel-18 study on expanded and improved NR positioning was approved [2]. This includes the study of sidelink positioning as shown below. One of the objectives is to identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums. In this contribution, we provide potential V2X use cases, scenarios, and requirements for sidelink positioning.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

# V2X use cases, scenarios, and requirements for sidelink positioning

## V2X scenarios for sidelink positioning

In this clause, we consider potential V2X scenarios for sidelink positioning. For most of V2X services, precise position information is one of the essential data elements that need to be exchanged over V2X communications (e.g., for Basic Safety Message (BSM), Collective Perception Message (CPM), Maneuver Coordination Message (MCM), Personal Safety Message (PSM), etc.). To obtain the position information, it was assumed that UEs use conventional positioning technologies (e.g., GNSS, inertial measurement unit (IMU)/dead reckoning, Uu-based positioning). However, precise positioning is challenging in some cases/scenarios [3] such as:

- Regarding GNSS, GNSS is not available or accurate due to NLOS and multipath in urban canyons, tunnels, parking structures / underground parking (as shown in Figure 1);

- Regarding IMU/dead reckoning, the positioning error increase over distance;

- Regarding Uu-based positioning, it is not available in out-of-coverage scenarios.

- Scenarios where GNSS is not available or accurate.

For those scenarios, sidelink positioning would be beneficial. Therefore, those V2X scenarios should be considered and supported in sidelink positioning.

- The following V2X scenarios are considered and supported for sidelink positioning:

- Scenarios where GNSS is not available or accurate (e.g., urban canyons, tunnels, parking structures / underground parking)

- All network coverage scenarios (i.e., in-coverage, partial coverage, out-of-coverage scenarios)

## V2X use cases and UE types for sidelink positioning

In this clause, we consider potential V2X use cases for sidelink positioning. 3GPP TR 38.845 [4] summarizes V2X use cases for sidelink positioning from multiple sources. 5GAA [5] groups V2X use cases for sidelink positioning into three groups based on positioning requirements:

- Group 1 (Lax positioning requirement, tens of meters with 1 sigma) for information provisioning (e.g., road warning, route information)

- Group 2 (Lane level positioning requirement, 1.5 m with 3 sigma) for Day 1 safety use cases (e.g., left turn assist, intersection movement assist, emergency brake warning)

- Group 3 (Below meter positioning requirement, e.g. 0.1 m with 3 sigma) for teleoperated driving, vulnerable road user (VRU) protection, etc.

These use cases include V2V, V2I, and V2VRU safety use cases. Therefore, the following UE types need to be supported for sidelink positioning:

- Vehicle-type UEs

- RSU-type UEs

- VRU-type UEs (e.g., pedestrians, cyclists, motorcyclists)

Further, various combinations of UE types are possible as shown in Figure 2 and need to be supported, in which UEs exchange Sidelink Positioning Reference Signals (SL-PRS) for sidelink positioning.

- Combinations of different UE types for V2X sidelink positioning.

- The following UE types are supported for sidelink positioning:

- Vehicle-type UEs

- RSU-type UEs

- VRU-type UEs (e.g., pedestrians, cyclists, motorcyclists)

- The following combinations of UE types are supported for exchange of SL-PRS in sidelink positioning:

- Vehicle to Vehicle

- Vehicle to Infrastructure

- Vehicle to VRU

- VRU to Infrastructure

## V2X requirements for sidelink positioning

In this clause, we consider potential V2X requirements for sidelink positioning. 3GPP TR 38.845 [4] summarizes V2X positioning requirements (e.g., absolute/relative positioning accuracy, confidence level, positioning service latency, etc.) from multiple sources as follows:

It is observed that the positioning requirements in V2X depend on the service the UE operates. Also, the requirements are applicable to relative and absolute positioning depending on the use case or the positioning service level. In terms of the horizontal or lateral/longitudinal accuracy, the requirements for the absolute position or relative position can be categorised into three sets as follows by incorporating the requirements from the sources mentioned above: - Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 in [5] and Service level 1 in [3]. - Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 in [5], Service level 2, 3, 4 in [3]. - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 in [5], Service level 5, 6, 7 in [3], the requirements in [4]. It is noted that all the three sets are applicable for absolute positioning and relative positioning. Requirements for other performance metrics are also defined in a range depending on the positioning service level in TS 22.261 [3]; 2 – 3 m (absolute) or 0.2 m (relative) vertical accuracy, 95 – 99.9% positioning service availability, 10 ms – 1 s positioning service latency. Positioning service should be provided in indoor, outdoor, tunnel areas. The UE velocity up to 250 km/h needs to be supported for outdoor and tunnel areas. As long as the UE operates a V2X use case having the corresponding positioning requirements, the requirements should be fulfilled when the UE is inside the network coverage as well as when it is outside the network coverage. The requirements should be also fulfilled when the GNSS-based positioning is not available or not accurate enough.

In addition to these requirements, the following requirements/features need to be supported for sidelink positioning in V2X scenarios from RAN1 perspective:

- Enough range (e.g., > 300 m) for transmission and reception of SL-PRS:

  - V2X safety use cases require enough communication range (e.g., > 300 m) [6] so that vehicles and VRUs can have enough time to avoid traffic accidents. If sidelink-based relative positioning is used for V2X safety services, UEs (e.g., vehicles, VRUs) in proximity need to perform sidelink-based relative positioning at a sufficient distance so that they can have enough time to avoid traffic accidents. Therefore, we need to ensure enough range for transmission and reception of SL-PRS.

- Efficient use of radio resources while meeting positioning requirements:

  - Precise sidelink positioning requires a large amount of bandwidth, while the radio resources are limited, particularly in congested scenarios. Therefore, we need techniques to efficiently use radio resources, while ensuring required positioning accuracy, range, latency, reliability, etc.

- Supporting different UE types (vehicles, RSUs, VRUs, etc.):

  - Different UE types (vehicles, RSUs, VRUs, etc.) have different mobility characteristics, use cases, and positioning requirements. Therefore, we need to support different UE types and corresponding positioning requirements.

- Power saving for portable devices for V2P/VRU use cases:

  - V2P/VRU protection use cases require precise positioning, but VRU-type UEs are typically portable devices with a limited battery. Therefore, power saving features are needed for transmission and reception of SL-PRS for V2P/VRU use cases.

- In addition to the V2X positioning requirements in TR 38.845, the following requirements and features are supported for sidelink positioning in V2X scenarios from RAN1 perspective:

- Enough range (e.g., > 300 m) for transmission and reception of SL-PRS

- Efficient use of radio resources

- Supporting different UE types (vehicles, RSUs, VRUs, etc.)

- Power saving for portable devices in V2P/VRU scenarios

# Conclusion

In this contribution we provided potential V2X scenarios, use cases, and requirements for sidelink positioning. Our proposals are summarized as follows:

- The following V2X scenarios are considered and supported for sidelink positioning:

- Scenarios where GNSS is not available or accurate (e.g., urban canyons, tunnels, parking structures / underground parking)

- All network coverage scenarios (i.e., in-coverage, partial coverage, out-of-coverage scenarios)

- The following UE types are supported for sidelink positioning:

- Vehicle-type UEs

- RSU-type UEs

- VRU-type UEs (e.g., pedestrians, cyclists, motorcyclists)

- The following combinations of UE types are supported for exchange of SL-PRS in sidelink positioning:

- Vehicle to Vehicle

- Vehicle to Infrastructure

- Vehicle to VRU

- VRU to Infrastructure

- In addition to the V2X positioning requirements in TR 38.845, the following requirements and features are supported for V2X sidelink positioning from RAN1 perspective:

- Enough range (e.g., > 300 m) for transmission and reception of SL-PRS

- Efficient use of radio resources while meeting positioning requirements

- Supporting different UE types (vehicles, RSUs, VRUs, etc.)

- Power saving for portable devices in V2P/VRU scenarios

# Reference

- RP-220842, “Meeting Report for TSG RAN meeting #94e”, RAN #94e, Dec., 2021.

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN #94e, Dec., 2021.

- RP-210036, “Reply LS to 3GPP TSG RAN on requirements of in-coverage, partial coverage and out-of-coverage positioning use cases,” SAE, Mar., 2021.

- 3GPP TR 38.845 V17.0.0, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases,” Sept., 2021.

- RP-210040, “Reply LS to RP-201390 on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases,” 5GAA, Mar., 2021.

- SAE J2945/1, “On-Board System Requirements for V2V Safety Communications”, Apr. 2020.