3GPP TSG RAN WG1 Meeting #109-e	R1-2204097

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.4.1.1

Source:	FUTUREWEI

Title:	Discussion of channel access mechanism for sidelink transmission in unlicensed spectrum

Document for:	Discussion and decision

# Introduction

At RAN#94-e, a WI on sidelink evolution was agreed for Rel-18 (RP-213678) [1]. In this WI, an objective on sidelink in unlicensed spectrum is included as:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we first review the channel access mechanism in NR unlicensed spectrum and then present of our views on the channel access mechanisms for sidelink transmission in unlicensed spectrum.

# Discussions

## Channel Access Mechanism in NR Unlicensed (NR-U) Spectrum

NR channel access procedures in shared (unlicensed) spectrum are specified in TS 37.213, which is aligned with the requirements specified for compliance in European harmonized standard for WAS/RLAN in 5 GHz [2]. The basic mechanism is Listen Before Talk (LBT), i.e., before transmitting anything, a UE performs an energy detection (ED) based clear channel assessment (CCA)  sensing to determine whether the channel is idle or occupied. If LBT is successful, i.e., the channel is idle, the UE (or gNB) can transmit.

### Summary of LBT Types

Several types of LBT are supported in NR-U for both DL and UL channel accesses.

- Type 1 (CAT4 LBT): 25μs sensing interval which starts and ends by a 9 μs sensing slot

- Type 2A (CAT2 LBT with 25μs gap) 25μs sensing interval which starts and ends by a 9 μs sensing slot

- Type 2B (CAT2 LBT with 16μs gap): a sensing slot occurs within the last 9μs. The channel is idle if it is sensed and determined to be idle for a total of at least 5μs with at least 4μs of sensing occurring in the last sensing slot.

- Type 2C (CAT1 or No LBT): no LBT is required for a gap of up to 16 μs but the transmission duration is limited to 584μs.

In the sequel, we elaborate more on the channel access procedures based on the above four different LBT types. These channel access procedures are applied to both DL and UL. For UL transmissions, the type of channel access procedure is signaled or configured by the network such as channel access type, channel access priority class (CAPC), and cyclic prefix extension (CPE). CPE is used for adjusting the gap between two consecutive transmissions.

### Type 1 channel access procedure

Type 1 channel access procedure is applicable to any DL transmissions by gNB and UL PUCCH/PUSCH/SRS transmissions, and transmissions related to random access procedure in 5G NR.

As described in TS 37.213, a gNB or a UE may transmit the transmission using Type 1 channel access procedure according to following steps after first sensing the channel to be idle during the slot durations of a defer duration Td, and after the counter N is zero in step 4.

1) set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4; 2) if N>0 and the UE chooses to decrement the counter, set N=N-1; 3) sense the channel for an additional slot duration, and if the additional slot duration is idle, go to step 4; else, go to step 5; 4) if N=0, stop; else, go to step 2. 5) sense the channel until either a busy slot is detected within an additional defer duration Td or all the slots of the additional defer duration Td are detected to be idle; 6) if the channel is sensed to be idle during all the slot durations of the additional defer duration Td, go to step 4; else, go to step 5;

The defer duration Td consists of duration Tf=16us  immediately followed by mp consecutive slot durations where each slot duration is Tsl=9us, and Tf includes an idle slot duration Tsl at start of Tf. CWmin,p≤CWp≤CWmax,p is the contention window, where CWmin,p and CWmax,p are chosen before step 1. The parameters mp, CWmin,p, and CWmax,p are based on the CAPC p as shown in Table 4.1.1-1 and Table 4.2.1-1 (as copied below) from TS37.213 for DL and UL transmissions, respectively.

Table 4.2.1-1 (TS37.213): Channel Access Priority Class (CAPC) for UL

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tulmcot,p | allowed CWp sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For p=3,4, Tulmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided , otherwise, Tulmcot,p=6ms. NOTE 2: When Tulmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100us. The maximum duration before including any such gap shall be 6ms. |  |  |  |  |  |

### Type 2 channel access procedures

#### Type 2A channel access procedure

A gNB or a UE may transmit immediately after sensing the channel to be idle for at least a sensing interval Tshort=25us. Tshort consists of a duration Tf=16us  immediately followed by one sensing slot and Tf includes a sensing slot at start of Tf. The channel is considered to be idle for Tshort if both sensing slots of Tshort are sensed to be idle. A UE is indicated to use Type 2A UL channel access procedures for a UL transmission.

#### Type 2B channel access procedure

A gNB or a UE may transmit immediately after sensing the channel to be idle within a duration of Tf=16us. Tf includes a sensing slot that occurs within the last 9us of Tf. The channel is considered to be idle within the duration Tf if the channel is sensed to be idle for total of at least 5us with at least 4us of sensing occurring in the sensing slot.

#### Type 2C channel access procedure

With Type 2C channel access, the gNB or the UE (if indicated to perform Type 2C UL channel access procedures) does not sense the channel before the transmission. The duration of the corresponding UL transmission is at most 584us.

### CAPC and COT

After Type 1 LBT based CCA is successful, the gNB or the UE can occupy the channel for a period, i.e., channel occupancy time (COT). The COT is randomly selected by the gNB or the UE with a maximum COT time, i.e., Tmcot,p based on CAPC p value. For example, the UL maximum COT time, Tulmcot,p, for different p’s is provided in Table 4.2.1-1, where a lower CAPC value means higher priority, and a higher CAPC value means lower priority. As seen in Table 4.2.1-1, when using a higher CAPC, due to smaller CWmax and maximum COT Tulmcot,p, a device on average accesses the channel faster but for a shorter duration.

As specified in TS 38.300, the CAPC of radio bearers and MAC CEs is either fixed or configurable. When configuring the CAPC, the gNB considers the 5QIs (5G QoS Indicator) of all the QoS flows multiplexed in that DRB while considering fairness between different traffic types and transmissions. The mapping between the CAPC and 5QI is shown in Table 5.6.2-1 in TS38.300 copied below, which shows which CAPC should be used for which standardized 5QIs i.e., the CAPC value to use for a given QoS flow.

Table 5.6.2-1 (TS38.300):  Mapping between Channel Access Priority Classes and 5QI

| CAPC | 5QI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |
| 2 | 2, 7, 71 |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

## Channel Access Mechanism for Sidelink in Unlicensed Spectrum

### LBT based Channel Access vs Legacy Sensing Based Resource Reservation

In sidelink licensed transmission, two resource allocation modes for channel access were specified, i.e., Mode 1 and Mode 2 resource allocations.

- Mode 1 resource allocation: the gNB performs the scheduling for UE’s transmissions in sidelink. Therefore, Mode 1 is for the UEs under gNB’s coverage.

- Mode 2 resource allocation: the UE performs the sensing for resource selection by detecting the PSSCH reservations from other UEs. Based on the sensing outcome, the UE selects resources for its transmissions in sidelink.

A two-stage SCI structure was adopted in sidelink. The first stage SCI provides future periodic or aperiodic resource reservations, i.e., the information necessary for sensing.

As described in Section 2.1, in NR unlicensed (NR-U) access, the device accesses the channel following a LBT based procedure. When the channel is determined to be idle after LBT, the device occupies the channel within a generated COT which is subject to a maximum COT specified based on CAPC.

We can see that LBT based channel access in NR-U and sensing based resource reservation in sidelink sensing are both based on sensing. However, the objectives of the two channel access methods are different. LBT based channel access procedure with a generated COT is more contention-based channel access, i.e., the device accesses the channel in a greedy manner, while sidelink sensing-based channel allocation with congestion control is to achieve fair sharing of the spectrum.

The channel access based on CAPC is mandated for any type of device that operates in EU 5GHz unlicensed bands and is specified by ETSI. The LBT based channel access in the shared spectrum is aligned with the requirements specified for compliance in European harmonized standard. Therefore, as described in SID, the NR-U LBT based channel access mechanism is reused for sidelink in unlicensed spectrum. If LBT based channel access is adopted in sidelink unlicensed access, a question arises whether we still need a sensing-based resource reservation. Sidelink sensing is a key feature to avoid transmission conflicts and achieve better resource sharing and resource utilization among sidelink UEs. The existing design of the PSCCH and SCI structure facilitates sensing-based resource allocation and fair channel access. Hence, we prefer to retain the existing sidelink channel reservation structure. For each reserved PSSCH, LBT based CCA or other types of LBT based channel access procedures are performed before the transmission based on the time gap between two transmissions.

Proposal 1: Reuse NR-U LBT based channel access mechanism while still retaining the sidelink resource reservation from Rel-16/Rel-17.

NR-U specifies the LBT based channel access procedures for both DL and UL. Although the basic LBT procedures in DL and UL are the same, the indication and the CAPC based settings are different. For Mode 1 SL transmissions, the gNB provides the scheduling grant to the UE for its SL transmissions after the UE sends a scheduling request to the gNB via PUCCH, which is similar to UL transmissions. NR-U UL channel access procedures can be considered for Mode 1 transmission in unlicensed spectrum.  For Mode 2 SL transmissions, the UE reserves the resources and accesses the channel by itself. It may be more appropriate to reuse NR-U DL channel access mechanism for Mode 2 transmission in unlicensed spectrum. Thus, we need to study channel access procedures for SL Mode 1 and Mode 2 separately.  Also, four types of LBT based channel access procedures were specified in NR-U. We support all four types of LBT. But a brief study is needed to confirm the applicability.

Proposal 2: Study applicability of existing Type 1 and Type 2 LBT based DL and UL channel access procedures for sidelink Mode 1 and Mode 2 transmissions in unlicensed spectrum.

### LBT and UE Types

In Rel-16 NR V2X, full sensing-based resource allocation is supported. In Rel-17 sidelink enhancement, two new Tx capabilities were introduced to reduce power consumption, i.e., random resource selection (RRS) and partial sensing. In order to support full sensing and partial sensing, the UE needs to support PSCCH reception. Therefore, energy detection can be supported without much increase on the power consumption. For RRS, the UE either does not have the capability of PSCCH reception or is fully capable of PSCCH reception but (pre-)configured not to perform sensing in a pool. However, the energy detection in LBT is simply an energy measurement without needing of decoding PSCCH. Therefore, UE that only supports RRS Tx UE feature can also support LBT based channel access.  In summary, all types of SL UEs can support LBT based channel access in unlicensed spectrum, meaning that these UE Tx features in Rel-16 and Rel-17 can be supported in sidelink unlicensed access.

Proposal 3: Support all sidelink Tx features in Rel-16 and Rel-17 in sidelink unlicensed access.

### CAPC and priority in sidelink

As discussed in Section 2.1, the settings for Type 1 LBT based channel access procedure is categorized by the four CAPC values. The CAPC is chosen based on the 5QIs (5G QoS Indicator) of all the QoS flows considered by gNB.

In sidelink, the priority of data is provided by the upper layers which is based on PQI (PC5 QoS Indicator) values. The mapping between the priority level and PQI values is presented in TS 23.287 with different QoS requirements such as packet delay budget, packet error rate, etc. Based on the priority of data traffic, a PSSCH priority is assigned in PHY, which is indicated in the priority field in SCI 1-A by a 3-bit number, p=0,...,7 (total 8 levels) in SL transmissions. As with CAPC, a lower value means a higher priority.

The CAPC is required for Type 1 channel access procedure in unlicensed spectrum, which should be specified for sidelink QoS traffic in sidelink unlicensed access. However, the 5QI used for CAPC does not capture all QoS requirements in PQI specified in TS 23.287. Therefore, it is necessary to build a table between CAPC and PQI or expand Table 5.6.2-1 in TS38.300 for CAPC-5QI mapping with PQI included.

Observation: It is necessary to build a table between CAPC and PQI or expand Table 5.6.2-1 in TS38.300 for CAPC-5QI mapping with PQI included.

Since CAPC is not defined for all possible values of SL priorities used for resource reservation, one potential issue may occur is that when the CAPC for unlicensed channel access and SL resource priority are used independently and based on the existing spec, the two priorities may be inconsistent in terms of QoS requirement. For example, as shown in Table 5.4.4.-1 TS23.287, for SL application of video sharing for cooperative collision avoidance (PQI=90), the SL priority is quite high (3 on scale of 1-8) and the delay requirement is also stringent with packet delay budget of 10ms. From NR 5QI table (Table 5.7.4-1 in TS23.501), except for conversational video (live streaming), the 5QI values for other video buffered streaming services are 4, 6, and 8, with the packet delay budget of 300ms, which is much larger than that for video sharing in sidelink. The corresponding CAPC for 5QI = {4,6,8} is 3. If this is used for sidelink service, it will result in an undesirable lower priority and higher latency for channel access. Such inconsistency may lead to unfair channel access and resource allocation, which may be detrimental to QoS for different flows. Therefore, the mapping between CAPC and SL priority should be studied for QoS consistency between the two priorities. In addition, CAPC should be made available to PHY for LBT based channel access procedures in SL either independently or based on a specified mapping between CAPC and SL priority.

Proposal 4: Study the appropriate priority mapping between CAPC and sidelink priority for QoS consistency in SL-U.

### Mode 1 Configurations for SL Unlicensed Access

In NR-U, certain parameters are configured by the network and signaled to UE, for instance, energy detection threshold, CAPC, etc. As specified in TS38.212, CAPC used for uplink transmissions in unlicensed spectrum is configured by the network for SRB2 and DRBs via DCI format 0_1.  In SL, the necessary parameters for SL unlicensed access by the network are not available. In SL Mode 1 resource allocation, gNB schedules UE’s sidelink transmissions. DCI format 3_0 is used to inform the SL UE the scheduled PSSCH. Like DCI format 0_1, we can extend the existing DCI format 3_0 to include the necessary parameter settings and configurations from gNB for SL unlicensed access. Similarly for DCI format 3_1 which is used for scheduling of LTE PSCCH and LTE PSSCH in one cell.

Proposal 5: Extend existing DCI formats, e.g., DCI format 3_0 and DCI format 3_1, to include necessary parameters, e.g., energy detection threshold, CAPC, etc., for SL unlicensed access.

### COT Sharing

COT sharing is supported in NR-U. In DL transmissions, the COT at the gNB is signaled to the UE in DCI format 2_0. COT sharing in DL helps the UE to determine which signals are expected to be present on the DL. COT knowledge at the UE also allows the UE to change the periodicity for monitoring PDCCH, i.e., for  frame based periodic access, more frequent monitoring within a COT from gNB and less frequent outside a COT, to achieve power savings.

In NR-U, COT at the UE can also be shared with gNB via UCI. The UL COT sharing allows the DL transmissions following the CG PUSCH in the same COT so that the network only needs to perform Type 2 LBT without going through longer Type 1 LBT channel access procedures.  Via UCI, UE informs the gNB the start time for potential DL transmission, and the remaining time in the COT.

COT sharing can be supported in sidelink unlicensed access. Following a successful Type 1 LBT, the UE initiates a COT. The UE may inform other UE(s) via unicast or groupcast the COT remaining duration. Other UEs may reserve the resources and transmit the transmission right after the transmission from the COT initiating UE without LBT if the gap between the transmissions from two UEs is short enough, i.e., less than 16ms. If the gap is between 16 to 25 ms, the UE receiving the shared COT only needs a short LBT, e.g., Type 2 LBT, before its transmissions. If the gap is larger than 25ms, a Type 1 LBT may be required while still sharing the COT.

The remaining COT duration can be sent via a second stage SCI in a new field. The UE may also send the energy detection threshold when indicating the remaining COT, where the threshold is used by the other UE when performing LBT.

Proposal 6: Support COT sharing for SL transmissions in unlicensed spectrum.

### Congestion Control

SL congestion control is based two measurements, i.e., channel occupancy ratio (CR) and channel busy ratio (CBR) measurements. With the measured CBR, a corresponding CRlimit can be derived. With CRlimit and CR measurement, a restriction is then applied for UE’s new resource allocation. The existing CBR and CR definitions only consider SL transmissions. In shared spectrum, this is not the case as other RAT transmissions may occupy the channel. If other RAT transmissions are not considered in the measurements, these transmissions may negatively affect the performance of congestion control in sidelink.

As specified in 38.215, the SL CBR is defined as the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots configured by upper layer. For partial sensing, SL RSSI is measured in slots where the UE performs partial sensing and where the UE performs PSCCH/PSSCH reception within the CBR measurement window. The calculation of SL CBR is limited within the slots for which the SL RSSI is measured.

Since the channel may be occupied by other RATs during the CBR measurement window [n-a, n-1], we may modify the current definition for CBR measurement. In the modified CBR, the measurement would exclude the slots or resources from the CBR measurement window that are occupied by transmissions outside of SL UE RAT, i.e., the resources with measured RSSI below a threshold. Alternatively, the CBR measurement can also be updated by including the slots occupied by other RATs which is considered as the channel busy time.

Similarly for SL CR measurement. SL CR evaluated at slot n is defined as the total number of sub-channels used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b]. We can see that a similar issue arises on CR measurement when other RAT transmissions are present. When the channel is mostly accessed by other RATs, the measured CR will become very small if there is no change on the legacy CR definition. It will result in a large CRlimit on the SL UE so that the UE may allocate many resources which causes congestion. Similar solutions for SL CBR measurements can be applied, e.g., excluding the slots or resources that are occupied by other RAT from the CR measurement window.

Proposal 7: Modify the definition of CBR measurement in SL unlicensed access by taking into account the transmissions from other RATs.

Proposal 8: Update CR definition for SL unlicensed transmissions by taking into account the transmissions from other RATs.

# Conclusion

The remaining details for inter-UE coordination were discussed. We observe and propose the following:

Observation: It is necessary to build a table between CAPC and PQI or expand Table 5.6.2-1 in TS38.300 for CAPC-5QI mapping with PQI included.

Proposal 1: Reuse NR-U LBT based channel access mechanism while still retaining the sidelink resource reservation from Rel-16/Rel-17.

Proposal 2: Study applicability of existing Type 1 and Type 2 LBT based DL and UL channel access procedures for sidelink Mode 1 and Mode 2 transmissions in unlicensed spectrum.

Proposal 3: Support all sidelink Tx features in Rel-16 and Rel-17 in sidelink unlicensed access.

Proposal 4: Study the appropriate priority mapping between CAPC and sidelink priority for QoS consistency in SL-U.

Proposal 5: Extend existing DCI formats, e.g., DCI format 3_0 and DCI format 3_1, to include necessary parameters, e.g., energy detection threshold, CAPC, etc., for SL unlicensed access.

Proposal 6: Support COT sharing for SL transmissions in unlicensed spectrum.

Proposal 7: Modify the definition of CBR measurement in SL unlicensed access by taking into account the transmissions from other RATs.

Proposal 8: Update CR definition for SL unlicensed transmissions by taking into account the transmissions from other RATs.

# References

RP-213678, “NR sidelink evolution”, RAN#94-e, Dec. 2021.

ETSI EN 301 893 5 GHz RLAN; Harmonised Standard covering the essential requirements