3GPP TSG RAN WG1 Meeting #109-e	R1-2204102

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.3.1

Source:	Futurewei

Title:	Discussion on evaluation of AI/ML for beam management use case

Document for:	Discussion and decision

# Introduction

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] .

In RAN#94-e, the Rel. 18 NR_AIML_Air study was approved, and the scopes and objectives were agreed  [1]. The initial set of use cases agreed are:

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

For each use case under consideration, the following objectives were agreed [1]:

In this contribution, we present our views on the evaluation methodology and KPIs for performance and complexity evaluation pertinent to the AI/ML-based beam management use case.

# Evaluation methodology and KPIs for AI/ML based beam management

In [2], we have discussed potential sub use cases of the AI/ML-based beam management use case and we shared our views on the benefits of leveraging AI/ML techniques and potential standards impact for the sub use cases.  In this section, we further discuss the evaluation methodology and applicable evaluation metrics to assess the performance and complexity of AI/ML-based beam management use case.

## Evaluation methodology

For evaluation methodology (EVM) and assumptions for AI/ML based beam management, scenarios, channel models, antenna configurations and related parameters agreed in R1-2007151 [4] for Rel-17 can be used as a starting point. For each sub use cases, some adjustments may be needed. First, system level simulation to evaluate capacity and spectrum efficiency or user perceived throughput is not necessary for these use cases. It is rather sufficient to simply assess the accuracy of beam prediction performance.

For sub use cases of AI/ML-based beam prediction in spatial domain, datasets for training and testing can be generated according to some selected combinations of the agreed scenarios, channel models, and antenna configurations in [4]. UEs are randomly dropped in the system to generate channels accordingly without considering mobility. Study and evaluation can then be carried out in a rather straightforward manner.

For sub use cases of AI/ML-based beam prediction in time domain, scenarios with mobility should be used where the channel between the gNB and the UE evolves in time. To properly model and assess the performance of prediction in time domain, spatial consistency along the mobility trajectory needs to be ensured. However, this is not considered for Rel-17 beam management work. Therefore, mobility and spatial consistency modeling should be introduced in Rel-18 for this sub use case. For example, a simplified setup for mobility with spatial consistency may include a transmitter, and a few scatters randomly located within a certain area, may be considered. UE may travel in several random directions at a fixed speed inside the area. Scattered channels with spatial consistency, considering the path gains, delays, phase shifts etc., could be generated as UE moves along its trajectories for performance evaluation of beam prediction in time domain.

Observation 1: System level simulation to evaluate capacity and spectrum efficiency or user perceived throughput is not necessary for the use case of beam management.

Proposal 1: For sub use cases of AI/ML-based beam prediction in time domain, mobility and spatial consistency modeling should be introduced.

## Evaluation metrics / key performance indicators (KPIs)

In this sub section, we discuss the KPIs for evaluating the performance and complexity of AI/ML-based beam management use case.

### Performance related KPIs

For both sub use cases, the goal is to predict the optimal beam pair accurately using input with either reduced spatial dimensions (for sub use case of “beam prediction in spatial domain”) or input from current and historical beam and/or other measurements (for sub use case of “beam prediction in time domain”). To measure the prediction performance of AI/ML-based approach, the following KPIs can be used.

- Top-1 prediction accuracy:

  - This metric calculates the ratio of testing samples in which the predicted best beam pairs match the ground truth, i.e., is the actual / true best beam pair). The top-1 prediction accuracy, denoted as Top1acc, can be expressed as

Top1acc=1Ui=1UOpti==Top1*i,

where U is total number of testing samples, Top1*i is the true optimal beam pair for testing sample i, and Opt(i) is the predicted best beam pair for testing sample i.

- Top-K prediction accuracy, where K=3 is recommended:

  - This metric calculates the ratio of testing samples in which the predicted best beam pairs are among the actual top-K beam pairs. For example, assuming K=3, this metric measures whether the predicted best beam pair is the true best beam pair (top-1) or is the true second-best beam pair (top-2) or is the true third-best beam pair (top-3). The Top-K prediction accuracy when K=3, denoted as Top3acc, can be expressed as

Top3acc=1Ui=1U(Opt(i) ∈ {Top1*i,Top2*i,Top3*i}),

where U is total number of testing samples, Top1*i, Top2*i  and Top3*i are the true first, second and third best beam pairs respectively for testing sample i, and Opt(i) is the predicted best beam pair for testing sample i.

- RSRP

  - This KPI is to evaluate the AI/ML-based beam prediction performance by comparing the (measured) RSRP for the optimal pair from prediction result with the (measured) RSRP measurements for the true optimal beam.

Proposal 2: Include Top-1 prediction accuracy (Top1acc) as one of the evaluation metrics for AI/ML-based beam management use case.

Proposal 3: Include Top-K (e.g., Top-3) prediction accuracy as one of the evaluation metrics for AI/ML-based beam management use case.

Proposal 4: Include measured RSRP as one of the evaluation metrics for AI/ML-based beam management use case.

### Complexity related KPIs

Assumption: the discussion in this sub section assumes that a neural network (NN) based AI/ML architecture is leveraged for beam prediction in spatial domain and beam prediction in time domain.

In designing AI/ML-based approach, it is very useful and important to take into account its associated complexity to make sure the requirements for memory and computation can be supported by the hardware (HW) and software (SW) environments in which the AI/ML model will be hosted/used. Understanding the complexity also helps in designing more HW, SW and data efficient AI/ML models. The term “complexity” for neural network-based models may cover many aspects, like memory, computation, time, space, energy, etc. while some of the aspects may be HW or SW environment-dependent, e.g., time and energy.

For NN based AI/ML models, complexity mainly concerns two aspects that are not dependent on the HW or SW environment, the first is the amount of space or memory needed and the other aspect is the amount of computational operations required to run the NN model.

- Space: this aspect mainly relates to amount memory usage during training and inference.

- Computational operations: this aspect mainly concerns the computation power and time needed to support or run the designed NN model.

In the following, we briefly describe space (or model) complexity and computational complexity, and their components while the details can be found in [3].

- Model (or space) complexity can be characterized using the following:

  - Number of parameters in AI/ML model(s): this measures the number of coefficients of the NN model, which need to be optimized during training according to a given optimization strategy.

  - Memory usage

    - Memory usage of NN-based AI/ML model consists of various components and number of parameters is one of the contributors. In general, memory usage is separately calculated for training and inference and includes the following aspects:

      - Training time:

        - Memory used to store outputs of the intermediate layers (forward propagation)

        - Memory used to store parameters

        - Memory used to store error signals

        - Memory used to store gradient of parameters

        - Additional memory used by the chosen optimizer, e.g., for momentum

      - Inference time:

        - Memory used to store outputs of the intermediate layers

        - Memory to store parameters

Figure 2.2.2-1: Memory usage during model Training vs. memory usage during Inference

Figure 2.2.2-1 depicts the memory usage during training time and inference time.

As shown in Figure 2.2.2-1, true memory usage depends on implementation. In addition, memory usage during training time can be significantly larger than memory usage during inference time; thus, it is desirable to calculate them separately. This is also very helpful when model training and inference are taken place in different entities. For example, in the scenario when model training is performed on gNB while UE side is doing the inference, UE only needs to consider the memory usage during inference time.

- Computational complexity: computations in (deep) neural network or deep learning-based AI/ML model is dominated by multiply-adds in fully connection layers and convolutional layers. Typically, number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs), is used to estimate the multiply-adds operations in the forward propagation, and any non-linearities, dropout, and normalization layers can be ignored as they have negligible computation cost.

  - Floating Point Operations (FLOPs): separate estimation for encoder and decoder is preferred.

    - For fully connected layer:

FLOPs = number_of_inputs  number_of_outputs (ignoring biases)

    - For convolution layer and variants of convolutional layer: Chapter 2 (Model Efficiency: Metrics of model efficiency) in [3] describes the formulas for calculating FLOPs for convolution layer and its variants.

Observation 2: When leveraging AI/ML-based approach, it is very important to understand the associated complexity to make sure the requirements for memory and computation can be supported by the hardware (HW) and software (SW) environments in which the AI/ML model will be hosted/used.

Observation 3: Complexity for NN-based AI/ML models can be measured using model (or space) complexity and computational complexity, which reflect the memory usage and computation power needed respectively.

Proposal 5: Include model/space complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for beam management use case. Model/space complexity measurements include number of parameters in the model and memory usage.

Proposal 6: Include computational complexity as one of the KPI list to evaluate the complexity of AI/ML-based approach for beam management use case. Computational complexity can be estimated using number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs).

# Conclusions

In this contribution, we present our views on the evaluation methodology and KPIs for performance evaluation pertinent to the AI/ML-based beam management use case.  Based on the discussions in the previous sections, our observations and proposals are as follows.

Evaluation methodology:

Observation 1: System level simulation to evaluate capacity and spectrum efficiency or user perceived throughput is not necessary for the use case of beam management.

Proposal 1: For sub use cases of AI/ML-based beam prediction in time domain, mobility and spatial consistency modeling should be introduced.

Evaluation metrics:

Observation 2: When leveraging AI/ML-based approach, it is very important to understand the associated complexity to make sure the requirements for memory and computation can be supported by the hardware (HW) and software (SW) environments in which the AI/ML model will be hosted/used.

Observation 3: Complexity for NN-based AI/ML models can be measured using model (or space) complexity and computational complexity, which reflect the memory usage and computation power needed respectively.

Proposal 2: Include Top-1 prediction accuracy (Top1acc) as one of the evaluation metrics for AI/ML-based beam management use case.

Proposal 3: Include Top-K (e.g., Top-3) prediction accuracy as one of the evaluation metrics for AI/ML-based beam management use case.

Proposal 4: Include measured RSRP as one of the evaluation metrics for AI/ML-based beam management use case.

Proposal 5: Include model/space complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for beam management use case. Model/space complexity measurements include number of parameters in the model and memory usage.

Proposal 6: Include computational complexity as one of the KPI list to evaluate the complexity of AI/ML-based approach for beam management use case. Computational complexity can be estimated using number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs).

# References

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94-e.

R1-2204103, “Discussion on sub use cases of AI/ML for beam management use case”, Futurewei.

B. Wu, Efficient Deep Neural Networks, PhD thesis, EECS Department, University of California, Berkeley, Aug 2019.

R1-2007151, Moderator summary for multi-beam enhancement: EVM, Samsung.