3GPP TSG RAN WG1 Meeting #109-e	R1-2204104

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.4.1

Source:	Futurewei

Title:	Discussion on evaluation of AI/ML for positioning accuracy enhancements use case

Document for:	Discussion and decision

# Introduction

In RAN#94-e, the Rel. 18 NR_AIML_Air study was approved, and scope and objectives were agreed  [1].

The initial set of use cases includes:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

For each use case under consideration, the following objectives were agreed:

In this contribution, we present our views on the evaluation methodology and KPIs for performance evaluation pertinent to the AI/ML-based positioning accuracy enhancement use case.

# Evaluation methodology and KPIs for AI/ML based positioning accuracy enhancements

In [2], we have discussed the sub use cases under the AI/ML-based positioning accuracy enhancements use cases and we shared our views on the benefits of leveraging AI/ML techniques and potential standards impact for “AI/ML-based LOS / NLOS classification” and “AI/ML-based UE position estimation” sub use cases. In this section, we focus on discussing the evaluation methodology and needed KPIs to assess the performance and complexity of AI/ML for positioning accuracy enhancements for these sub use cases.

## Release 17 evaluation methodology and KPIs

Rel-17 study item of “Study on NR Positioning Enhancements” has agreed and adopted an evaluation methodology with various use cases, both indoor and outdoor, and covering macro as well as indoor deployments [3]. These use case categories considered in Rel-17 include IIOT and general commercial use cases. In this section, we essentially propose to reuse these assumptions and parameters for the Rel-18 study on AI/ML for positioning accuracy enhancements, specifically for the two sub cases that we proposed in [2].

### Evaluation methodology

Scenarios:

As shown in [3], scenarios with heavy NLOS/multipath situations are more challenging for existing methods to meet performance targets (both horizontal and vertical positioning performance were worse for InF-DH scenarios vs. InF-SH scenarios). In Rel-18, we study the potential of leveraging AI/ML techniques to improve positioning accuracy that can be achieved in Rel-17, particular for those scenarios with significant NLOS conditions.

For the study of AI/ML-based LOS / NLOS classification, we would propose that the IIOT scenarios as considered in Rel-17 to be the main scenario. For IIOT use cases, the following scenarios specified for Rel-17 study can be reused for Rel-18:

-	Scenario 1. InF-SH for FR1 and FR2

-	Scenario 2. InF-DH for FR1 and FR2

For the more general objective of AI/ML-based UE position estimation, we would propose a commercial use case. In Rel-17, we reused the scenarios and channel models from Rel-16 as specified in TR 38.855 [4] and the following scenarios can be reused for Rel-18:

-	Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

-	Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

Scenario Parameters:

In Rel-17, [3] specifies common parameters applicable to all scenarios in Table 6-1 and these parameters can be reused in Rel-18 study.

For IIOT use case, [3] specifies common parameters for InF scenarios (including FR1 and FR2) in Table 6.1-1 and these parameters can be reused in Rel-18 study.

For UMi and UMa scenarios under general commercial use cases, in [4], Table 6.1.1-4 specifies parameters for Scenario 2: Urban micro (UMi) scenario and Table 6.1.1-6 specifies parameters for Scenario 3: Outdoor macro (UMa) scenario. We can reuse these scenario parameters in Rel-18 study.

Proposal 1: For evaluation methodology, reuse the following scenarios specified in [3]:

- IIOT use case:

  - Scenario 1. InF-SH for FR1 and FR2

  - Scenario 2. InF-DH for FR1 and FR2

- General commercial use cases:

  - Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

  - Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

Proposal 2: For scenario parameters, reuse the following:

- Common parameters applicable to all scenarios: reuse the common parameters for Rel-17, specified in Table 6-1 of TR 38.857 [3].

- Common parameters for InF scenarios: reuse the common parameters for Rel-17, specified in Table 6.1-1 of TR 38.587 [3].

- Parameters for Urban micro (UMi) scenario: reuse parameters specified in Table 6.1.1-4 of TR 38.855 [4].

- Parameters for Urban macro (UMa) scenario: reuse parameters specified in Table 6.1.1-6 of TR 38.855 [4].

### Evaluation metrics and performance targets

Clause 5.2 of [3] has specified the following performance evaluation metrics for Rel-17:

For evaluating performance of NR positioning technologies, the following metrics apply. The following percentiles of positioning error are analyzed: 50%, 67%, 80%, 90%.

Clause 5 of [3] also documented the agreed-upon RAT-dependent positioning performance targets for general commercial use cases and IIoT use cases as follows:

In Rel-17 target positioning requirements for commercial use cases are defined as follows:

-	Horizontal position accuracy (< 1 m) for 90% of UEs

-	Vertical position accuracy (< 3 m) for 90% of UEs

-	End-to-end latency for position estimation of UE (< 100 ms)

-	Physical layer latency for position estimation of UE (< 10 ms)

In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

-	Horizontal position accuracy (< 0.2 m) for 90% of UEs

-	Vertical position accuracy (< 1 m) for 90% of UEs

-	End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

-	Physical layer latency for position estimation of UE (<10ms)

Note 1:	Target positioning requirements may not necessarily be reached for all scenarios and deployments

Note 2:	For some scenarios the requirement for Horizontal position accuracy can be relaxed to < 0.5 m in IIoT use cases.

Note 3:	All positioning techniques may not achieve the target positioning requirements over all scenarios.

The above-mentioned Rel-17 performance evaluation metrics and targets can be adopted as the KPIs and performance baseline for Rel-18 AI/ML for positioning enhancements use case.

Proposal 3: Reuse the Rel-17 performance evaluation metrics and performance targets specified in TR 38.587 [3] on NR positioning enhancements for Rel-18 AI/ML-based positioning accuracy enhancements use case.

## Other KPIs

In this sub section, we discuss additional KPIs for evaluating the performance and complexity of AI/ML-based positioning accuracy enhancements. In our view, these additional KPIs are beneficial due to the use of AI/ML in this study and where applicable, can be considered optional metrics to facilitate better understanding and comparison of the different AI/ML approaches.

As discussed in [2], we propose considering both “AI/ML-based LOS / NLOS classification” and “AI/ML-based UE position estimation” as representative sub use cases for positioning accuracy enhancement. As existing Rel-17 evaluation metrics can be reused to cover the performance evaluation related to the “AI/ML-based UE position estimation” sub use case, we will focus on discussing other KPIs for “AI/ML-based LOS/NLOS classification” sub use case.

### Performance related KPIs

When discussing how good a classification algorithm works, the first thought is to use accuracy as the KPI for performance assessment. Typically, accuracy is used as a starting point to understand how good a classification algorithm works, however, accuracy is not necessarily a good indicator for real-world scenarios which we will discuss later in this sub section.

Before introducing performance metrics for AI/ML-based LOS / NLOS classification problem, we need to understand the following information (assuming positive detection means NLOS):

- True Positive (TP): A classification result that correctly identifies the presence of NLOS

- False Positive (FP): A classification result that wrongly identifies the presence of NLOS

- True Negative (TN): A classification result that correctly identifies the absence of NLOS

- False Negative (FN): A classification result that wrongly identifies the absence of NLOS

Figure 2.2.1-1: Typical Confusion Matrix

A general / common representation of the above information is using the confusion matrix as shown in Figure 2.2.1-1.

Accuracy of a classification outcome measures the number of correct predictions divided by the total number of predictions made, multiplied by 100 to turn it into a percentage. Using the above specified terms, it can be defined as following:

Accuracy = TP+TNTP+FP+TN+FN

In reality, it is very likely to have scenarios with imbalanced observations in which either LOS links are dominant or NLOS links are dominant. In such situations, as long as the classification algorithm makes good predictions on the majority/dominant class (e.g., LOS when LOS links are dominant in the data), it will achieve a high accuracy score regardless how good/bad the algorithm works in terms of predicting the minority class (e.g., NLOS when LOS links are dominant). Thus, measuring accuracy alone may provide mis-leading/incorrect information about the LOS/NLOS classification performance.

From the above description, it should be apparent that using accuracy alone is not sufficient to evaluate the performance of a classification algorithm.

We now discuss additional metrics needed to compliment the accuracy metric for AI/ML-based LOS/NLOS classification.

As the results of LOS vs. NLOS classification contain only 2 possible outcomes (or classes), either LOS or NLOS, for simplicity, the following metrics/KPIs are defined using the terms for classification explained above (TP, FP, TN, FN) to provide more accurate measures of the classification performance.

- Precision: Precision measures the percentage of positive instances out of the total predicted positive instances. It can be interpreted as ‘percentage of instances the model outputs the right class label when it says the input is classified as the specific class’. Following is its mathematic form.

Precision = TPTP+FP  , where TP = true positives, FP = false positives

- Recall: Recall measures the percentage of positive instances out of the total actual positive instances. It can be used to determine ‘how much extra right ones, the model missed when it showed the right ones’. Following shows its mathematic form.

Recall = TPTP+FN  , where TP = true positives, FN = false negatives

- F1-score: In some situations, using one metric may be more convenient and F1-score can serve this purpose. F1-score takes the harmonic mean of Precision and Recall. It combines the contribution of both, so higher the F1-score, the better.

F1-score = 2 * precision * recallprecision + recall

As F1-score can be derived from the Precision and Recall, there is no need to include it as another KPI, however, it can be calculated as a reference after the Precision and Recall calculations.

Observation 1: For LOS / NLOS classification sub use case, using accuracy alone is not sufficient to evaluate the performance of the classification algorithm.

Proposal 4: Adopt additional KPIs in assessing AI/ML-based LOS / NLOS classification performance. Examples of such metric to be considered are the ‘Precision’ and ‘Recall’ metrics as described.

### Complexity related KPIs

In designing AI/ML-based approach, it is very useful and important to take into account its associated complexity to make sure the requirements for memory and computation can be supported by the hardware (HW) and software (SW) environments in which the AI/ML model will be hosted/used. Understanding the complexity also helps in designing more HW, SW and data efficient AI/ML models.

As AI/ML modeling choices are left to implementation, the complexity evaluation / calculation may be different for neural network (NN) based approach vs. non-NN based approach. This sub section discusses complexity calculation for NN-based approach and non-NN based approach separately.

#### Neural Network based AI/ML models

The term “complexity” for neural network-based models may cover many aspects, like memory, computation, execution time, energy, etc. while some of the aspects may be HW or SW environment-dependent, e.g., execution time and energy.

For NN based AI/ML models, complexity mainly concerns two aspects that are not dependent on the HW or SW environment, the first is the amount of space or memory needed and the other aspect is the number of computational operations required to run the NN model.

- Space: this aspect mainly relates to amount memory usage during training time and inference time.

- Computational operations: this aspect mainly concerns the computation power and time needed to support or run the designed NN model.

In the following, we briefly describe model (or space) complexity and computational (or operation) complexity, and their components while the details can be found in [5].

- Model (or space) complexity can be characterized using the following:

  - Number of parameters in AI/ML model(s): this measures the number of coefficients of the NN model, which need to be optimized during training according to a given optimization strategy.

  - Memory usage

    - Memory usage of NN-based AI/ML model consists of various components and number of parameters is one of the contributors. In general, memory usage calculation is separately calculated for training and inference and includes the following aspects:

      - Training time:

        - Memory used to store outputs of the intermediate layers (forward propagation)

        - Memory used to store parameters

        - Memory used to store error signals

        - Memory used to store gradient of parameters

        - Additional memory used by the chosen optimizer, e.g., for momentum

      - Inference time:

        - Memory used to store outputs of the intermediate layers

        - Memory to store parameters

Figure 2.2.2.1-1: Memory usage during model Training vs. memory usage during Inference

Figure 2.2.2,1-1 depicts the memory usage during training time and inference time.

As shown in Figure 2.2.2.1-1, true memory usage depends on implementation. In addition, memory usage during training time can be significantly larger than memory usage during inference time. It would be better to calculate them separately. This is also very helpful when model training and inference are taken place in different entities. For example, in the scenario when model training is performed on gNB while UE side is doing the inference, UE only needs to consider the memory usage during inference time.

- Computational complexity: computations in (deep) neural network or deep learning-based AI/ML model is dominated by multiply-adds in fully connection layers and convolutional layers. Typically, number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs), is used to estimate the multiply-adds operations in the forward propagation, and any non-linearities, dropout, and normalization layers can be ignored as they have negligible computation cost.

  - Floating Point Operations (FLOPs): separate estimation for encoder and decoder is preferred.

    - For fully connected layer:

FLOPs = number_of_inputs  number_of_outputs (ignoring biases)

    - For convolution layer and variants of convolutional layer: Chapter 2 (“Model Efficiency: Metrics of model efficiency”) in [5] describes the formulas for calculating FLOPs for convolution layer and its variants.

#### Non-Neural Network based AI/ML models

For non-NN based AI/ML models, it is not intuitive to measure the number of parameters and memory usage to store those parameters, outputs, and other intermediate information as some may not be applicable. Instead, Big O notation is popularly used to classify AI/ML algorithms according to how their operations (run time) or space requirements grow as the input size grows. In the following, we briefly describe space complexity and time complexity.

- Space complexity

  - The space complexity reflects the amount of space taken by an algorithm to complete the task as a function of the size of the input, and/or configurations/settings of the AI/ML model.

- Computational (or time) complexity

  - The operation (or time) complexity reflects the number of operations taken by an algorithm to complete the task as a function of the size of the input, and/or configurations/settings of the AI/ML model.

Example: assuming a classification task and given n as number of data points in the dataset, d as the dimensions of the data, and c as the number of classes. If the amount of space needed by an AI/ML algorithm to perform the classification task grows with n, d and c linearly, then its space complexity can be expressed as O (n  d  c).

Like NN-based AI/ML models, the space complexity and the operation / time complexity may be significantly different during training time and inference time for non-NN based AI/ML models; thus, it is recommended to calculate them separately. This is also very helpful when model training and inference are taken place in different entities.

Based on the above, our observations and proposals are as follows.

Observation 2: When designing or using AI/ML-based approach, it is important to take into account its associated complexity while measuring complexity for NN-based AI/ML models and non-NN based AI/ML models may be different.

Observation 3: Complexity for NN-based AI/ML models can be measured using model (or space) complexity and computational complexity, which reflect the memory usage and computation power needed respectively.

Observation 4: Complexity for non-NN based AI/ML models can be represented using the Big O notation for space complexity and computational (or time) complexity, which reflect the amount of space and number of operations taken by the AI/ML algorithm respectively.

Proposal 5: Adopt model / space complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for positioning accuracy enhancements use case.

For NN-based models, model or space complexity measurements include number of parameters in the model and memory usage. For non-NN-based models, space complexity reflects the amount of space taken by the AI/ML model to complete the task as a function of the size of the input, and/or configurations/settings of the AI/ML model.

Proposal 6: Adopt computational complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for positioning accuracy enhancements use case.

For NN-based models, computational complexity can be estimated using number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs). For non-NN-based models, computational complexity reflects the number of operations taken by the AI/ML model as a function of the size of the input, and/or configurations/settings of the AI/ML model.

# Conclusions

In this contribution, we present our views on the evaluation methodology and KPIs for performance evaluation pertinent to the AI/ML-based positioning accuracy enhancement use case.  Based on the discussions in the previous sections, our observations and proposals are as follows.

Existing evaluation methodology and KPIs for positioning enhancement:

Proposal 1: For evaluation methodology, reuse the following scenarios specified in [3]:

- IIOT use case:

  - Scenario 1. InF-SH for FR1 and FR2

  - Scenario 2. InF-DH for FR1 and FR2

- General commercial use cases:

  - Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

  - Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

Proposal 2: For scenario parameters, reuse the following:

- Common parameters applicable to all scenarios: reuse the common parameters for Rel-17, specified in Table 6-1 of TR 38.857 [3].

- Common parameters for InF scenarios: reuse the common parameters for Rel-17, specified in Table 6.1-1 of TR 38.587 [3].

- Parameters for Urban micro (UMi) scenario: reuse parameters specified in Table 6.1.1-4 of TR 38.855 [4].

- Parameters for Urban macro (UMa) scenario: reuse parameters specified in Table 6.1.1-6 of TR 38.855 [4].

Proposal 3: Reuse the Rel-17 performance evaluation metrics and performance targets specified in TR 38.587 [3] on NR positioning enhancements for Rel-18 AI/ML-based positioning accuracy enhancements use case.

Other KPIs from AI/ML modeling perspective:

Observation 1: For LOS / NLOS classification sub use case, using accuracy alone is not sufficient to evaluate the performance of the classification algorithm.

Observation 2: When designing or using AI/ML-based approach, it is important to take into account its associated complexity while measuring complexity for NN-based AI/ML models and non-NN based AI/ML models may be different.

Observation 3: Complexity for NN-based AI/ML models can be measured using model (or space) complexity and computational complexity, which reflect the memory usage and computation power needed respectively.

Observation 4: Complexity for non-NN based AI/ML models can be represented using the Big O notation for space complexity and computational (or time) complexity, which reflect the amount of space and number of operations taken by the AI/ML algorithm respectively.

Proposal 4: Adopt additional KPIs in assessing AI/ML-based LOS / NLOS classification performance. Examples of such metric to be considered are the ‘Precision’ and ‘Recall’ metrics as described.

Proposal 5: Adopt model / space complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for positioning accuracy enhancements use case.

For NN-based models, model or space complexity measurements include number of parameters in the model and memory usage. For non-NN-based models, space complexity reflects the amount of space taken by the AI/ML model to complete the task as a function of the size of the input, and/or configurations/settings of the AI/ML model.

Proposal 6: Adopt computational complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for positioning accuracy enhancements use case.

For NN-based models, computational complexity can be estimated using number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs). For non-NN-based models, computational complexity reflects the number of operations taken by the AI/ML model as a function of the size of the input, and/or configurations/settings of the AI/ML model.

# References

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94-e.

R1-2204105, “Discussion on sub use cases of AI/ML for positioning accuracy enhancements use case”, Futurewei.

TR 38.857, “Study on NR Positioning Enhancements” V17.0.0.

TR 38.855, “Study on NR Positioning Support (Release 16)”, v16.0.0.

B. Wu, Efficient Deep Neural Networks, PhD thesis, EECS Department, University of California, Berkeley, Aug 2019.