3GPP TSG RAN WG1 Meeting #109-e	R1-2204105

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.4.2

Source:	Futurewei

Title:	Discussion on sub use cases of AI/ML for positioning accuracy enhancements use case

# Document for:	Discussion and decision Introduction

In RAN#94-e, the Rel. 18 NR_AIML_Air study was approved, and scope and objectives were agreed  [1].

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

The initial use cases to focus for the study are as follows.

In this contribution, we present our views on sub use cases under the AI/ML-based positioning accuracy enhancements use case.

# AI/ML for positioning accuracy improvement

Positioning has long been recognized as an important technology / feature for the 5G cellular networks due to the large number of commercial use cases that would benefit from access to position information. During the SI phase of Rel-17 study item of “Study on NR Positioning Enhancements” [2], companies have conducted extensive studies and analysis on the enhancements needed in NR positioning and we should leverage and reuse these evaluations as much as possible.

One of the main challenges in UE positioning estimation is for NLOS prevalent deployments as existence of NLOS links can greatly reduce the accuracy in location estimation using analytics-based methods. For commercial 5G deployment use cases in urban areas, many demand high positioning accuracy. However, urban areas also suffer greatly from NLOS conditions as buildings and other moving obstacles block and reflect radio signals. In general, for scenarios with heavy NLOS probability, UE positioning estimation still has much room for accuracy improvement even with the mitigating specification supports introduced in Rel-17. Considering this, further distinguishing LOS vs. NLOS links should continue to be one of the important tasks in UE positioning estimation.

In recent years, thanks to the advances in computing power, artificial intelligence (AI) / machine learning (ML) has achieved great success in various areas like function approximation, image classification, and natural language processing. AI/ML techniques are thus well suited for addressing classification problems as well given its success in image classification. Applying AI/ML-based approaches to the identification of LOS/NLOS conditions has shown promising and accurate results in academia research works, thanks to the strength of AI/ML in extracting the pattern and latent features from multiple dimensions of channel features [3]. However, most of the proposed AI/ML-based approaches were verified using data generated from simplified assumptions / configurations for simulation and are not validated using practical parameters for real communication network deployment scenarios.

During RAN#94-e NWM discussion, companies agreed that AI/ML has good potential in improving the accuracy for UE positioning in various scenarios [1], especially and including those with many NLOS paths. Not to be deprioritized, improving the positioning estimation for horizontal and vertical domain in multipath channels remain an important objective. Hence, we see two important sub use cases for AI/ML-based positioning accuracy enhancements to focus on:

- AI/ML for positioning estimation

- AI/ML for LOS / NLOS scenario classification

The goal of leveraging AI/ML is to improve the overall positioning accuracy including scenarios with many NLOS paths, e.g., outdoor macro deployments with severe multipath and mobility. To achieve such goal, correct identification of LOS vs. NLOS paths such as in indoor IOT (IIOT) is an important intermediate step, which can also benefit from AI/ML techniques as discussed above.

Even though the impacts to the Standards are yet to be studied and analysed, some initial considerations for the two above mentioned sub use cases are listed in Table 2-1.

Table 2-1: AI/ML for positioning accuracy enhancements: sub use cases

| Sub use case | Description | AI/ML Potential | Potential Standards impact |
| --- | --- | --- | --- |
| AI/ML for UE position estimation | Leverage AI/ML to predict estimate UE positions. | - Improve estimation accuracy in UE positioning for outdoor scenarios with severe multipath and mobility. | Level of impacts depends on algorithm design, which entity performs AI/ML model training / inference. |
| AI/ML for LOS / NLOS classification | Leverage AI/ML to classify LOS vs. NLOS scenarios/links. | - Improve LOS vs. NLOS classification accuracy, which will improve positioning estimation accuracy when different estimation approaches are utilized in scenarios with significant NLOS paths. | Level of impacts depends on algorithm design and which entity performs the model training/inference. |

Based on the above-mentioned summaries, we have the following observation.

Observation 1: AI/ML-based LOS / NLOS classification for scenarios with significant NLOS conditions and AI/ML-based position estimation enhancements for outdoor multipath environment can potentially improve the UE position estimation accuracy in addition to the enhancements that are currently supported in Release 17.

We thus propose:

Proposal 1: Support “AI/ML-based UE position estimation” and “AI/ML-based LOS / NLOS classification” as sub use cases for AI/ML-based positioning accuracy enhancement.

# Conclusions

In this contribution, we present our views on potential sub use cases of the AI/ML for positioning accuracy enhancements use case.  Based on the discussions in the previous sections our observation and proposal are as follows.

Observation 1: AI/ML-based LOS / NLOS classification for scenarios with significant NLOS conditions and AI/ML-based position estimation enhancements for outdoor multipath environment can potentially improve the UE position estimation accuracy in addition to the enhancements that are currently supported in Release 17.

Proposal 1: Support “AI/ML-based UE position estimation” and “AI/ML-based LOS / NLOS classification” as sub use cases for AI/ML-based positioning accuracy enhancement.

# References

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94-e.

TR 38.857, “Study on NR Positioning Enhancements” V17.0.0.

Machine Learning-Enabled LOS/NLOS Identification for MIMO System in Dynamic Environment IEEE Transactions on Wireless Communications (Volume: 19, Issue: 6, June 2020)