3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204106

e-Meeting, 9th – 20th May, 2022

Agenda Item:	9.3.1

Source:	Ericsson

Title:	Evaluation of NR duplex evolution

Document for:	Discussion, Decision

Contents

1	Introduction	3

2	Background	4

2.1	Key aspects to consider for SBFD operation	4

2.1.1	Self-interference and link level aspects	4

2.1.2	Cross-link interference aspects	5

2.1.3	Other aspects	6

3	Radio and Antenna Modeling	7

3.1	gNB Modeling	7

3.1.1	gNB Transmitter	7

3.1.2	gNB Antenna and Self-Interference Channel Modeling	13

3.1.3	gNB Receiver	15

3.2	UE Modeling	16

3.2.1	UE Transmitter	17

3.2.2	UE Receiver	18

3.3	 Summary of Modeling Discussion	19

4	System Level Evaluation Scenarios	19

4.1	General	19

4.1.1 Antenna configurations	20

4.1.2	System level model of cross link and cross operator interference	21

4.1.3	UE clustering	22

4.2	Urban macro deployment	23

5	System Level Evaluation Methodology	24

5.1	Traffic	24

5.2	Performance Metrics	25

6	System Level Simulation Results	27

6.1	General	27

6.2	Simulation limitations	27

6.3 Simulation results for Urban Macro in FR1 with UE clustering	28

6.3.1	Resource utilization per direction	28

6.3.2	Coverage	30

6.3.3	User Throughput	31

Conclusion	34

References	37

Annex A	38

A.1 GMP coefficients for ~2 GHz GaAs PA at 491.52 MHz sampling rate	38

A.2 GMP coefficients for DPD	38

A.3 GMP coefficients for net effects of DPD and PA	39

Annex B: Parameters for system level evaluations	40

# 1	Introduction

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

The Rel-18 study item on evolution of NR duplex operation[1] contains the following list of objectives related to both subband non-overlapping full duplex (SBFD) and dynamic TDD, with the RAN1 objectives highlighted in yellow.

In this paper, we address the following aspects of the SID in the context of both SBFD and dynamic TDD:

- Deployment scenarios

- Evaluation methodology

  - Including radio and antenna modelling aspects

- Performance evaluation results at both link level and system level

# 2	Background

SBFD, i.e., simultaneous transmission and reception by a device in non-overlapped frequency resources within a carrier or within multiple aggregated carriers is considered in this SI for Rel-18. As is well known, full duplex in mobile communication networks can cause both self-interference (within a device) and cross-link interference (between different UEs or between intra and inter operator gNBs). Furthermore, enhancements to mitigation of co-channel and adjacent channel cross-link interference that can occur with dynamic TDD schemes without full duplex operation have also been brought up. Key motivations that have been cited for considering duplex enhancements are the potential for improved UL coverage and improved latency due to the provision of more UL opportunities compared to a static TDD UL/DL pattern.

Since the benefits of SBFD depend on careful antenna and hardware design and on the deployment scenario, it is important to evaluate performance with proper models in place that capture practical effects to the largest extent possible. In the following we discuss some key aspects that need to be considered in the study.

## 2.1	Key aspects to consider for SBFD operation

### 2.1.1	Self-interference and link level aspects

An important aspect  for SBFD operation in non-overlapped frequency resources within a carrier is the mitigation of the self-interference within a SBFD capable gNB. Some of the aspects associated to self-interference are as follows:

- Receiver linearity and dynamic range: The receiver(s) may need to be able to handle a very high power transmit signal (i.e., unwanted self-interference signal) and low power RX signal (i.e., wanted signal) in the radio front end with a potentially huge power difference (>100 dB). This implies e.g. very high receiver linearity and high dynamic range including ADCs.

- Non-linearities in Tx and Rx radio front ends: The radio front end in both TX and RX will have non-linear processes that impact interference estimation and cancellation including CFR, DPD, PA non-linearity distortions, antenna mis-calibration, phase discontinuities, and receiver Automatic Gain Control (AGC). The nature of these non-linearities is likely to be very specifically linked to the particular radio design and also potentially to the instantaneously transmitted signal, so that impacts may change dynamically depending on e.g. scheduled resource blocks (RBs), modulation order etc.

- Coupling between different transceivers in an AAS array: This could cause intermodulation effects between different transmitters. Any interference cancellation would need to consider couplings within the array.

- Reflections: Even if the transmitted signal is cancelled perfectly or there is very good antenna isolation between transmit and receive antennas, reflected signals occurring due to objects close to the gNB could be significantly stronger than the desired receive signal and need to be considered.

- Intermodulation products from the transmitter degrading the noise floor/sensitivity in the receiver.

- Selectivity of the receiver with respect to the RBs that are allocated for transmission and attenuation in transmit RBs.

- Limitations on the level of filtering that may be achieved with limited numbers of guard sub-carriers between the transmit and receive signals.

In addition to the above, power consumption can also be an issue for SBFD operations with potentially significant increase arising due to different solutions implemented for self-interference mitigation in the end-to-end TX/RX chains at a SBFD capable device.

It is important that the above aspects are carefully studied and modeled accurately to assess the possible gains from SBFD operation.

### 2.1.2	Cross-link interference aspects

While techniques to mitigate self-interference including possible high isolation antennas are needed to secure the link between a gNB and a UE, this is not enough to ensure good system level performance. That requires consideration of the entire system, particularly the effects of cross-link interference (CLI), between both gNBs (gNB-gNB CLI) and UEs (UE-UE CLI), as shown in Figure 1.

It is important to consider not only co-channel CLI (within an operator's network), but also cross/adjacent channel CLI, i.e., introduced by the co-existence with another operator deployed on an channel. While this is typically RAN4 territory, it was specifically added to the SID as a RAN1 objective. Although cross/adjacent carrier interference is supressed to some extent by filtering (ACLR and ACS), extensive studies have shown that inter-operator interference can be a limiting factor in deploying dynamic TDD in several key deployment scenarios. This is true also for SBFD operation as shown in Figure 2.

Figure 1: SBFD operation within one operator's network. Both gNB1 (victim) and gNB2 (aggressor) operate with SBFD.

Figure 2: SBFD operation in a two-operator scenario. gNB1 (victim) operates with SBFD; gNB2 (aggressor) operates in legacy mode (static TDD).

### 2.1.3	Other aspects

Apart from the self-interference and CLI, some additional aspects should be considered:

- To achieve sufficient isolation between the transmit and receive signals, the gNB antenna array may need to be partitioned such that a part of the array can transmit whilst the other section receives, with sufficient isolation between the sub-arrays implying not only physical separation but also other sophisticated schemes. This is needed even if the duplexing is on separate frequency resources. Partitioning of the array into two halves implies that for the downlink (DL), and for uplink (UL) reception at least during SBFD timeslots the effective antenna area is halved. As a consequence, at least a 3 dB array loss compared to a baseline of using the full available array area for TX or RX during separate slots arises.

- Even if spatial isolation between sub-array sections and frequency isolation creates reasonable amounts of isolation between transmit and receive, it is still likely that interference cancellation is needed. If separate frequency resources are used for DL and UL, then the DL related interference to UL will consist of e.g. PA non-linearity products (even with PA linearization) that are not suppressed by adjacent channel filtering. Cancellation of PA unwanted emissions is a more complex task than traditional cancellation of known in-band signals. There are more sources contributing to DL related interference to UL such as PA thermal noise floor, DAC quantization noise and CFR distortion etc.

- If the DL and UL are isolated using spatial and frequency separation, then TX/RX reciprocity will be compromised to some extent, which may impact the ability to optimize throughput using advanced MIMO techniques.

- UL receiver performance can be degraded by the presence of strong DL signal close in frequency. The ability of UL receiver to avoid such degradation is called selectivity. Typical receiver selectivity needed for normal UL reception is not likely to be enough to suppress the strong DL signal. Achieving improved selectivity might require a more complex receiver and this should be investigated. In addition, the receiver phase noise will be superimposed on DL PRBs and leak into receiver which is called reciprocal mixing of phase noise.

- System simulations measuring performance gains should employ realistic traffic models, as opposed to a full buffer traffic model, to assess real-word performance.

Some or many of the factors described above need to be sufficiently modeled in the link and system level simulations in order to determine realistic gains for SBFD operation.

# 3	Radio and Antenna Modeling

## 3.1	gNB Modeling

### 3.1.1	gNB Transmitter

During SBFD slots, the gNB transmitter would generate DL NR waveforms that are situated adjacent with small guard to the UL RBs. For example, with a DL-UL-DL configuration of the non-overlapping subbands, the  gNB transmitter would generate DL NR waveforms on either side of the UL RBs. The digital and RF components in the transmitter chain will cause transmitter energy to be present across the UL RBs (i.e. adjacent to the transmitted RBs) in the form of unwanted emissions consisting of multiple impairments in the transmitter e.g. PA non-linearities. Although TX/RX antenna isolation solutions can to some extent suppress the power from these emissions, the power level of the emissions still needs to be suppressed sufficiently to avoid that the emissions power causes desensitization of the UL receiver. Even after suppression of the unwanted emissions, there may still be a need for further interference cancellation techniques in the UL receiver to further reduce the desensitization.

The degree to which transmitter unwanted emissions can be suppressed and cancelled is critical to achieving gains using SBFD, since desensitizing the receiver for SBFD UEs will reduce or eliminate the usefulness of the additional UL transmission opportunities. It is thus of key importance that the processes generating the unwanted emissions are properly modelled so that the feasibility of achieving sufficient suppression and cancellation with a real implementation is properly characterized.

Processes within the transmitter chain that cause emissions to the UL RBs include the following:

- Before conversion to an analog signal, digital filtering is used to confine the transmitted signal to be within the allocated RBs. This is needed because the OFDM waveform intrinsically contains energy at multiples of the FFT frequency points outside of the allocated spectrum. Design of the spectral confinement filters must take into account several factors, including the amount of suppression needed, the group delay caused by the filtering, the complexity (amount of taps), delay and cost of the filters in digital hardware implementation and in-band effects such as ripple caused by the filtering. Due to the need to trade-off these factors, the filtering will not be perfect and some amount of guard will be needed between the DL and UL resource blocks.

- The peak to average power ratio (PAPR) of the transmit signal must be managed in order that the analog PA can operate efficiently without large backoff and meeting linearity requirements. To achieve this, so-called crest factor reduction (CFR) of the transmit signal is applied in the digital domain, which basically involves clipping of large samples. Clipping is a non-linear function, and thus has the effect of generating distortion both inside and outside the bandwidth of the wanted signal. This is filtered in the digital domain in order to confine the clipping distortion in the adjacent channels and not contribute to increase of unwanted emissions that face stringent requirements. Note that the unwanted emission requirements are regulated by different administrations and shall be fulfilled under all circumstances.

- The digital signal must be converted to an analog waveform by means of digital to analog conversion. The D/A conversion process introduces further D/A quantization noise which will be amplified through the PA and leads to increased transmitter noise floor. In addition, thermal noise floor of the transmitter need to be considered. The noise floor will impact the energy transmitted in the UL RBs.

- The last stage of the analog processing is the amplification of the transmit signal using a power amplifier (PA). The PA will generate intermodulation products due to not having ideal linearity, which will fall into the UL RBs. There is a trade-off between operating the PA close to its maximum power for efficiency and the linearity of the PA. In order to optimize power consumption, techniques known as Digital Pre-distortion (DPD), which condition the input signal to the PA in order to counteract the PA non-linearities are applied in modern BS transmitters. A model is needed of the total net effect of PA and DPD.

- The oscillators, used for frequency conversion, in the analog chain will introduce phase noise (PN). PN needs to be modelled in order that the real-world performance of cancellation techniques can be properly assessed with a real signal.

Modelling is needed for all of the above mentioned effects. For assessing the level of emissions, models that generate power levels are sufficient. However, to assess self-interference cancellation algorithms, a more detailed representation of frequency and phase variation of the residual distortion is essential for a realistic assessment of the performance. This is important also to determine positive and negative interference patterns occurring in the receiver antenna array due to multiple transmit antennas, which may affect the performance on an individual basis in the different receivers.

#### 3.1.1.1	Initial example of modelling implementation for link level studies

In this section, we provide some example implementations for the transmitter models discussed in the above.

TR 38.803 Annex A lists several PA models suitable for link level studies. For instance, the intput, x(n), and output, yPA(n), relationship of a memory PA can be characterized by a generalized memory polynomial (GMP) :

yPAn=k∈Kal∈LaaklPAxn-lxn-l2k+k∈Kbl∈Lbm∈MbklmPAxn-lx(n-m)2k

An example model for commercially available GaAs PA for ~2 GHz was given in TR 38.803 Annex A with Ka=Kb=8, La=5, Lb=2, and M=1. The total number of parameters is hence 8×7=56. Its AM-AM and AM-PM responses are given in TR 38.803 and copied in Figure 3 for ease of reference.

(a) AM-AM characteristic	(b) AM-PM characteristic

Figure 3: Characteristics of ~2 GHz GaAs PA (copied from TR 38.803).

The expected input sampling rate is 307.2 MHz for the model parameters given in TR 38.803. To apply this model to an NR signal of 100 MHz, it is necessary to upsample the expected input sampling rate to 4×122.88=491.52 MHz. With this upsampling, the La set size increases by one. The sizes of the other sets remain the same. The total number of parameters is hence 8×8=64. The complete sets of parameters aklPA and bklmPA for the upsampled model are provided in Annex A.1 below.

It is well known that the nonlinear responses of the PA can generate substantial out-of-band emissions. To mitigate these issues while maintaining PA output efficiency, several common practices are adopted in a typical modern base station.

- A digital predistortion (DPD) is designed to preprocess the input signals to the PA in order to linearize the net responses of going through both DPD and PA.

- Crest factor reduction techniques are deployed to increase PA power efficiency.

Digital predistortion (DPD)

A digital predistortion (DPD) is designed to preprocess the input signals to the PA in order to linearize the net responses. The input/output relationship of a DPD can also be described by a GMP:

yDPDn=k∈KaDPDl∈LaDPDaklDPDxn-lxn-l2k

For the above PA, a DPD with KaDPD=8 and LaDPD=3 can be designed to achieve -45 dBc ACLR. The complete set of parameters aklDPD is provided in Annex A.2 below. The AM-AM response of passing through both DPD and PA is shown in Figure 4 (a), where the DPD and PA are modelled separately. Comparing to the AM-AM response shown in Figure 3, it can be observed that the PA has become substantially linearized by use of a DPD. Note that the PA compresses the input signal by up to 2 dB for normalized input power close to 1 as shown in Figure 3. The DPD correspondingly can only linearize input magnitude of at most -2 dB = 0.79433 relative to the peak magnitude of 1.

(a) simulation of DPD and PA with separate models		(b) net effect model of DPD+PA

Figure 4: AM-AM characteristics of DPD and PA.

One may observe that it may not be necessary or desirable to perform simulations using separate models for the DPD and PA in the link level studies. This is because what matters to the ACLR and leakage issue is the fidelity and representativeness of the net effect of both devices rather than those of the individual devices. Further, specific PA and DPD implementations may vary significantly from company to company. However, what can be common is that all these implementations are careful tradeoffs of complexity/costs/compliance. Hence, it may be more feasible for RAN1 to agree on a net effect model that captures the essential behaviors of a DPD and PA combination with -45 dBc ACLR compliance.

Since DPD is designed to linearize a nonlinear PA, the net effect of both devices is substantially more linear as discussed in the above. That is, such net effect can be captured by the a GMP model as

ynetn=k∈Kanetl∈Lanetaklnetxn-lxn-l2k

with fewer parameters than those for the PA alone. This can be quite beneficial to speeding up link level simulations. For instance, we found the net effect can be captured with the Kanet=8 and Lanet=5 and the complete set of parameters aklnet is provided in Annex A.3 below. The AM-AM response of the net effect model is shown in Figure 4 (b). It can be observed the (one-step) net effect model captures the behaviors of passing through the DPD and the PA quite well.

It is not necessary to perform link level simulations using separate models for DPD and PA.

Adopt a net effect model that captures the essential behaviours of a realistic DPD and PA combination with -45 dBc ACLR compliance. This requires input from RAN4.

Crest factor reduction (CFR)

It is well known the OFDM signal can be approximately viewed as a complex Gaussian signal with unbounded magnitude:

CCDFx(n)≈exp-xn2σ2

The PAPR of an example OFDM signal is shown in Figure 5(a) and is used as an illustration to demonstrate the effects of the models to be discussed below. It can be observed that the PAPR is around 9.4 dB as expected (measured at CCDF value 10-4).

(a) PAPR of OFDM signal			(b) PAPR of CFR processed OFDM signal

Figure 5: Example of PAPR of OFDM signal without or with CFR processing.

With such characteristics, inputs with large magnitudes to the DPD and PA (or their equivalent net effect) models can still drive into the nonlinear region and cause substantial out-of-band emissions. One approach to avoid these issues is to back off the input signal power such that the probability of unacceptable large magnitude becomes negligible. This however comes at substantial reduction of the transmit power. For instance, if the signal is scaled such that the mean sample power to the DPD and PA is around -12.2 dB, the spectra of such a signal going through the different models discussed in the above can be observed in Figure 6. The ACLR of this power back off approach will satisfy -45 dBc limit.

Figure 6: Example of spectra of OFDM signals with power back-off (mean sample power = -12.2 dB).

A better approach, which is used in practice, is to suppress the peak power to average power ratio (PAPR) of the OFDM signal to control the probability of unacceptable large magnitudes. A simple single-stage CFR model can be as follows:

- Apply hard clipper on the signal:

xn=xn,if x(n)≤LLej∠x(n),otherwise

where, for an example of clipping at 5.7 dB PAPR, the parameter L=105.7/10⋅avgxn2. Crest factor reduction, regardless of the exact approach used,  generates both distortions to the signal and out-of-band emissions as shown in Figure 7.

- Apply band pass filter with, e.g., 25 dB stop band suppression, on the hard clipper output.

It is therefore necessary to apply filtering after the hard clipper to suppress the out-of-band emissions as shown in Figure 7.

A simple single stage CFR would facilitate the modelling and selection of algorithms while it provides necessary characteristics of the distortion which is induced in UL PRB:s which  need to be considered when addition possible cancellation schemes are considered.

After applying the CFR processing, the PAPR of the OFDM signal is reduced to around 7.9 dB (measured at CCDF value 10-4) as shown in Figure 5(b). The error vector magnitude (EVM) for the OFDM signal after CFR is roughly 3%, where EVM is a measure of in-band distortion.

Figure 7: Example of spectra of OFDM signals before, during and after CFR processing.

The benefits of such CFR processing become clear when comparing the CFR processed example shown in Figure 8 with the power back-off example shown in Figure 6. With CFR processing, it is now possible to raise the input signal power to -10.4 dB, which represents a transmit power gain of almost 2 dB compared to the previous example – a significant increase. The ACLR of this CFR processed example will satisfy  -45 dBc limit.

Figure 8: Example of spectra of OFDM signals with CFR processing (mean sample power = -10.4 dB).

Adopt a simple crest factor processing model, e.g., hard clipping, that captures the essential behaviours of a BS design to increase transmit power. This requires input from RAN4.

We summarize the initial examples of gNB transmitter modelling incorporating the components discussed in the above in Figure 9. As discussed in the previous section, representing the various non-linearities in the transmit chain faithfully is essential to studying the impact of gNB self interference cancellation as well as gNB-gNB cross link interference for both SBFD and dynamic TDD. This is vital since both intra-operator and inter-operator CLI is due to spectral leakage, both inter-subband, and an inter-channel, due to these non-linearities. In this section we initiate generic modelling approach that can be used for both link-level and system-level evaluations. From this foundation, additional details and modelling of other transmitter components such as the DAC noises and phase noises can be further added.

Figure 9: Summary of initial example of gNB transmitter modeling.

### 3.1.2	gNB Antenna and Self-Interference Channel Modeling

The feasibility and performance of a suitable antenna structure that can achieve significant isolation between transmitted DL and received UL signals in SBFD slots is a key consideration for assessment. Firstly, it is necessary to assume an up-to-date and relevant baseline model that corresponds to a typical antenna array implementation in a state of the art basestation. Recently, the antenna model in TR 38.803 has been extended in order to properly model sub-array based array implementations. We propose that the model, considering sub-array based design and together with the antenna parameters agreed for the frequency range 1710-4990 MHz are used as a baseline for the SBFD study.

To create a model for a SBFD base-station, a modelling of the structure used for achieving TX/RX isolation is needed. The isolation structure may have two aspects to consider from a system behaviour perspective. Firstly, the relationship between the achieved isolation and the TX and RX beam directions (over steering range of the antenna array) needs to be understood and modelled, as there is the possibility that the isolation may vary significantly depending on beam direction. In addition, isolation to the different RX antennas may vary for the different sources of emission, effectively creating wavelength dependent interference patterns over the RX antenna array. Secondly, it should be investigated and understood whether the isolation structure has any impact on the TX and RX far field beam patterns, as this would impact the gain evaluations.

The needed isolation between RX and TX will depend on the deployment and the power levels. For FR1, high power deployments above the roof-top result in largest dynamic range considering transmitter with TRP of ~50 dBm and EIRP of ~80 dBm. This is quite common in current deployments, and the needed isolation could be significantly larger than ~80 dB, since for such deployments the sensitivity requirements due to UL coverage are quite stringent. For below the roof-top or small cell deployments not only the transmit powers are lower but also the receiver sensitivity is relaxed implying much lower dynamic range compared above the roof top. The needed isolation for different deployments and the feasibility of achieving the needed isolation need to be studied in detail.

In order to study feasibility of SBFD, it is necessary to consider interference cancellation algorithms on the RX side. The RX interference is created by coupling to a large set of TX sub-arrays, and in order to properly assess the complexity and gain of interference cancellation algorithms, it is necessary to capture the signal structure accurately. For this reason, a model is also needed that captures the gain and phase contributions for each of the TX/RX sub-array couplings for link level simulation.

Apart from modelling of the intra-gNB antenna interactions, it is also of importance to consider achievable isolation at site level. For a 3 sector site, backlobes from other sectors can cause interference towards a SBFD receiver. The suppression of the backlobes may cause degradation of the SBFD receiver sensitivity. For AAS, the backlobe impact to other sectors may depend on TX and RX beam directions. It is possible that the backlobe coupling may be lower for AAS BS than for passive antennas. In addition to other sectors, if other operators are co-located at the same site then a model of the coupling loss between other operators BS and SBFD BS is needed. This is in particular the case if the other operators are not operating SBFD.

Although more related to propagation characteristics than antenna structure, one further thing to consider is the propagation model between gNBs. If the gNBs are mounted higher than the UEs, this may be line of sight and so interference between gNBs needs to be modelled.

#### 3.1.2.1	Initial example of self-inteference channel modelling implementation for link level studies

The total self-interference on RX port n originating from TX ports m=0, 1, …M can be modeled as tapped delay lines,

snRxt=p=0P-1m=0M-1hpn,m ∙smTxt-τpn,m

where smTx(t) and snRx(t) are the transmitted and received signals, respectively, at time sample t, where hpn,m is the complex channel coefficient for tap p, and where τpn,m is the propagation delay for tap p.

As discussed above, the gNB antenna system may be equipped with sub-arrays in accordance with Section 5.2.3.2.4 of TR 38.803 v14.3.0 [2], in which case the individual elements within a TX or RX sub-array are assumed to be connected via fixed (complex-valued) weights, and each TX model chain in Section above3.1.1.1	Initial example of modelling implementation for link level studies is used to drive one sub-array which corresponds only to a one polarization. One can then let each TX or RX port in the sefl-interference channel model represent one sub-array as illustrated in Figure 10. This reduces channel model computational complexity and can also allow for capturing of non-linear coupling effects between elements within a sub-array if needed. Beamforming can then be performed by applying appropriate TX and RX weights on the sub-arrays.

The self-interference channel should be modeled as a set of tapped delay lines directly from TX sub-array ports to RX sub-array ports.

Figure 10: Illustration of self-interference channel modeling for panels with sub-arrays

Appropriate values for the channel coefficents and delays remain to be determined. There may be contributions both from direct propagation from TX ports to RX ports and from reflections from the environment. For the direct propagation path, it is crucial that the channel coefficients are based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations. For example, one needs to capture the fact that RX ports closer to the TX panel may have lower isolation than those farther apart, and that the channel coefficients may be frequency dependent.

Self-interference channel coefficients should be based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations.

### 3.1.3	gNB Receiver

Similarly to the transmitter, the operation of the radio within the gNB receiver will have an impact on the noise floor and sensitivity of the receiver chain. It is similarly important to model these effects in order that the receiver contributions to the overall UL link budget and gains are understood, and in order that receiver radio effects are captured in sufficient detail that the feasibility and efficiency of interference cancellation can be properly established.

Of particular relevance to the receiver is that, since the receiver must be able to be configured to different carrier frequencies within the band, and also since the receiver must be able to receive both full UL slots and an UL subband in SBFD slots, the analog front end of the receiver will experience both residue of the DL PRB:s and the received UL signal during SBFD slots. The DL PRB:s will be subject to TX to RX antenna suppression, but unlike transmitter noise leakage in the UL resource blocks, the DL carriers are not subject to any ACLR suppression. This will lead to a receive signal structure consisting of relatively high power signals coming into the receiver on the DL PRBs and for most cases a lower power RX signal from the UL RBs. The analog receiver must, even in the presence of the strong DL PRB:s pass the UL signal to the digital domain without significantly degrading the receiver noise floor implying linear operation without going to compression in any part. The impact of the mixture of large and small power signals must be modelled and understood in order to provide a proper input to system simulations of the receiver noise floor behavior.

Processes within the receiver that can impact the UL receiver sensitivity and should be considered include:

- The receiver will have a finite dynamic range (i.e., the difference between the largest power input signal and lowest power input signal). The power difference between the interferer level from the DL part and the UL receive signal needs to be within the dynamic range capability of the receiver to avoid degradations.

- In order for the receiver to operate within the dynamic range of the ADC when receiving a strong signal level, the receiver may operate an automatic gain control (AGC), which scales down the gain of the receiver, for instance the LNA and baseband. Scaling down the receiver gain will lead to an increase in the receiver noise figure, which will degrade the sensitivity. The behavior of AGC and it’s impact on the receiver noise floor depending on input signal levels needs to be understood and modelled.

- To further suppress power from the DL interferer (SBFD DL PRB:s) from reaching the ADC, analog filtering in the receiver might be needed. The feasibility of the needed analog filtering needs to be studied and understood by considering, for example, aspects such as the realization complexity and subband configuration flexibility.

- Digital filtering is needed to separate the UL receive RBs from the DL carrier in the digital domain. The filters need to trade off complexity, filter length, group delay, ripple etc. and will impact the characteristics of the receive signal. The impact of the filtering may impact the performance of digital cancellation algorithms and link level receiver performance. Also, some guard band between the DL and UL parts is likely to be needed, depending on the filter assumptions.

- The receiver needs to amplify the received signal using a Low Noise Amplifier (LNA). The LNA will, like any amplifier have some degree of non-linearity. If the DL interference signal has a significantly larger power than the UL receive signal then IM distortion from the DL part will arise in the UL receive band due to non-linearity in the receiver chain which is characterized by input third order intercept point. In order to characterize the noise floor and to properly model the signal that should be subject to interference cancellation, a suitable model for the receiver chain non-linearity should be used.

- The receiver may contain frequency mixer stages as part of downconversion. These also may create IM products, causing noise in the UL receive RBs due to the higher power DL part of the signal.

- The receiver phase noise will be subject to reciprocal mixing of phase noise with the higher power DL signal at low guard, which will lead to superposition of the phase noise in DL PRB:s and leak without filtering in the UL RBs that are to be received. A modelling of the phase noise and reciprocal mixing characteristics is needed since this impacts the noise level and the nature of interference in the UL.

To exemplify a single and simple receiver parameter, a 3GPP compliant wide area BS (high power BS for above the roof top deployment) must have the capability of receiving an interfering signal of -52 dBm in adjacent channel while degrading the receiver sensitivity by no more than 6 dB according to RAN4 BS specifications. Considering a gNB implementing SBFD, a transmitter with 50 dBm TRP and an assumed 80 dB of antenna isolation would result in interferer level induced by DL PRB:s in SBFD corresponding to -30 dBm which is significantly higher than a standard compliant spec could handle. Maintaining the noise floor and sensitivity would require ~20 dB additional reduction of the interferer level. The consequence of high interference level is either high sensitivity degradation than the RAN4 spec currently allows or the receiver being blocked or saturated.

Based on this discussion, the studies and modelling of the gNB receiver for an SBFD system is equally important as for the transmitter side.

The gNB receiver in an SBFD system needs to be carefully studied and modelled with respect to various aspects such as selectivity, linearity etc.

## 3.2	UE Modeling

In the following two subsections, we discuss current RAN4 requirements on emissions (ACLR and SEM) and selectivity (ACS) that can be used together to determine the power levels of UE-UE CLI when also accounting for pathloss between a pair of UEs. We discuss these both in the context of an intra-operator case where the UE-UE CLI is between subbands within the same carrier and also the inter-operator case where the UE-UE CLI is between different carriers.

### 3.2.1	UE Transmitter

The UE transmit and receive requirements in RAN4 were developed based on conventional UL scenarios in which all receivers, whether intended or victim are at basestations. For wide area BS, this implies a substantial minimum distance and coupling loss between the BS receiver and the UE, whilst when within a small BS deployment, UEs are not expected to transmit at maximum power.

For SBFD, UEs will transmit in RBs around the centre of the carrier within a DL slot and may be in close proximity to other UEs that are receiving DL. This scenario necessitates an investigation of interference to DL users during SBFD slots and towards other operators, whose UEs will be receiving DL.

A UE transmitter will generate unwanted emissions that will fall onto RBs carrying DL and can cause interference. Currently, UE unwanted emissions requirements are regulated in three ways.

- The Spectrum Emissions Mask (SEM) provides an upper limit to emissions Power Spectral Density (PSD) outside of the transmitted carrier, regardless of the UE output power. The SEM presents a worst case for the unwanted emissions levels. For frequency offsets further away from the carrier, the PSD is -25 dBm/MHz, but this is likely to be pessimistic compared to real emissions from a UE.

- The Adjacent Channel Leakage Ratio (ACLR) limits the ratio of power on the carrier to power immediately adjacent to the carrier

- The In-Band Emissions (IBE) limit the unwanted emissions power within the carrier on RBs that are not allocated to the UE. The IBE depends on a number of factors, but is of a similar order of magnitude to the ACLR.

Which requirement applies and which part of the requirement depends on the configuration of the UE; for example whether the UL RBs for SBFD operation are configured as a separate carrier or as a subband of a wider carrier.

Further investigation is needed ot how to apply the requirements and whether the requirements are sufficient to avoid UE-UE CLI.

As a quantitative example, an FR1 scenario is considered:

- Band n78 (3300 – 3800 MHz)

- Total bandwidth of SBFD carrier is 100 MHz

- During SBFD slots, UL transmission is in the central 20 MHz, surrounded by 40 MHz DL on either side

- Neighbor operator bandwidth is 100 MHz

- The UE is configured such that the 20 MHz transmission is handled as a 20 MHz carrier and adjacent channel emissions limits apply

Based on this scenario, the emissions towards the DL of other UEs of the same operator and towards the DL of other operators due to SBFD UE UL transmission can be estimated by assuming a UE that just meets RAN4 requirements.

#### 3.2.1.1	Emissions towards the DL part of the SBFD operator

For the first 20 MHz on either side of the UL, the total power of the Spectrum Emissions Mask (SEM) adds up to around 1.5 dBm. However, the UE must meet an ACLR requirement of 30 dB. This implies that if the UE meets the ACLR then the total emissions in the first 20 MHz of DL on either side of the 20 MHz UL will be -7 dBm for a PC3 (23 dBm) UE transmitting at full power. Obviously, the ACLR is more stringent than the SEM in this case and it can be assumed that the ACLR is met.

For the next 20 MHz on either side of the UL (20 MHz outer edges of the SBFD carrier) the separation from the UL transmission is 20 MHz. There is no ACLR limit in this case, but the SEM is -25 dBm/MHz, which translates to -12 dBm over the 20 MHz.

So the total emissions from an SBFD UE to the 40 MHz DL part of the carrier on either side is:

- Max -7 dBm in the first 20 MHz next to the UL transmission

- Worst case -12 dBm in the second 20 MHz; i.e. the outer 20 MHz of the SBFD carrier

- Addition of the above results in approximately -6 dBm over the whole 40 MHz DL on each side of the SBFD carrier corresponding to a UE at full output power

#### 3.2.1.2	Emissions towards the neighbor operator:

The neighbor operator is separated from the UL transmission by 40 MHz and has a 100 MHz bandwidth. The SEM in the RAN4 specificaiton is -25 dBm/MHz in this region, which adds up to -5 dBm in the 100 MHz carrier of the neighbor operator for a UE that just meets SEM. This is based on just meeting the RAN4 minimum SEM requirement and is likely pessimistic compared to real UE performance.

### 3.2.2	UE Receiver

The UE receiver requirements in RAN4 have been designed considering DL scenarios, in which the in-band receiver power is relatively similar over the whole channel bandwidth. Adjacent channel receive power can be larger than the wanted channel power if the UE is close to an aggressor basestation. However if the UE within a wide area network then there will be a significant minimum distance/coupling loss between the UE and the basestation, and with a low power BS the interference power will be lower.

A UE attempting to receive DL during an SBFD slot may experience a large interference signal if it is in proximity to a UE transmitting UL in the same SBFD slot. The UE blocking and selectivity requirements provide some indication as to the required rejection of such interferers by UEs. However, there is no RAN4 requirement on selectivity within the channel bandwidth.

An adjacent operator UE may also experience a significant interferer due to a SBFD UE transmitting nearby. However, the interferer will be further away in frequency for an adjacent operator DL than for the SBFD operator. The selectivity and blocking performance of adjacent operator UEs with a suitable frequency offset needs to be investigated.

Some quantitative consideration of receiver performance for a UE meeting the RAN4 minimum requirements is provided below, under the following assumptions:

- Band n78 (3300 – 3800 MHz)

- Total bandwidth of SBFD carrier is 100 MHz

- During SBFD slots, UL transmission is in the central 20 MHz, surrounded by 40 MHz DL on either side

- Neighbor operator bandwidth is 100 MHz

- The interfering UE is configured such that the 20 MHz transmission is handled as a 20 MHz carrier and adjacent channel emissions limits apply

For a UE of the same operator that is receiving the DL, if an interfering UE transmitting in the central 20 MHz is close by, then the interference will occur within the carrier. There are no RAN4 requirements for in-carrier selectivity. The adjacent channel selectivity (ACS) requirement is 33 dB. If the UE would respond to the 20 MHz in the same manner as to an adjacent channel interferer then 33 dB could be taken as an ACS in the first 20 MHz adjacent to the UL part of the carrier. RAN1/4 needs to study whether this selectivity is applicable to legacy pre-Rel-18 UEs and/or Rel-18 UEs.

There is no ACS requirement for the next adjacent channel, i.e. the next 20 MHz, at the outer edges of the SBFD carrier. Examination of the in-band blocking requirement reveals that it allows for a 6 dB degradation in sensitivity with a -56 dBm, 20 MHz blocker signal applied in the second 20 MHz adjacent channel. This implies an selectivity of around 34 dB in the outer 20 MHz of the DL part of the SBFD carrier. Since the selectivity needed to reject an in-band blocker in the 2nd 20 MHz channel is almost the same as the ACS in the 1st 20 MHz channel, we can summize that the selectivity over the whole 40 MHz of a UE just meeting the 3GPP requirement would be around 30 dB if the rejection of the UL signal in the centre of the SBFD carrier is the same as adjacent channel selectivity. Whether the selectivity can really be compared with adjacent channel selectivity and the achievable values needs further discussion in RAN4.

For the inter-operator case, there is again no selectivity requirement, but examination of the in-band blocking requirement reveals that a 6dB degradation in sensitivity is allowed for a -44 dBm, 5 MHz blocker placed in the 3rd 20 MHz adjacent channel from the DL; i.e. 40 MHz away from the edge of the DL carrier. This implies a selectivity of at least 46 dB towards the UL transmission for the adjacent operator. Real UE implementations may perform better than the minimum requirement.

Considering there is no RAN4 requirement on inter-subband selectivity within the channel bandwidth, RAN1/4 needs to study whether the rejection of UL signal in the UL part of an SBFD carrier by a UE receiving DL signal in the DL part is the same as adjacent channel selecitivity requirement.

## 3.3	 Summary of Modeling Discussion

In the prior sections, we have discussed various aspects of radio and antenna modelling that need to be considered as part of the RAN1 evaluations of SBFD. Many of these aspects require input from RAN4 in order for RAN1 to establish practical evaluation assumptions for both system level and link level, and we think it will be necessary to send an LS to RAN4 asking for input these aspects. In our companion contribution , we discuss and propose what input is required from RAN4 and propose that an LS should be sent.

Send an LS to RAN4 requesting feedback on various radio and antenna modelling aspects that are required for RAN1 to establish evaluation assumptions for both system-level and link-level simulations.

# 4	System Level Evaluation Scenarios

## 4.1	General

Section 2.1.2 above provides a description of the interference scenarios that occur between TDD and SBFD networks within the same operating channel and on adjacent channels. As shown in Figure 1, for a single operator network, UE-UE and gNB-gNB CLI interferences occur during periods of time in which both cells transmit in an SBFD slot using the same channel. However, in reality it is unusual that only a single network operates in a region. In commercial network deployments, there are multiple operators, at least two that may be co-located or in the vicinity of one another. This is shown in Figure 2. When SBFD is operated by one network (e.g., on Channel 1) and static TDD is operated by another (e.g., on adjacent Channel 2), the adjacent channel CLI between BS-BS (from static TDD to SBFD operator) and between UE-UE (from SBFD to static TDD operator) can cause co-existence issues. In the NR CLI RIM study in 38.828 [4], it was concluded that if dynamic TDD is operated at co-sited base stations, during subframes in which the victim BS is receiving uplink whilst an aggressor BS is transmitting downlink, the victim BS receiver would fail due to interference from the aggressor. No simulation analysis was done for the co-sited base station scenario, since the uplink receiver blocking will occur in all subframes for which one BS receives whilst the other transmits. However, for the SBFD case, there is a DL subband between the adjacent channel and the UL channel in the victim network (SBFD operator) and hence, this needs to be considered in the simulations for system level evaluations, since it is expected to reduce the gNB-gNB interference across operators, improving feasibility of the deployment compared to dynamic TDD. Furthermore, as we discuss in our companion contribution [5], it is beneficial to configure an "UL-only slot" in the TDD UL/DL pattern which is protected from CLI. Additionally, considering two operator networks with one performing static TDD is a way to study impact to legacy systems which is an explicit objective listed in the SID. In [6], we propose a phased approach to simulations focusing on two operator scenarios starting with Phase 0 characterized as follows:

Phase 0 (Baseline)

- Operator A (SBFD): same UL subband size for all gNBs

  - UL subband size is static

- Operator B (static TDD)

  - Time domain TDD DL/UL pattern is static, and the same as Operator A

This phase should consider the same baseline time domain TDD UL/DL pattern for both operators, e.g., D-D-D-D-U. Furthermore, slots with D-U-D configuration for Operator A are defined only in the 'D' slots of the time domain pattern. In addition, one of the main potential benefits of SBFD is the UL coverage enhancement. However, the UL coverage is heavily dependent on the UL resource allocation/subband size. Therefore, in order to study accurately the UL coverage enhancement of SBFD, the ratio of DL-UL resource needs to be realistic taking into account the impact on DL performance. Given the practical aspects discussed in Section 2, the overall gains or losses yielded by SBFD operation are highly scenario dependent. Hence, although a careful study of system level performance in a diverse range of scenarios is important, it is also equally important to consider gains beyond what can already be achieved using simple modifications such as employing TDD patterns with increased number of UL slots or using dynamic TDD opportunistically. It is important to focus on scenarios where system level performance improvements for SBFD is realistically possible.

Scenarios where SBFD performance improvements may be realistically possible and can be simulated/evaluated by participating entities should be prioritized.

Regarding evaluations, both FR1 and FR2 should be considered in the study.

### 4.1.1 Antenna configurations

Realistic antenna configurations providing realistic isolation should be assumed as part of the studied scenarios. Any impact of the needed antenna configuration on the output power and gains needs to be understood and taken into account. In addition, when comparing SBFD and Static TDD, it must be done with similar assumptions on antenna configurations. For example, same antenna gain could be assumed for transmissions by a static TDD gNB and a SBFD gNB. This would mean that antenna array size would have to be larger for the SBFD gNB. Alternatively, the antenna size could be halved for TX and RX in SBFD networks, but this can impact coverage. In addition, assuming that per antenna element power cannot be increased, when the number of antenna elements is halved, the total transmitted power will also be halved. This is illustrated as follows.

Figure 11 (Left) Same antenna gain for both static TDD and SBFD schemes, physical size of SBFD antenna is more than doubled. (Right) Almost same physical antenna size, number of antenna elements and thus antenna gain and TX power is halved compared to static TDD.

For system level evaluations, when comparing SBFD performance to static TDD, two cases should be considered: SBFD network with 1) same antenna array size (with halved transmit power) and 2) same antenna array gain (with same transmit power) as static TDD network.

### 4.1.2	System level model of cross link and cross operator interference

One essential part of system level model assumptions is how cross link (e.g. DL-UL) and cross operator (both cross link and in the same direction) interference is modelled, both within a channel and between adjacent channels. Adjacent channel aspects are traditionally handled by RAN4, but for this study item, they also need to be part of RAN1 evaluations. The RAN4 framework of adjacent channel leakage ratio (ACLR), adjacent channel selectivity (ACS) and adjacent channel interference ratio (ACIR) are all metrics that are tailored towards defining requirements, and in particular requirements that can be tested. For system simulations, in our view it would be preferable if RAN1 agrees on a simplified framework instead of trying to apply the RAN4 concepts directly. Naturally, when deriving the parameters for the RAN1 system simulation framework, RAN4 requirements, where available, should be taken into account.

In our view the most straightforward way of defining the amount of interference experienced by a victim node is in terms of a relative metric between the power spectral density (PSD) of the aggressor signal and the PSD of the resulting interference. In general, the interference PSD at the victim node is a combination of that the TX signal of the aggressor leaks into the bandwidth of the victim (roughly ACLR in RAN4 terms) and that the receiver at the victim picks up power outside of the intended bandwidth (roughly ACS in RAN4 terms). As discussed in Section 3, there are many factors effecting these and thus a number of different cases need to be considered, e.g. intra-network gNB-gNB and UE-UE CLI isolation inside of the SBFD network and inter-network BS-BS and UE-UE isolation (both CLI and non-CLI). In Figure 12 we illustrate the general concept by looking just at how the TX signal from the gNB effects the RX sub-band for SBFD.

Figure 12: Illustration of the proposed relative PSD metric taking only the TX side into account. For details, see the text related to Figure 8.

As can be seen in Figure 12, for SBFD, the TX signal leaks into the UL sub-band due to non-linearities in the transmit chain. In the SBFD case, there is in-channel leakage into the UL subband, and based on only the TX side, the relative metric would be roughly 45dB in this example which assumes that the gNB is able to meet a 45 dB ACLR requirement. In addition to the TX signal leaking into the UL sub-band, the UL receiver will also pick up power from the DL sub-bands that will add to the actual interference experienced at the receiver. Thus, for the purpose of system level evaluations, the net effect isolation level would need to be lower than 45dB to model both transmitter and receiver effects. In our initial evaluations we have for example used the value 42dB for this parameter. The exact effect on both the transmitter and the receiver side and the resulting net isolation level to use for different cases need to be studied and agreed by RAN1. A similar concept can be applied also for UE-UE (intra- and inter-network), BS-BS (intra and inter-network), BS-UE and UE-BS (inter-network). Based on this discussion we make the following proposal:

RAN1 to agree on net effect relative PSD interference metrics for system level evaluations taking into account both transmitter and receiver. In general, different values are needed for different interference cases.

### 4.1.3	UE clustering

As discussed above,one of the main potential benefits of SBFD is coverage enhancements. There are basically two reasons a UE can be in poor coverage, either it is very far away from the base station or it is indoor. To study the benefits of SBFD, sufficiently many UEs should be in poor coverage. In our opinion, the most relevant scenario for this is an urban macro deployment with a high proportion of indoor users. The urban macro cellular deployment scenario focuses on high user densities and traffic loads in city centres, dense urban areas and high traffic / event hot spots (i.e., UE clustering). The key characteristics of this deployment scenario are high traffic loads, high user density, outdoor and outdoor-to-indoor coverage for mobile UEs. For such a scenario, it is important to study UE-UE interference. For UE-UE interference to occur, UEs need to be in close vicinity to each other. In other words, for any UL transmission in the SBFD slot to exceed the co-channel interference level, the interfering UE needs to be close to the victim UE. This can occur when there are groups of indoor UEs clustered in buildings, similar to layout 2 in Table 5.2.1.1.2-1 in 38.828 [4]. One building per cell is placed randomly and the buildings should be open, so that there is no wall loss between UEs in the same building. In addition, we propose to use a scenario with 7 hexagonal cells with wrap around to limit evaluation time, especially when considering two operators. Here we have used the same building size as in 38.828, i.e. 120x50 m, however in our view, something smaller could also be considered.

Furthermore, while scheduling UEs in the UL, the allocated RBs for UL transmission (within the semi-static UL subband configured for SBFD) could be adaptive based on the offered traffic. For example, in static TDD with 100 MHz carrier BW, it is unusual for a gNB to schedule the whole 100 MHz to a single UE in UL, especially for UEs in poor coverage, or in cases with high DL traffic. Therefore, in the evaluations the UL resource allocation method needs to be reported.

Considering the above discussions, we provide an example of the layout per-operator that we think should be used as a baseline for generating multi-operator scenarios with various grid shifts.

Figure 13: Exemplary per-operator urban macro layout with randomly placed buildings with 70% indoor users. Users outside of buildings are all outdoor.

## 4.2	Urban macro deployment

For system level evaluations, we propose to consider urban macro deployment with two operators as a baseline. To properly study the effects of gNB-gNB CLI and to reflect real world deployments where operators are often co-sited or at least placed close to each other, we propose to study different grid shifts. One could e.g. consider 0% (co-sited), 10% and 100%.

Figure 14: Two operator urban macro with randomly placed buildings with 70% indoor users. Users outside of buildings are all outdoor. Example with 10% grid shift.

Considering the above discussions, we propose the following.

Regarding evaluation scenarios, two-operator urban macro scenario should be considered as the baseline scenario. We do not preclude other scenarios but it is important to study at least one real-world deployment.

It is important to consider UE-UE interference in the study which is a potential consequence of the clustered distribution UEs that are of indoors.

For multi operator scenarios, it is important to consider realistic grid shifts. This should also include lower grid shifts. 0% grid shift (co-sited operators) should also be considered with proper channel model assumed between the gNBs of different operators that may be deployed at different heights. Consider studying 0%, 10% and 100% grid shifts.

In the system level simulations, the scheduled bandwidth for UL transmissions (within a semi-statically configured UL subband) should be adaptive based on achieving a target S(I)NR, for both static TDD and SBFD networks.

# 5	System Level Evaluation Methodology

## 5.1	Traffic

Because the amount of CLI (between nodes/sectors and within nodes) varies with the system load, it is important to consider different load points in system level simulation. The criteria for traffic load could be based on resource utilization. For example, the resource utilizations can be below 10%, 25-35%, and above 55% for Low load, Medium load, and High load, respectively. For static TDD and SBFD the UL/DL resource allocation is fixed, I.e., it does not adapt to the input traffic, so that the utilization for DL and UL needs to be calculated separately. For simplicity, we propose to use an aggregated system utilization computed as rDLuDL+rULuUL, where r is the ratio of DL/UL resource allocation and u is the DL/UL resource utilization, i.e., the average ratio of used vs available resources in the respective direction. For example, for a 80:20 DL:UL split rDL=0.8 and rDL=0.2. If e.g. the utilizations for DL and UL turns out to be 0.45 and 0.90 respectively, the aggregated system utilization would be 0.8*0.45+0.2*0.9=0.54. Note that the utilization for DL and UL are outputs from the simulations and would depend on how efficiently the system handles the input traffic in the respective directions. For static TDD and SBFD we also think it is important to report the individual utilization values for UL and DL as it gives an idea of how loaded a particular direction is.

Another aspect is the split of DL and UL traffic. To not completely overload the UL in the static TDD system, we think it makes sense that the configured resource split is equal to the traffic split. For example, if the traffic DL:UL split is 80:20, then an 80:20 DL:UL split should be configured with an appropriate TDD UL/DL pattern.

When comparing different duplexing modes, CLI mitigation techniques and/or configurations, one can consider two options can be considered when it comes to input traffic; (1) same input traffic or (2) same resource utilization. The former makes it straightforward to evaluate metrics such as user throughput and latency, because the system’s ability to handle more or less traffic will be reflected in those metrics. However, if the latter option is used, comparing these metrics is not so straightforward, because the two systems are serving different amount of traffic. As a result, in our view, (1) is the preferred option. That is, one system is chosen as the baseline and then the input traffic is chosen to result in a certain utilization (Low, Medium and High) for that system. Then the same input traffic is used for the other cases, allowing metrics like user throughput to be compared.

When evaluating the impact on one network from another network it is important in addition to the basic case of same input traffic in both networks, also consider having unequal input traffic. In particular the case of high load in the aggressor network and low load in the victim network will reveal whether or not there are any coexistence issues.

In system level simulation, for the initial phase of performance evaluation comparison is performed between different duplex modes based on the same amount of input traffic within each networks.

For co-existence evaluations (e.g. between two networks), in addition to the case of same input traffic in the two networks, also consider high input traffic in the aggressor and low in the victim.

For traffic load in system level simulations, it is based on an aggregated system resource utilization between UL and DL denoted as rDLuDL+rULuUL, where r is the ratio of DL/UL resource allocation and u is the DL/UL resource utilization, i.e., the average ratio of used versus available resources in the respective direction.

## 5.2	Performance Metrics

One of the claimed benefits of SBFD is increased UL coverage. One way to visualize that is by reporting a low percentile user throughput, for example the 5%-tile. In addition, it is desirable to define a metric that reflects a dB improvement in coverage, analogous to coverage assessment in link budget studies. To define such a coverage metric, one can create a scatter plot of mean user throughput generated by system simulation as a function of the logged pathloss between gNB and UE. Different throughput values clustered around a similar path loss value are then averaged to create a single curve of average user throughput vs. pathloss. Then, based on such a plot, one can derive a maximum path loss (MPL) value for which a certain average bitrate can be maintained. For example, average bit rate values (10Mbps for DL and 1Mbps for UL) from “Target data rates for eMBB” in Table A.1-1 of 38.830 can be used as a basis for deriving the corresponding MPL. See Figure 10 for an illustration of the concept.

Figure 15 Example illustration of how MPL can be computed. Red curve is a moving average smoothing (using a window of 10% of the samples) of the blue average user throughput versus path loss samples. MPL is then defined as the highest PL value for which a specified average throughput (e.g. 1 Mbps) can be achieved.

In addition to the coverage metrics above, it is also important to study higher percentiles such as 50%-tile and 95%-tile both for UL and DL, because the proposed flexible duplex solutions could impact those metrics in a negative way. For example, UE-UE CLI in SBFD slots can negatively impact DL throughput and if only DL slots are used for SBFD operation, resources are transferred from DL to UL, which naturally will degrade DL performance.

Another claimed benefit of both SBFD and dynamic TDD is latency improvements. In our view these claimed latency improvements stem from two effects. The first one being that the probability that UL (or DL) resources are available when data arrives is higher and thus there is less time wasted waiting for an appropriate resource. The second being that the system has higher capacity, e.g. because more UL resources can be allocated if needed, power limited UEs can be given more time-domain resources instead of frequency-domain resources and for the case of dynamic TDD, UL and DL resources will better match the actual need. Here we can observe that the first effect is mostly present at low load when there is no queueing. Also, the potential improvements, will depend heavily on detailed physical layer aspects, such as scheduling, PDCCH/PUCCH capacity/periodicity, scheduling request periodicity and so forth. Once the load gets high enough, what really matters is the system capacity as queuing delays will dominate. Thus, in our view, many of the latency aspects can either be evaluated analytically (low load) or will follow the same trends as the throughput metrics discussed above (high load). In addition, we are also concerned that due to the required level of detail to capture the first aspect, aligning results among companies will be challenging. Based on this discussion, we do not preclude latency metrics per se, but we think they should be of lower priority compared to throughput metrics.

For performance metrics to be used in the system level simulations, the following are proposed:

- Resource utilization: Resource utilization per transmission direction

- Coverage: Target Maximum Pathloss (MPL) to achieve a certain average bit rate (10Mbps for DL and 1Mbps for UL).

- User Throughput: Mean user throughput and 5%ile user throughput

# 6	System Level Simulation Results

## 6.1	General

In this section, three different cases are considered for the evaluations as listed below. In the evaluations, Network 1 (N1) is changed between Static TDD and SBFD networks while Network 2 (N2) remains as static TDD for all cases.

- Case 1: (Static TDD)

  - N1: Static TDD urban macro network with DDDDU TDD pattern

  - N2: Static TDD urban macro network with same TDD pattern synchronized with N1

  - 0% and 100% grid shift between N1 and N2

- Case 2: (SBFD with same antenna gain, impliying double the antenna size – see Figure 11)

  - N1: SBFD urban macro network using DDDDU time domain TDD pattern with all D slots configured for SBFD, same offered traffic, and same antenna gain as case 1

  - N2: Static TDD urban macro network with DDDDU time domain TDD pattern synchronized with N1

  - 0% and 100% grid shift between N1 and N2

  - See Figure 2 for reference

- Case 3: (SBFD with same antenna area, implying same antenna size – see Figure 11)

  - N1: SBFD urban macro network using DDDDU time domain TDD pattern with all D slots configured for SBFD, same offered traffic, and same antenna area as case 1

  - N2: Static TDD urban macro network with DDDDU time domain TDD pattern synchronized with N1

  - 0% and 100% grid shift between N1 and N2

  - See Figure 2 for reference

## 6.2	Simulation limitations

Some limitations in the simulator may have the potential to influence the results in the system level simulations. Some of these aspects may be as follows:

- Ideal receiver model is assumed in the simulations. Clipping and dynamic range effects are not considered which means that the receiver has infinite dynamic range. As a consequence, the results for SBFD are a bit optimistic.

- No receiver beamforming performed at the receivers of victim nodes. This is because receiver beamforming is usually done in the digital domain. It is worth studying if the victim nodes can first handle the power levels in adjacent channel without any receiver beamforming.

## 6.3 Simulation results for Urban Macro in FR1 with UE clustering

### 6.3.1	Resource utilization per direction

Figure 16 Comparison of mean RU in DL (left) and UL( right) at low, medium and high loads for Network 1 deploying static TDD or SBFD  with 0% (top) and 100% (bottom) grid shift to Network 2 doing deploying static TDD

In the above figure, resource utilization per direction (UL, DL) is plotted. Three example network traffic load points are chosen based on the reference static TDD system. For the lowest load point the aggregated DL + UL resource utilization is 8% for low load, 29% for medium load and 66% for high load for 100% grid shift (see Section 5.1 for discussion on aggregated resource utilization). Similar values are obtained for 0% grid shift. The utilization values per direction are relative to the total number of resources available per direction i.e. the DL/UL resource split  (e.g. 80% DL and 20% UL for static TDD).

We can make several observations from the resource utilization per direction plots. First, for all load points, SBFD networks have higher mean DL utilization than a static TDD network coexisting with a static TDD network. This is because the amount of DL resources of the SBFD network has decreased in comparison to the reference static TDD network, while the load remains the same. On the other hand, the mean utilization for UL has decreased for SBFD networks in comparison with reference static TDD network, due to the fact that the amount of resources used for UL have increased. This is an obvious inherent aspect of SBFD: resources are traded from DL to UL to provide more UL transmission opportunities.

SBFD operation results in higher DL resource utilization and lower UL resource utilization by dedicating a portion of the DL resource for UL communications.

An SBFD network can maintain the same TX/RX beamforming gains by roughly doubling the total antenna size as shown in Figure 11. However, since there are fewer DL resource than a static TDD network, the DL resource utilization is still elevated in comparison to a static TDD network for the same traffic loads. If the SBFD network is equipped antennas with same antenna area, the DL resource utilization is further elevated because of the loss of output powers and beamforming gains. For instance, at medium traffic load, the DL utilization is increased from 25% for the static TDD network to 30% and 42% for SBFD networks with double- or same-sized antennas, respectively.

An SBFD network has elevated DL resource utilization in comparison to a static TDD network for the same traffic loads, particularly if the antenna area has to be maintained.

Compared to an SBFD network with double-sized antenna, an SBFD network with same-sized antenna achieves less UL resource utilization relative to a static TDD system. For instance, at medium traffic load, the UL utilization is reduced from 48% for the static TDD network to 22% and 28% for SBFD networks with double- or same-sized antennas, respectively. That is, the UL utilization is reduced by almost half. However, the UL resource utilization reduction relative to a static TDD system becomes smaller as the traffic load increases. At high traffic load, the SBFD networks reduce the UL utilization from 79% to around 70%. The reduction is less than a seventh.

An SBFD network has reduced UL resource utilization in comparison to a static TDD network for the same traffic loads. Less reduction is achieved if the antenna area has to be maintained or at higher traffic loads.

Between the 0% and 100% grid shifts, we observe small differences in terms of DL or UL resource utilization. We observe more pronounced differences between the two different grid shifts in the cell edge user throughput statistics to be discussed below in Section 6.3.3.

RAN1 studies SBFD’s effects on DL and UL resource utilization per direction to provide better understanding of SBFD operations.

### 6.3.2	Coverage

Figure 17 Comparison of Maximum Pathloss to achieve DL bitrate of 10 Mbps(left) and UL bitrate of 1 Mbps(Right) at low, medium and high loads for Network 1 deploying static TDD or SBFD with 0%(top) and 100%(bottom) grid shift to Network 2 doing deploying static TDD.

In the above figures, coverage is plotted based on the maximimum path loss (MPL) metric that we discuss in Section 5.2. In summary, MPL is the largest path loss that is allowed while still achieving a desired target average bit rate. The target bitrate assumed in the above graphs is for DL and 1 Mbps for UL. With this metric, a system with larger MPL has better coverage. At low and medium loads, an SBFD network with double-sized antenna (SBFD same gain) provides similar DL coverage as a static TDD network since both are equipped with equal output powers and beamforming gains. However, at high loads, the SBFD network even with double-sized antenna provides slightly worse coverage because of elevated network interference as already indicated by the DL utilization discussion in the above. An SBFD network with same-sized antenna (SBFD same area) provides clearly worsened DL coverage and significantly so at high traffic loads.

An SBFD network with double-sized antenna provides similar DL coverage as a static TDD network at low or medium traffic loads and slightly smaller coverage at high traffic loads. An SBFD network with same-sized antenna provides worsened DL coverage and significantly so at high traffic loads.

An SBFD network with double-sized antenna provides better UL coverage than a static TDD network at low traffic loads for both 0% and 100% grid shift. However, as the traffic loads increase, the coverage gains are lost and can turn into losses. An SBFD network with same-sized antenna provides slightly worse or slightly better UL coverage than a static TDD network at low traffic loads for the 0% and 100% grid shift cases, respectively. As the traffic loads increases, an SBFD network with same-sized antenna provides substantially worse coverage than a static TDD network.

At low traffic loads, an SBFD network with double-sized antenna provides better UL coverage than a static TDD network.  An SBFD network with same-sized antenna provides similar UL coverage as a static TDD network. As traffic load increase, the UL coverage gains are reduced and can turn into significant losses, particularly for an SBFD network with same-sized antenna.

RAN1 studies SBFD’s effects on DL and UL coverage under different traffic loads / resource utilization levels to provide better understanding of SBFD operations.

It is important to also view the coverage improvements in parallel with throughput improvements which is shown in the next sub-section.

### 6.3.3	User Throughput

In the following analysis, the mean user throughput and 5%ile throughput plot for DL and UL are plotted for all the cases 1 to 3 in Figure 18 and Figure 19, respectively.

Figure 18 Comparison of mean user throughput for DL (left) and UL (right) at low, medium and high loads for Network 1 doing deploying static TDD or SBFD with 0%(top) and 100%(bottom) grid shift to Network 2 deploying static TDD.

Figure 19 Comparison of 5%ile user throughput for DL (left) and UL (right) at low, medium and high loads for Network 1 deploying static TDD or SBFD with 0%(top) and 100%(bottom) grid shift to Network 2 deploying static TDD

We can make several observations from the user throughput plots. First, for all load points, SBFD networks have lower DL mean and cell-edge throughputs than a static TDD network coexisting with a static TDD network. This is because the amount of DL resources of the SBFD network has decreased in comparison to the reference static TDD network, thus preventing SBFD networks from achieving high DL user throughputs at low loads. At medium and higher loads, SBFD networks further suffer from higher network interference as the networks have been driven to higher resource utilization levels as discussed in Section 6.3.1 above. An SBFD network with same-sized antenna provides significantly worsened user throughputs, particularly to those at the cell edges. At low and medium loads, the cell-edge user throughputs of an SBFD network with same-sized antenna are less than half of those of an static TDD network. At high loads, the cell-edge user throughputs of an SBFD network with same-sized antenna are less than a tenth of those of a static TDD network.

An SBFD network provides lower DL mean and cell-edge user throughputs than those of a static TDD network. An SBFD network with same-sized antenna provides significantly worsened DL user throughputs.

An SBFD network with double-sized antenna provides better UL mean and cell-edge user throughputs than a static TDD network and significantly so at low loads. For instance, the mean UL user throughput is increased by more 50% and the cell-edge user throughput is increased by up to 100%, depending on the grid shift between the two coexisting networks. However, the throughput gains reduces with increased traffic loads and, at high loads, the UL mean and cell-edge user throughputs are similar for the types of networks. Note also, as discussed in Section 6.3.2 above, at medium and high traffic loads, an SBFD network with double-sized antenna provides smaller coverage than a static TDD network.

An SBFD network with double-sized antenna provides larger UL coverage and significantly better UL mean and cell-edge user throughput at low traffic loads than a static TDD network. At medium and high traffic loads, the SBFD network provides comparable UL user throughputs but smaller UL coverage than a static TDD network.

An SBFD network with same-sized antenna can provide significantly better UL mean user throughput than a static TDD network at low loads. But it fails to do so for the cell-edge users and, in fact, may even degrade the cell-edge user throughput depending on the grid shift. At medium and high loads, an SBFD network with same-sized antenna provides slightly worse mean UL throughput and significantly worse cell-edge user throughput than a static TDD network. In fact, at high traffic load, the cell-edge user throughput becomes zero.

An SBFD network with same-sized antenna provides comparble UL coverage and significantly better UL mean user throughput at low traffic loads than a static TDD network. At medium and high traffic loads, the SBFD network provides comparable UL mean user throughputs but at a smaller coverage. For cell-edge users, the SBFD network provides significantly worsened performance.

RAN1 studies SBFD’s effects on DL and UL mean and cell-edge user throughputs under different traffic loads and grid shifts to provide better understanding of SBFD operations.

Based on the discussions and simulation results, we propose to agree on system level parameters as listed in Annex B.

RAN1 to agree the system level simulation parameters listed in Annex B.

# Conclusion

In this contribution, we make the following observations and proposals.

Observation 1	It is not necessary to perform link level simulations using separate models for DPD and PA.

Observation 2	SBFD operation results in higher DL resource utilization and lower UL resource utilization by dedicating a portion of the DL resource for UL communications.

Observation 3	An SBFD network has elevated DL resource utilization in comparison to a static TDD network for the same traffic loads, particularly if the antenna area has to be maintained.

Observation 4	An SBFD network has reduced UL resource utilization in comparison to a static TDD network for the same traffic loads. Less reduction is achieved if the antenna area has to be maintained or at higher traffic loads.

Observation 5	An SBFD network with double-sized antenna provides similar DL coverage as a static TDD network at low or medium traffic loads and slightly smaller coverage at high traffic loads. An SBFD network with same-sized antenna provides worsened DL coverage and significantly so at high traffic loads.

Observation 6	At low traffic loads, an SBFD network with double-sized antenna provides better UL coverage than a static TDD network.  An SBFD network with same-sized antenna provides similar UL coverage as a static TDD network. As traffic load increase, the UL coverage gains are reduced and can turn into significant losses, particularly for an SBFD network with same-sized antenna.

Observation 7	An SBFD network provides lower DL mean and cell-edge user throughputs than those of a static TDD network. An SBFD network with same-sized antenna provides significantly worsened DL user throughputs.

Observation 8	An SBFD network with double-sized antenna provides larger UL coverage and significantly better UL mean and cell-edge user throughput at low traffic loads than a static TDD network. At medium and high traffic loads, the SBFD network provides comparable UL user throughputs but smaller UL coverage than a static TDD network.

Observation 9	An SBFD network with same-sized antenna provides comparble UL coverage and significantly better UL mean user throughput at low traffic loads than a static TDD network. At medium and high traffic loads, the SBFD network provides comparable UL mean user throughputs but at a smaller coverage. For cell-edge users, the SBFD network provides significantly worsened performance.

Proposal 1	Adopt a net effect model that captures the essential behaviours of a realistic DPD and PA combination with -45 dBc ACLR compliance. This requires input from RAN4.

Proposal 2	Adopt a simple crest factor processing model, e.g., hard clipping, that captures the essential behaviours of a BS design to increase transmit power. This requires input from RAN4.

Proposal 3	The self-interference channel should be modeled as a set of tapped delay lines directly from TX sub-array ports to RX sub-array ports.

Proposal 4	Self-interference channel coefficients should be based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations.

Proposal 5	The gNB receiver in an SBFD system needs to be carefully studied and modelled with respect to various aspects such as selectivity, linearity etc.

Proposal 6	Considering there is no RAN4 requirement on inter-subband selectivity within the channel bandwidth, RAN1/4 needs to study whether the rejection of UL signal in the UL part of an SBFD carrier by a UE receiving DL signal in the DL part is the same as adjacent channel selecitivity requirement.

Proposal 7	Send an LS to RAN4 requesting feedback on various radio and antenna modelling aspects that are required for RAN1 to establish evaluation assumptions for both system-level and link-level simulations.

Proposal 8	Scenarios where SBFD performance improvements may be realistically possible and can be simulated/evaluated by participating entities should be prioritized.

Proposal 9	Regarding evaluations, both FR1 and FR2 should be considered in the study.

Proposal 10	For system level evaluations, when comparing SBFD performance to static TDD, two cases should be considered: SBFD network with 1) same antenna array size (with halved transmit power) and 2) same antenna array gain (with same transmit power) as static TDD network.

Proposal 11	RAN1 to agree on net effect relative PSD interference metrics for system level evaluations taking into account both transmitter and receiver. In general, different values are needed for different interference cases.

Proposal 12	Regarding evaluation scenarios, two-operator urban macro scenario should be considered as the baseline scenario. We do not preclude other scenarios but it is important to study at least one real-world deployment.

Proposal 13	It is important to consider UE-UE interference in the study which is a potential consequence of the clustered distribution UEs that are of indoors.

Proposal 14	For multi operator scenarios, it is important to consider realistic grid shifts. This should also include lower grid shifts. 0% grid shift (co-sited operators) should also be considered with proper channel model assumed between the gNBs of different operators that may be deployed at different heights. Consider studying 0%, 10% and 100% grid shifts.

Proposal 15	In the system level simulations, the scheduled bandwidth for UL transmissions (within a semi-statically configured UL subband) should be adaptive based on achieving a target S(I)NR, for both static TDD and SBFD networks.

Proposal 16	In system level simulation, for the initial phase of performance evaluation comparison is performed between different duplex modes based on the same amount of input traffic within each networks.

Proposal 17	For co-existence evaluations (e.g. between two networks), in addition to the case of same input traffic in the two networks, also consider high input traffic in the aggressor and low in the victim.

Proposal 18	For traffic load in system level simulations, it is based on an aggregated system resource utilization between UL and DL denoted as rDLuDL+rULuUL, where r is the ratio of DL/UL resource allocation and u is the DL/UL resource utilization, i.e., the average ratio of used versus available resources in the respective direction.

Proposal 19	For performance metrics to be used in the system level simulations, the following are proposed:

	Resource utilization: Resource utilization per transmission direction

	Coverage: Target Maximum Pathloss (MPL) to achieve a certain average bit rate (10Mbps for DL and 1Mbps for UL).

	User Throughput: Mean user throughput and 5%ile user throughput

Proposal 20	RAN1 studies SBFD’s effects on DL and UL resource utilization per direction to provide better understanding of SBFD operations.

Proposal 21	RAN1 studies SBFD’s effects on DL and UL coverage under different traffic loads / resource utilization levels to provide better understanding of SBFD operations.

Proposal 22	RAN1 studies SBFD’s effects on DL and UL mean and cell-edge user throughputs under different traffic loads and grid shifts to provide better understanding of SBFD operations.

Proposal 23	RAN1 to agree the system level simulation parameters listed in Annex B.

# References

RP-213591, New SI: Study on evolution of NR duplex operation, December 2021.

3GPP TR 38.803, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)”, v14.3.0, March 2022.

Singh et. al., “Full Duplex Radios: Are we there yet?”, HotNets 2020.

3GPP TR 38.828, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16).

R1-2204107, "Sub-band non-overlapping full duplex," Ericsson, RAN1#109-e, May 2022.

R1-2204108, "Potential enhancements of dynamic TDD," Ericsson, RAN1#109-e, May 2022.

R1-2204109, " Inputs needed from RAN4 to facilitate RAN1 study," Ericsson, RAN1#109-e, May 2022.

# Annex A

## A.1 GMP coefficients for ~2 GHz GaAs PA at 491.52 MHz sampling rate

aklPA with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2,3]:

Columns 1 through 3

0.98947 -    0.15864i     0.011394 +  0.0059418i     -0.02595 +   0.015964i

-0.11904 +     3.4785i      0.16004 -    0.78936i     -0.29104 +    0.69356i

1.7868 -     24.829i       2.1494 -     1.3763i      0.25595 -     1.0622i

-47.427 +     88.258i      -18.278 +      11.46i       2.8096 +     1.7827i

288.93 -     131.07i       52.929 -     30.994i      -8.4112 -     4.6164i

-720.19 +      29.44i      -79.679 +      44.03i       12.366 +     7.7142i

802.16 +     101.94i       61.067 -     31.849i      -9.3202 -     6.5858i

-330.14 -     68.444i      -18.677 +      9.129i       2.8211 +     2.2413i

Columns 4 through 6

-0.021808 +   0.023932i      0.08137 -   0.066328i    -0.028821 +   0.020777i

0.13968 -     0.3031i    -0.055638 +    0.12317i    0.0025261 -   0.064521i

0.28392 -    0.03186i     -0.16621 -    0.21719i       0.2051 +    0.52131i

-4.9523 +     3.6373i       2.0428 -    0.11929i     -0.91185 -     2.7641i

14.48 -      11.55i      -4.1458 -    0.76602i      0.11711 +     9.2853i

-21.085 +     17.707i       3.5662 +     3.4663i       3.5276 -     16.405i

15.092 -     13.222i      -0.7511 -     4.5032i      -5.2505 +     14.281i

-4.1686 +     3.7789i      -0.3596 +     1.9478i       2.2631 -     4.8192i

bkl0PA with rows k∈[0,1,…,7] and columns l∈-1,1:

0.011394 +  0.0059418i    -0.021808 +   0.023932i

0.78918 -     1.1787i     -0.23118 +     0.3404i

-12.954 +     13.546i       2.9895 -     2.6228i

105.73 -     48.006i      -23.359 +     6.2294i

-440.48 +     47.942i       99.672 +      6.479i

938.49 +     62.701i      -219.66 -     47.314i

-973.27 -     152.07i       236.54 +     65.644i

387.58 +     78.941i      -98.001 -     29.277i

## A.2 GMP coefficients for DPD

aklDPD with rows k∈[0,1,…,7] and columns l∈[-1,0,1]:

0.96483 +    0.16281i     0.074212 -   0.057611i    -0.044338 +   0.017753i

-0.047821 -     2.4287i     -0.18448 +    0.58547i      0.12368 -    0.32944i

1.7583 +     14.992i      0.50158 -     1.6556i      -1.1999 +     2.0859i

16.111 -     59.002i      -10.091 +     15.336i       13.956 -     15.765i

-161.71 +     122.46i       86.205 -     76.998i      -77.987 +     63.627i

475.69 -     119.08i       -274.5 +     171.61i       211.71 -     121.89i

-586.85 +     33.916i       387.33 -     161.53i      -275.54 +     97.897i

261.71 +     8.9873i      -201.17 +     46.666i       137.13 -     19.644i

## A.3 GMP coefficients for net effects of DPD and PA

aklnet with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2]:

Columns 1 through 3

0.97682 +  0.0087922i     0.088493 -   0.036024i     -0.10163 +   0.021892i

-0.18767 +    0.26147i     -0.10634 -    0.70684i    -0.087813 +    0.72218i

2.2628 -    0.57732i       6.4539 +     5.8653i      -2.3677 -     3.6178i

-19.312 +     3.3353i      -82.174 -     67.744i       36.961 +     42.629i

95.195 -     12.917i       478.42 +     397.01i      -209.17 -     257.76i

-242.02 +     16.214i      -1454.2 -     1166.6i       610.96 +     752.22i

301.27 +     3.3988i       2197.4 +       1683i      -886.87 -     1065.6i

-145.14 -     13.699i      -1299.6 -     944.73i       504.12 +     583.62i

Columns 4 through 5

0.04581 +   0.016268i   -0.0034128 -   0.011955i

0.21594 -    0.46616i     -0.14818 +   0.082903i

-0.93584 +     2.7021i        1.057 -      1.041i

1.2219 -     35.968i      -3.8339 +     18.483i

-14.062 +     223.71i       16.852 -     121.05i

68.885 -     651.61i      -58.653 +     363.29i

-130.09 +     907.48i       106.37 -     516.96i

83.445 -      484.6i      -71.781 +     280.56i

# Annex B: Parameters for system level evaluations

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 7 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | Carrier Frequency | 4 GHz |
|  | Duplex Type | Static TDD, SBFD |
|  | Static TDD pattern | 80:20 DL:UL |
|  | SBFD pattern | 80:20 SBFD slot:UL |
|  | Channel bandwidth | 100 MHz for STDD 40:20:40 MHz (DUD) for SBFD |
|  | Available resource blocks | 273 for STDD 108:51:108 (DUD) for SBFD |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 1000 |
|  | Channel model | gNB-UE: UMa gNB-gNB: UMa with 100% LoS UE-UE: UMi |
|  | UE to BS min 2D distance | 25 m |
|  | Gridshift | 0%,100% |
| BS | (Mg, Ng, M, N, P), where M, N indicate sub-array number | (1,1,4,8,2) (same antenna gain) (1,1,4,4,2) (same antenna area) |
|  | Sub-array configuration | 3x1 |
|  | Max gNB Tx Power | 50 dBm (same antenna gain) 47 dBm (same antenna area) |
|  | (dv, dh) | (0.7λ, 0.5λ) |
|  | Antenna element gain | 6.4 dBi |
|  | Antenna element and sub-array model | R4-2109872, Table 1-2: Macro urban |
|  | Subarray electrical downtilt | 3 deg |
|  | Mechanical downtilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 |
|  | SNR target | 16 dB |
|  | Noise figure | 9 dB |
|  | Max modulation | 64 QAM |
|  | UE distribution outdoor:indoor | 100:0 Outside buildings 0:100 Inside buildings |
|  | Buildings | One 120x50 m building per cell placed randomly |
| Traffic | Traffic split | 80:20 DL:UL |
|  | Target Resource utilization | <10%, 25-35%, 55% |
| Intra-network isolation values | Antenna isolation for self-interference | 80 dB |
|  | Front to back ratio | 45 dB |
|  | Digital cancellation | 15 dB |
|  | UE-UE SBFD operator | 28 dB |
|  | gNB-gNB SBFD operator | 42 dB |
| Inter-network isolation values | BS-BS (adjacent channels) | 45 dB |
|  | UE-UE (adjacent channels) | 28 dB |
|  | BS-UE (adjacent channels) | 33 dB |
|  | UE-BS (adjacent channels) | 30 dB |
|  | Co-sited isolation | 45 dB |