3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204110

e-Meeting, 9th – 20th May, 2022

Agenda Item:	8.2.1

Source:	Ericsson

Title:	Initial Access Aspects

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss remaining details on CORESET#0 configuration.

# 2	CORESET#0 Configuration

In RAN1#108-e, the following working assumption was agreed:

Working Assumption - The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2. - FFS: whether/how to define X, if defined, candidate values {56, 76} - Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed - Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization. Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; X. 10; 1; 96; 2; 0. 11; 1; 96; 2; X. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB > 0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB > 0. 15; 3; 48; 2; 48.

The remaining issue is whether or not a 2nd offset value should be defined for a 96 RB CORESET#0, and if it is defined, what offset value should be supported (value of X rows 9 and 11 of the above table). The two candidates under discussion for X are 56 and 76 RBs. The first thing to observe is that there is room for a second offset value for the 96 RB CORESET#0 configuration.

There is room in Table 13-10A for a second offset value for the 96 RB CORESET#0 configuration.

As discussed before, what offsets that are necessary or useful depends on the channelization which is decided in RAN4. For licensed operation, RAN4 has agreed to define a floating channelization raster, with a sync raster granularity of 3x17.28 MHz for operation with 120 kHz SCS [1]. For unlicensed operation, it is agreed to have a fixed channelization raster with a stepsize of 1680 (100.8 MHz) as a baseline for 120 khz SCS. However, the exact ARFCN and GSCN raster points have not been agreed yet.

In [2], an extensive evaluation of what RB offset values that are needed was presented. For the case of 96 RB CORESET#0, the conclusion was that RB offset X = 56 is applicable and useful for both the agreed floating design as well as for an exemplary fixed design.

The candidate offset value X = 76 has the benefit of maximizing the number of RBs that can be used for SIB1 in case PDSCH carrying SIB1 is multiplexed in the same slot as an SSB/PBCH block. We note that the already agreed offset values of 0 and 28 enables the same opportunity for 24 and 48 RB CORESET#0 configurations, so it makes sense to adopt the same principle for 96 RB CORESET#0.

Based on this discussion, we propose the following:

Define a 2nd offset value for a 96 RB CORESET#0 and add to Table 13-10A in 38.213.

Define the 2nd offset value for a 96 RB CORESET#0 to be X = 76.

# Conclusion

In this paper we observed the following:

Observation 1	There is room in Table 13-10A for a second offset value for the 96 RB CORESET#0 configuration.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define a 2nd offset value for a 96 RB CORESET#0 and add to Table 13-10A in 38.213.

Proposal 2	Define the 2nd offset value for a 96 RB CORESET#0 to be X = 76.

# References

R4-2206433, “Email discussion summary for [102-e][133] NR_ext_to_71GHz_Part_1”, RAN4#102-e, February 2022.

R1-2201734, “Initial Access Aspects”, Ericsson, RAN1#108-e, February 2022.