3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204112

e-Meeting, 9th – 20th May, 2022

Agenda Item:	8.2.3

Source:	Ericsson

Title:	PDSCH/PUSCH enhancements

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we discuss the remaining issues on PDSCH/PUSCH scheduling enhancements and HARQ feedback aspects for multiple scheduling introduced in Rel-17.

# 2	PDSH/PUSCH Scheduling

## 2.1	Clarification on out-of-order determination

In RAN#108-e the following agreement was made in respect of Out-of-Order scheduling:

### Agreement: - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B). - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16 - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16 - Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

The remaining issue is whether the rule for OOO scheduling is determined based on configured SLIVs or valid SLIVs. It is observed that in most cases OOO determination based on “configured” or “valid” SLIVs gives the same result. Only in some very special scenarios it makes some difference, as the one depicted in Figure 1 below.

Figure 1: OOO Determination Example

In Figure 1 DCI_a schedules a first set of PDSCHs with the last PDSCH (PDSCH_a4) colliding with UL symbols; DCI_b schedules a second set of PDSCHs for which the first PDSCH (PDSCH_b1) is in the same slot as PDSCH_a4 but doesn’t collide with the UL symbols. In this special case, the two schedulings are considered as OOO for determination based on “configured” SLIVs while not considered as OOO for determination based on “valid” SLIVs. In our view, such special scheduling cases can be avoided by gNB implementation without compromising much of scheduling flexibility.

On the other hand, determination of OOO based on “configured” SLIVs bring a number of benefits. First of all, with OOO determination based on “configured” SLIVs, gNB and UE can determine OOO scheduling simply by checking the time spans of two multi-PXSCH schedulings. Further SLIV validation with semi-static TDD configuration is not required. In our view this is the simplest solution and should be adopted. Secondly, in the previous meetings agreements were made for several other issues related to clarification on “scheduled PXSCH” in previous agreements, such as CSI request, Type 2 HARQ-ACK codebook with time domain bundling, etc. Companies agreed that for the aforementioned situations “scheduled PXSCH” is based on “configured” SLIVs instead of “valid” SLIVs. From specification and implementation simplicity point of view it is preferable to follow a consistent principle. Finally, as pointed out by companies in the last meeting, determination of OOO based on valid SLIVs can bring additional complexity to OOO determination and Type-1 HARQ-ACK codebook generation when time domain bundling is enabled.

For multi-PDSCH/multi-PUSCH scheduling in Rel-17, out-of-order scheduling is determined based on configured SLIVs.

## 2.2	Clarification on NN-K1 time-line requirement

Current Layer 1 specifications provide the time-line requirement for multiplexing HARQ-ACK information in a PUCCH/PUSCH in a slot indicated by the PDSCH-to-HARQ feedback timing indicator field in a scheduling DCI, where the HARQ-ACK information is due to an earlier DCI that schedules a single PDSCH and provides a non-numeric K1 value (NNK1) in its PDSCH-to-HARQ feedback timing indicator field. For a DCI that schedules multiple PDSCHs in Rel-17 and provides NNK1, we think the evaluation of the time-line requirement for application of NN-K1 should be based on “configured” SLIVs for simplicity reasons.

For multi-PDSCH scheduling in Rel-17, the evaluation of the time-line requirement for application of NN-K1 is based on configured SLIVs.

## 2.3	Indication of SCell Dormancy

In RAN1#109-e, the following agreement was made:

Agreement

When a DCI format indicates TCI state update without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV.

During the meeting, it was discussed to whether this same rule should be extended to the case of a DCI format that indicates SCell Dormancy. The view from all but one company was that it should. Based on this majority view, we propose the following:

When a DCI format indicates SCell dormancy without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV.

# 3	HARQ feedback aspects

## 3.1	Type-1 HARQ codebook generation with time domain bundling when DCI 1_2 schedules PDSCH repetition

In RAN1#108-e, there was discussion on Type-1 HARQ-ACK codebook generation for the case when time domain bundling is configured for multi-PDSCH scheduled by DCI 1_1 and PDSCH repetition is simultaneously configured for PDSCH scheduled by DCI 1_2. Two options were discussed, one in which the HARQ-ACK codebook generation procedure is enhanced to handle this case, and a second option that would disallow the simultaneous configuration of time domain bundling for multi-PDSCH scheduled by DCI 1_1 and repetition for PDSCH scheduled by DCI 1_2.

We think restricting simultaneous configuration of time domain bundling for multi-PDSCH scheduled by DCI 1_1 and repetition for PDSCH scheduled by DCI 1_2 is unnecessary from configuration flexibility perspective. Furthermore, we propose the following TP to reduce the redundancy so that N_repetition doesn’t apply to DCI 1_1 when a TDRA table with at least one row indicating multiple SLIVs is configured for the DCI format. The TP is based on TP#G1 in section 3.3 in the FL summary from the last meeting ([1]), with the addition of the red text.

------------------------------------------Start of TP-1 for TS 38.213 Clause 9.1.2.1 ----------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL if row r of set R belongs to time domain resource allocation table that only includes single SLIV rows, or at least one symbol of the PDSCH time resource derived by row r in slot n0,k+nD is configured as UL if row r of set R belongs to time domain resource allocation table that includes at least one row indicating multiple SLIVs, where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

------------------------------------------------------------End of TP-1----------------------------------------------------------------

Simultaneous configuration of time domain bundling for multi-PDSCH scheduled by DCI 1_1 and repetition for PDSCH scheduled by DCI 1_2 is allowed.

Adopt TP-1 for Type-1 HARQ-ACK codebook in physical uplink control channel (TS 38.213, Section 9.1.2.1) for multi-PDSCH scheduling.

## 3.2	Feedback-disabled HARQ processes

The Rel-17 NTN WI has decided to introduce the feature of feedback-disabled HARQ processes in the DL. For such HARQ processes, UL HARQ feedback is disabled to avoid stop-and-wait in the HARQ procedure, thus relying on RLC ARQ for data transfer reliability. In RAN1#107-e the following agreement was reached on Type-1 HARQ-ACK codebook enhancement to support feedback-disabled HARQ processes ([2]), while Type-2 HARQ-ACK codebook enhancement is still being discussed in the Rel-17 NTN WI.

Agreement For Type-1 HARQ codebook, the UE will consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH. [Initial Proposal 1.2.2-1] For the DCI of PDSCH with feedback-disabled HARQ processes, one of following Options is supported: - Option-1: the C-DAI and T-DAI are the same of the C-DAI and T-DAI of the most recent DCI of PDSCH with feedback-enabled processes, despite they are not incremented. - For the codebook generation, the UE should use the DAI in DCI of feedback-disabled HARQ process to detect if a previous DCI of feedback-enabled HARQ processes has been missed. If so, the UE should chose the HARQ codebook size based on the DAI in DCI of the feedback-disabled HARQ process, and the feedback-enabled HARQ process detected to be missed should be NACK. - if all DCIs of PDSCH are associated with feedback disabled HARQ process, - Alt-1: The value of C-DAI and T-DAI set to VC-DAIDL=4 and VT-DAIDL=4 - Alt-2: (proponent are encouraged to provide the detailed solution if there is concern on Alt-1) Option-2: The C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation.

For NR operation in FR2-2 frequency bands, it is the common consensus that downlink and uplink data transmission may encounter HARQ process starvation problem due to larger sub-carrier spacing and short slot duration. To mitigate this issue, it has been agreed that the maximum number of HARQ processes is increased from 16 to 32, subject to UE capability. For UEs that do not support 32 HARQ processes, the HARQ process starvation might still be a problem for achieving higher data throughput, for which enabling feedback-disabled HARQ processes could be beneficial. Furthermore, enabling feedback-disabled HARQ processes can also be beneficial in the sense of off-loading UE from HARQ-ACK codebook construction and reporting and hence reducing UE implementation complexity. Therefore, it is beneficial to support feedback-disabled HARQ processes in FR2-2.

From specification and implementation perspectives, there doesn't seem to be a fundamental conflict between feedback-disabled HARQ processes and multi-PDSCH scheduling in FR2-2, with and without time domain bundling. As it can be seen from the following discussion in the section, it seems straight-forward to support both features with minimal specification effort.

It is beneficial and straight-forward to support feedback-disabled HARQ processes in FR2-2

For Type-1 HARQ-ACK codebook, if time domain HARQ bundling is not configured, according to the above-mentioned agreement from NTN, the UE should report NACK for the feedback-disabled HARQ processes regardless of the decoding results of the corresponding PDSCHs. Furthermore, for Type-2 HARQ-ACK codebook, if time domain bundling is not configured, for either Option 1 or Option 2 of the above NTN proposal, if only some of the PDSCHs scheduled by a DCI are feedback-disabled, the UE should report NACK for the feedback-disabled HARQ processes, regardless of the decoding results of the corresponding PDSCHs.

For Type-1 and Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if time domain HARQ bundling is not configured, the UE should report NACK for the feedback-disabled HARQ processes regardless of the decoding results of the corresponding PDSCHs.

For Type-1 and Type-2 HARQ-ACK codebook, if time domain HARQ bundling is configured, and if all PDSCHs in a bundling group are feedback-disabled, NACK should be reported for the bundling group. In case only some of the PDSCHs in a bundling group are feedback-disabled, generating NACK for the feedback-disabled PDSCHs will lead to NACK always being reported for the bundling group. Therefore, logical AND operation should be applied across all valid feedback-enabled PDSCHs to generate the HARQ-ACK bit. It is worth to mention that for Type-1 HARQ-ACK codebook, according to the previous agreement on time domain bundling, all PDSCHs scheduled by a DCI belong to a single bundling group.

For Type-1 and Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if time domain HARQ bundling is configured,

- For a group with only feedback-disabled PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK

- Logical AND operation is applied across all valid feedback-enabled PDSCHs within the same bundling group, if any, to generate 1 HARQ-ACK bit per group

- Note: For Type-1 HARQ-ACK codebook, all PDSCHs scheduled by a DCI belong to a single bundling group.

For Type-2 HARQ-ACK codebook where the HARQ-ACK codebook size is derived from the DAI counters in the last scheduling DCI, the DAI counters in a DCI that schedule multiple PDSCHs still need to be incremented if any of the PDSCHs scheduled by the DCI are not feedback-disabled.

For Type-2 HARQ-ACK codebook, the DAI counters in a DCI that schedule multiple PDSCHs still need to be incremented if any of the PDSCHs scheduled by the DCI are not feedback-disabled.

For Type-2 HARQ-ACK codebook if all PDSCHs scheduled by a DCI are feedback-disabled, HARQ-ACK feedback for the scheduling should be skipped, regardless of whether time domain bundling is configured or not.

For Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if all PDSCHs scheduled by a DCI are feedback-disabled, HARQ-ACK feedback for the scheduling should be skipped, regardless of whether time domain bundling is configured or not.

# Conclusion

In this paper we made the following observations:

Observation 1	It is beneficial and straight-forward to support feedback-disabled HARQ processes in FR2-2

In this paper we made the following proposals:

Proposal 1	For multi-PDSCH/multi-PUSCH scheduling in Rel-17, out-of-order scheduling is determined based on configured SLIVs.

Proposal 2	For multi-PDSCH scheduling in Rel-17, the evaluation of the time-line requirement for application of NN-K1 is based on configured SLIVs.

Proposal 3	When a DCI format indicates SCell dormancy without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV.

Proposal 4	Simultaneous configuration of time domain bundling for multi-PDSCH scheduled by DCI 1_1 and repetition for PDSCH scheduled by DCI 1_2 is allowed.

Proposal 5	Adopt TP-1 for Type-1 HARQ-ACK codebook in physical uplink control channel (TS 38.213, Section 9.1.2.1) for multi-PDSCH scheduling.

Proposal 6	For Type-1 and Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if time domain HARQ bundling is not configured, the UE should report NACK for the feedback-disabled HARQ processes regardless of the decoding results of the corresponding PDSCHs.

Proposal 7	For Type-1 and Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if time domain HARQ bundling is configured,

	For a group with only feedback-disabled PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK

	Logical AND operation is applied across all valid feedback-enabled PDSCHs within the same bundling group, if any, to generate 1 HARQ-ACK bit per group

Note: For Type-1 HARQ-ACK codebook, all PDSCHs scheduled by a DCI belong to a single bundling group.

Proposal 8	For Type-2 HARQ-ACK codebook, the DAI counters in a DCI that schedule multiple PDSCHs still need to be incremented if any of the PDSCHs scheduled by the DCI are not feedback-disabled.

Proposal 9	For Type-2 HARQ-ACK codebook generation for multi-PDSCH scheduling, if all PDSCHs scheduled by a DCI are feedback-disabled, HARQ-ACK feedback for the scheduling should be skipped, regardless of whether time domain bundling is configured or not.

# References

R1-2202796, “Summary #3 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, Moderator (LG Electronics), RAN1#108-e, February 2022.