3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204114

e-Meeting, 9th – 20th May, 2022

Agenda Item:	8.2.5

Source:	Ericsson

Title:	Remaining issue for beam management

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss a remaining issue for beam management.

# Discussion

In the prior meeting, the following agreement was made:

Agreement

TP#1 below is endorsed for TS 38.214

- RAN1 continues discussion to down-select between the following Alt1 and Alt2.

- Alt1) The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH

- Alt2) The applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH

- Note: coordination with Rel-17 MIMO WI is allowed as necessary

TP#1

5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted ***

Excerpt from 38.214, Clause 5.1.5

If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. For a single slot PDSCH, the indicated TCI state(s) should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state(s) should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

In this agreement, the behavior for multiple PDSCHs scheduled by a single DCI follows the same behavior as for a multi-slot PDSCH, i.e., PDSCH with slot aggregation/repetition. This agreement has been captured in the latest version of 38.214 [1], and the complete clause is as follows:

A key observation is that the indicated TCI state in the above excerpt from 38.214 is a Rel-15/16 TCI state configured by the higher layer parameter TCI-State as shown in the highlighted green text. Hence the agreed behavior highlighted in yellow for multi-slot PDSCH and multi-PDSCH scheduling applies only when a Rel-15/16 TCI states are configured, not when Rel-17 unified TCI states are configured. Furthermore, the UE capability for the threshold timeDurationForQCL in the above excerpt is applicable only for Rel-15/16 TCI states. If a TP is needed to further clarify that the above paragraph does not apply for Rel-17 TCI unified TCI states, then it can be further discussed within the Rel-17 MIMO WI.

Excerpt from 38.214, Clause 5.1.5

If a UE receives a higher layer configuration of a single DLorJoint-TCIState, that can be used as an indicated TCI state, the UE obtains the QCL assumptions from the configured TCI state for DM-RS of PDSCH and DM-RS of PDCCH, and the CSI -RS applying the indicated TCI state.

If a UE receives a higher layer configuration of a single DLorJoint-TCIState or UL-TCIState, that can be used as an indicated TCI state, the UE determines an UL TX spatial filter, if applicable, from the configured TCI state for dynamic-grant and configured-grant based PUSCH and PUCCH, and SRS applying the indicated TCI state.

When the UE would transmit the last symbol of a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduling by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated DLorJointTCIState or UL-TCIstate should be applied starting from the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH. The first slot and the BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication.

For the Rel-17 unified TCI framework, the TCI field in DCI has been repurposed to indicate Rel-17 TCI states, i.e., those configured by the Rel-17 higher layer parameter DLorJoint-TCIState. Furthermore, a new paradigm for beam updates has been introduced: the beam switch timing has been defined to occur at a configured time BeamAppTime_r17 after the PUCCH that carries the ACK corresponding to the DCI that provides the beam update. Care has been taken so that beam switch timing occurs at a slot border for all the CCs that are affected by the switch. This is captured in the current version of 38.214 [1] as follows:

In our understanding, there are no exceptions to this timing. In other words, if Rel-17 TCI states are configured (i.e., the parameter DLorJoint-TCIState is present) then a reading of the current version of 38.214 already allows a beam update to occur within the time span of a multiple PDSCHs scheduled by a single DCI, i.e., Alt-1 in the above agreement. This can be understood since (1) the first excerpt from 38.214 shown above is only applicable when TCI-State is configured (Rel-15/16 TCI states), and (2) the second excerpt above provides the timing for the case when DLorJoint-TCIState is configured (Rel-17 TCI states), and no exception is made for multi-slot PDSCH and multi-PDSCH scheduling.

In the Rel-17 unified TCI framework, current specifications already allow for DCI to provide an updated Rel-17 TCI state within the time span of a multi-slot PDSCH (and by extension, during the time span of multiple PDSCHs scheduled by a single DCI). If further clarification is needed, this can be discussed in the Rel-17 MIMO WI.

For multiple PDSCHs scheduled with a single DCI, support Alt-1 in the RAN1#108-e agreement. Specifically, if a DCI with or without a DL assignement indicates an updated Rel-17 unified TCI state, the updated TCI state can be applied during the time span of the multiple PDSCHs. Note: As per Observation 1, this is already captured in current specifications.

# Conclusion

In this paper we observed the following:

Observation 1	In the Rel-17 unified TCI framework, current specifications already allow for DCI to provide an updated Rel-17 TCI state within the time span of a multi-slot PDSCH (and by extension, during the time span of multiple PDSCHs scheduled by a single DCI). If further clarification is needed, this can be discussed in the Rel-17 MIMO WI.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For multiple PDSCHs scheduled with a single DCI, support Alt-1 in the RAN1#108-e agreement. Specifically, if a DCI with or without a DL assignement indicates an updated Rel-17 unified TCI state, the updated TCI state can be applied during the time span of the multiple PDSCHs. Note: As per Observation 1, this is already captured in current specifications.

# References

3GPP TS 38.214, "Physical layer procedures for data," v17.1.0, March 2022.