3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2204115

e-Meeting, 9th – 20th May, 2022

Agenda Item:	8.16.2

Source:	Ericsson

Title:	UE features for extending current NR operation to 71 GHz

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss open issues regarding the UE features list for the WI Extending current NR operation to 71 GHz [1].

# 2	Proposed Modifications to UE Feature Group List

## 2.1	FG 24-1b/1c/4b/4c/5c

In RAN#95-e the following agreement was made [2]:

Final Proposal 5 (agreed): replace the notes under FGs 24-1c, 24-4c and 24-5c for multi-RB PUCCH, and replace the bracketed notes under FGs 24-1b and 24-4b for wideband PRACH, with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”

Based on this we propose

Modify note in FG 24-1b, 24-1c, 24-4b, 24-4c, 24-5c as shown below to capture RAN#95-e agreement

In previous meetings, there was discussion on whether or not FG 24-1b (wideband PRACH) and FG 24-1c (multi-RB PUCCH) should be mandatory for a UE that supports standalone operation in FR2-2, i.e., a UE that supports FG 24-2. Additionally, it was discussed whether or not FG 24-1c should be mandatory for a UE that supports UL in FR2-2. In our view, these features should not be mandatory since not all deployment scenarios are coverage limited. We understand that for a standalone deployment, there is no mechanism to indicate UE capability for wideband PRACH/multi-RB during initial access; however, if a network indicates in SIB1 that either of these features should be used, and the UE does not support them, the UE simply cannot access the system.

However, even if the network indicates legacy PRACH (L = 139) and legacy PUCCH (single RB), it is still useful for the UE to indicate capability for FG 24-1b/c after initial access from the perspective that the network can capture statistics on UE support for these features. Once a significant fraction of the UE fleet supports wideband PRACH/multi-RB PUCCH, then the features can be activated. This can be useful for an operator to decide which features should be deployed and when in a network. Hence, in our view it still makes sense that the feature is defined as "Optional with capability signaling."

Modify FG 24-1b and FG 24-1c as follows such that these FGs are not mandatory for either standalone operation or if the UE supports UL.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Per band | [A UE that supports FG 24-2 must indicate this FG is supported] [Note: This FG is only supported in bands for shared spectrum operation] This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Per band | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Per band | [Note: This FG is only supported in bands for shared spectrum operation] This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |

## 2.2	FG 24-4/5

For FG 24-4 and 24-5, the open issue is to address the FFS on the mandatory monitoring capability for Group (2) search spaces (type 1 CSS w/o RRC and type 0/0A/2 CSS). On this issue, the following agreement was made in RAN1#107bis-e which defines the mandatory capability:

Agreement

Clarify earlier agreement as follows:

- A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that:

  - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.

- Continue discussion on whether or not introducing other limitation for Group (2) SSs in RAN1#108-e.

Regarding the last bullet of the above agreement, there was continued discussion on whether or not to introduce other limitation for Group (2) SSs. The following agreement was made (only latter part shown) which contains a working assumption for Group (2) SSs.

Agreement

…

- Introduce new parameter monitoringSlotsWithinSlotGroup-r17

  - Two sizes L are supported for this parameter: L=4 bits and L=8 bits

  - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a group of L slots

  - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17.

  - A slot in each group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in monitoringSlotsWithinSlotGroup-r17 is set to '1'

    - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot

  - For Group (1) SSs

    - The slots indicated in the bitmap should be consecutive per group of L slots

    - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE

  - Working assumption: For Group (2) SSs

    - For Type0/0A/2 CSS

      - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L

    - For Type1 CSS without dedicated RRC

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

The description of Component 7 in FG 24-4 and 24-5 is still in square brackets as follows:

[7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.]

While this text is consistent with the RAN1#107bis-e agreement shown above, it may require further modification based on the above working assumption from RAN1#108-e (which itself contains an FFS). Since Group (2) monitoring is still under discussion it is fair to say that Component 7 should still be left in square brackets until discussion on this issue is concluded.

For FG 24-4 and FG 24-5, leave the text description of Component 7 in square brackets until discussion in concluded on monitoring of Group (2) SSs.

## 2.3	FG 24-6/7

For FG 24-6 and FG 24-7, there is an FFS on Component 2 on the LBT bandwidth. However, the following agreement was made in RAN1#108-e:

Agreement

For LBT for single carrier UL transmission, UE performs LBT over a BW that at least includes the active UL BWP bandwidth

- The BW that at least includes the active UL BWP bandwidth is captured as “channel” in 37.213

Based on this, it seems the square brackets can be resolved by defining LBT per channel as defined in 37.213 Clause 4.4:

[37.213 Clause 4.4]

When a gNB/UE senses a channel for availability to perform DL/UL transmission(s), the channel for sensing includes at least the corresponding active DL/UL bandwidth part(s) for the DL/UL transmission(s).

For FG 24-6 and FG 24-7, revise the description of Component 2 to capture the RAN1#108-e agreement on LBT bandwidth.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure [2. Support LBT performed per channel, as defined in 37.213 Clause 4.4 carrier/BWP bandwidth] | 24-1a | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure [2. Support LBT performed per channel, as defined in 37.213 Clause 4.4 carrier/BWP bandwidth] | 24-1a, 24-6 | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |

## 2.4	FG 24-8/9

For FG 24-8 and FG 24-9,  there is an FFS on the granularity of the capability signaling. In RAN1#108-e there was discussion on whether the signaling should be "per band" or "per FSPC." Our preference is "per band" consistent with the other FGs defined for NR up to 71 GHz. We have concerns on "per FSPC," since it leads to complicated UE capability processing at the gNB which cannot be underestimated. Capability processing is already a major task in the gNB considering the large number of capabilities that have been defined over 3 releases of 5G. Signaling capabilities per band at least keeps the processing in check. If capabilities are signaled per CC per band combination, the capability processing demand becomes too large. Furthermore, it complicates management of multiple carriers in a CA combination. The gNB would need to account for different capabilities on a per CC level across the whole fleet of UEs. For example, one UE could indicate support for four carriers but support 32 HARQ processes on two CCs, while another may indicate three carriers and support only 32 HARQ processes on one of the three CCs and so on. This creates too much complexity at the gNB side for managing these different permutations.

One argument that was used in the last meeting against per band capability signaling was that if a UE supports 32 HARQ processes in Band A and 16 in each of Bands B and C, then per band signaling would require the UE to support 32 + 16 + 16 = 64 HARQ processes for the band combination (A,B,C). It was stated that if this exhausts the UE's memory resources, then the UE would need to declare that it cannot support band combination (A,B,C) if it declares support for 32 HARQ processes for Band A. However, this argument does not seem to provide any motivation for why the capability signaling would need to be per CC within a band combination. Moreover, with per-band capability signaling, the UE can utilize other existing capabilities (e.g. related to peak rate signaling such as MIMO layers, BW, etc) to indicate a suitable peak rate that it can support for the band combination (A,B,C) with 32 HARQ processes indicated for band A without exhausting the UE’s memory resources. Finally, it is noted that UE soft buffer handling in NR is left to UE implementation providing significant freedom in managing memory resources.

Support "per band" capability signaling for FG 24-8 and 24-9.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | FFS Per band | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | FFS Per band | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |

# 3	Other UE capability aspects

## 3.1	New UE Capabilities for Multi-Slot PDCCH Monitoring for 480/960 kHz SCS for Carrier Aggregation

In RAN1#108-e the following agreement was made regarding UE capabilities for per-slot group PDCCH monitoring (also referred to as multi-slot monitoring):

Agreement

- For serving cells configured with 480 or 960 kHz SCS, the serving cells with the same SCS and Xs value are grouped together to determine a total BD/CCE budget for that group and the per-cell BD/CCE budget within the group.

- Support UE capability signaling for 4 additional cases :

  - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

    - Range of pdcch-BlindDetectionCA-R17: {[4, 5, …, , 16]}

  - Case 5: Capability on the number of CCs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells

    - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

    - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]}

      - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]}

  - Case 6: Capability on the number of CCs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

    - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

    - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: {[1, 2, …, 15]}

      - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[3, 4, …, 16]}

  - Case 7: Capability on the number of CCs with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

    - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

    - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]}

      - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17 : {[4, 5, …, 16]}

- For the case with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells (case 7) or any combination of 2 of the capabilities (i.e. case 5, and case 6), the UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) as UE capability. If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17), as in Rel-16, the gNB configures which combination for the UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability.

- FFS: Extension to NR-DC scenario

As shown in this agreement, UE capability signaling is agreed for 4 additional cases, hence new UE feature groups need to be defined to cover these cases. There is ongoing discussion in AI 8.2.2 on appropriate candidate values for the corresponding capability parameters, and once that discussion is concluded, the agreed values can be included into the definition of the new feature groups.

Define new FGs as shown below to cover multi-slot PDCCH monitoring for Cases 4, 5, 6, and 7 in the RAN1#108-e agreement for carrier aggregation.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24-11 | Number of CCs for BD/CCE scaling for DL CA with Rel-17 PDCCH monitoring capability on all the serving cells | Supported value(s) of pdcch-BlindDetectionCAr17 for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with Rel-17 PDCCH monitoring capability (per group of Xs slots) on all the serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of pdcch-BlindDetectionCAr17: [4,5, …,16] | Optional with capability signalling |
| 24-12 | Number of CCs for BD/CCE scaling for DL CA with mix of Rel-17 and Rel-15 PDCCH monitoring capability on different serving cells | Supported combinations(s) of {pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr15} for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with mix of Rel-17 monitoring capability (per group of Xs slots) and Rel-15 PDCCH monitoring capability (per slot) on different serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of the summation of pdcch-BlindDetectionCAr17 and pdcch-BlindDetectionCAr15: {4,5, …,16} Candidate values of pdcch-BlindDetectionCAr17: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr15: {1,2, …,15} | Optional with capability signalling |
| 24-13 | Number of CCs for BD/CCE scaling for DL CA with mix of Rel-17 and Rel-16 PDCCH monitoring capability on different serving cells | Supported combinations(s) of {pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr16} for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with mix of Rel-17 monitoring capability (per group of Xs slots) and Rel-16 PDCCH monitoring capability (per span) on different serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of the summation of pdcch-BlindDetectionCAr17 and pdcch-BlindDetectionCAr16: {4,5, …,16} Candidate values of pdcch-BlindDetectionCAr17: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr16: {1,2, …,15} | Optional with capability signalling |
| 24-14 | Number of CCs for BD/CCE scaling for DL CA with mix of Rel-17, Rel-16, and Rel-15 PDCCH monitoring capability on different serving cells | Supported combinations(s) of {pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr16, pdcch-BlindDetectionrCA15} for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with mix of Rel-17 monitoring capability (per group of Xs slots), Rel-16 PDCCH monitoring capability (per span), and Rel-15 PDCCH monitoring (per slot) on different serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of the summation of pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr16, and pdcch-BlindDetectionCAr17: {4,5, …,16} Candidate values of pdcch-BlindDetectionCAr17: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr16: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr15: {1,2, …,15} | Optional with capability signalling |

## 3.2	New UE Capabilities for Multi-Slot PDCCH Monitoring for 480/960 kHz SCS for Dual Connectivity

In the RAN1#108-e agreement shown in the previous section, there is an FFS on extension of the UE capability definitions for carrier aggregation to the NR-DC scenario. Since any new FG has ASN.1 impact, it would be wise to define "place holder" FGs for NR-DC such that RAN2 can at least define the signaling. RAN1 can then continue to work on the description of the new FGs. Further discussion is needed in RAN1#109-e on how many new FGs are required to cover possible combinations of Rel-15 (per-slot), Rel-16 (per-span), and Rel-17 (per-slot group) PDCCH monitoring.

Define new "place holder" FGs to cover multi-slot PDCCH monitoring for for the NR-DC scenario.

## 3.3	UE Capability on SCS Values for Cross-Carrier Scheduling

In RAN1#107bis-e, the following conclusion was reached:

Conclusion

Potential indications of UE capability related to a limited support of cross-carrier scheduling e.g. as a function of |μPDCCH − μPDSCH| can be discussed as part of the UE capability discussion.

We do not support addition of such a UE capability. RAN4 has defined inter-band carrier aggregation combinations between FR1 and FR2-2, and given that FR1 bands are defined with SCS as low as 15 kHz and that FR2-2 bands are defined with SCS up to 960 kHz, we don't think that additional SCS restrictions should not be introduced if the UE supports such a band combination. This would mean that the SCS difference can be as large as |μPDCCH − μPDSCH| = 6.

Do not introduce a UE capability on the supported value(s) of the SCS difference |μPDCCH − μPDSCH| for cross-carrier scheduling.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Modify note in FG 24-1b, 24-1c, 24-4b, 24-4c, 24-5c as shown below to capture RAN#95-e agreement

Proposal 2	Modify FG 24-1b and FG 24-1c as follows such that these FGs are not mandatory for either standalone operation or if the UE supports UL.

Proposal 3	For FG 24-4 and FG 24-5, leave the text description of Component 7 in square brackets until discussion in concluded on monitoring of Group (2) SSs.

Proposal 4	For FG 24-6 and FG 24-7, revise the description of Component 2 to capture the RAN1#108-e agreement on LBT bandwidth.

Proposal 5	Support "per band" capability signaling for FG 24-8 and 24-9.

Proposal 6	Define new FGs as shown below to cover multi-slot PDCCH monitoring for Cases 4, 5, 6, and 7 in the RAN1#108-e agreement for carrier aggregation.

Proposal 7	Define new "place holder" FGs to cover multi-slot PDCCH monitoring for for the NR-DC scenario.

Proposal 8	Do not introduce a UE capability on the supported value(s) of the SCS difference |μPDCCH − μPDSCH| for cross-carrier scheduling.

# References

R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e　including remaining RAN1 issues” Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108-e, February 2022.

RP-220951, "Moderator's conclusion for discussion [95e-29-R17-52.6-71GHz-WI]," RAN1 Vice-Chair (Huawei), RAN#95-e, March 2022.

# Appendix A – Consolidated Changes

In this appendix, the FG modifications proposed in the previous sections of this paper are consolidated into one table.

| Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  | per band | A UE that supports FR2-2 must indicate this FG is supported | Optional with capability signalling |
| 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | 24-1 | per band |  | Optional with capability signalling |
| 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Per band | [A UE that supports FG 24-2 must indicate this FG is supported] [Note: This FG is only supported in bands for shared spectrum operation] This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Per band | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | 24-1 | Per band |  | Optional with capability signalling |
| 24-1f | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI |  | Per band |  | Optional with capability signalling |
| 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Per band |  | Optional with capability signalling |
| 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | Per band |  | Optional with capability signalling |
| 24-2 | 120KHz SSB support for initial access in FR2-2 | 1. Support 120KHz SSB for initial access in FR2-2 | 24-1, 24-1a | per band |  | Optional with capability signalling |
| 24-3 | 480KHz SSB support for initial access in FR2-2 | 1. Support 480KHz SSB for initial in FR2-2 | 24-2, 24-4, 24-4a | per band |  | Optional with capability signalling |
| 24-4 | 480KHz SCS support for DL | 1. 480KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (4, 3) and (7, 3) are supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Perband |  | Optional with capability signalling |
| 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | Per band |  | Optional with capability signalling |
| 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Per band | [Note: This FG is only supported in bands for shared spectrum operation] This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=(4,2) 2.) Within each of the Ys = 2 slots (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot | 24-4 | Per band |  | Optional with capability signalling |
| 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=8), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y)= (7, 3) is supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Per band |  | Optional with capability signalling |
| 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1a, 24-5 | Per band |  | Optional with capability signalling |
| 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
| 24-5f | Enhanced PDCCH monitoring for 960KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys) 2.) Within each of the Ys = 2 (with Xs=4) or Ys = 4 (with Xs=8) slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot or within the Ys = 1 (with Xs=4) slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (7, 3) | 24-5 | Per band | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} | Optional with capability signalling |
| 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure [2. Support LBT performed over a bandwidth that at least includes the active UL BWP bandwidth per carrier/BWP bandwidth] | 24-1a | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure [2. Support LBT performed over a bandwidth that at least includes the active UL BWP bandwidth per carrier/BWP bandwidth] | 24-1a, 24-6 | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | FFS Per band | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | FFS Per band | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| 24-10 | Reduced beam switching time delay | Support of reduced beam switching time delay d = 56 symbols for 480 kHz SCS |  | per band | If this capability is not reported and the UE supports both FG 24-4 and 24-5, the default value of 112 symbols is assumed | Optional with capability signalling |
| 24-11 | Number of CCs for BD/CCE scaling for DL CA with Rel-17 PDCCH monitoring capability on all the serving cells | Supported value(s) of pdcch-BlindDetectionCAr17 for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with Rel-17 PDCCH monitoring capability (per group of Xs slots) on all the serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of pdcch-BlindDetectionCAr17: [4,5, …,16] | Optional with capability signalling |
| 24-12 | Number of CCs for BD/CCE scaling for DL CA with mix of Rel-17 and Rel-15 PDCCH monitoring capability on different serving cells | Supported combinations(s) of {pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr15} for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with mix of Rel-17 monitoring capability (per group of Xs slots) and Rel-15 PDCCH monitoring capability (per slot) on different serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of the summation of pdcch-BlindDetectionCAr17 and pdcch-BlindDetectionCAr15: {4,5, …,16} Candidate values of pdcch-BlindDetectionCAr17: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr15: {1,2, …,15} | Optional with capability signalling |
| 24-13 | Number of CCs for BD/CCE scaling for DL CA with mix of Rel-17 and Rel-16 PDCCH monitoring capability on different serving cells | Supported combinations(s) of {pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr16} for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with mix of Rel-17 monitoring capability (per group of Xs slots) and Rel-16 PDCCH monitoring capability (per span) on different serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of the summation of pdcch-BlindDetectionCAr17 and pdcch-BlindDetectionCAr16: {4,5, …,16} Candidate values of pdcch-BlindDetectionCAr17: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr16: {1,2, …,15} | Optional with capability signalling |
| 24-14 | Number of CCs for BD/CCE scaling for DL CA with mix of Rel-17, Rel-16, and Rel-15 PDCCH monitoring capability on different serving cells | Supported combinations(s) of {pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr16, pdcch-BlindDetectionrCA15} for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with mix of Rel-17 monitoring capability (per group of Xs slots), Rel-16 PDCCH monitoring capability (per span), and Rel-15 PDCCH monitoring (per slot) on different serving cells. | 24-4 and/or 24-5 | Per band | Candidate values of the summation of pdcch-BlindDetectionCAr17, pdcch-BlindDetectionCAr16, and pdcch-BlindDetectionCAr17: {4,5, …,16} Candidate values of pdcch-BlindDetectionCAr17: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr16: {1,2, …,15} Candidate values of pdcch-BlindDetectionCAr15: {1,2, …,15} | Optional with capability signalling |