3GPP TSG RAN WG1 #109-e			R1-2204121

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.4.1.2

Source: 	Hyundai Motors

Title:	Discussion on physical channel design framework for sidelink on unlicensed spectrum

Document for:	Discussion and Decision

Introduction

In the Rel-18 WID for NR sidelink evolution [1], an objective to study and specify support of sidelink on unlicensed spectrum is as follows:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we discuss physical channel design frame for supporting of sidelink on unlicensed spectrum.

Discussion

According to WID for NR sidelink evolution, it is necessary to support of sidelink on unlicensed spectrum for both mode 1 and mode 2. For this support, sidelink physical channel structure completed in Rel-16 shall be optimized for sidelink unlicensed operation. In addition, the existing NR-U channel structure and access mechanisms shall be a starting point of SL-U. In NR-U, channel access in both downlink and uplink rely on the listen-before-talk (LBT) feature. UE (or gNB) must apply a Clear Channel Assessment (CCA) check for transmission. CCA relies on Energy Detection (ED) to detect the presence or absence of other signals of the communications channel. If the detecting energy during the CCA duration is below a certain threshold, UE determines when and where to transmit and receive based on an indication of a channel occupancy time (COT) structure.

In sidelink system, UE(s) also needs the LBT feature, CCA, and COT structure to operate on unlicensed spectrum. Similar to NR-U, the COT structure in SL can be set until an UE(s) receives data (or HARQ feedback or CSI report) from other UE(s) (Fig. 1). Alternatively, the COT structure can be set whenever each UE(s) transmits data (Fig. 2). For example, for unicast, COT structure can be set until UE(s) receives data from other UE(s). Alternatively, for groupcast or broadcast, COT structure can be set whenever each UE(s) transmission.

Figure 1.

Figure 2.

Proposal 1: In SL-U, the COT structure can be set (1) until an UE(s) receives data (or HARQ feedback or CSI report) from other UE(s) (2) or whenever each UE(s) transmits data.

In SL, if CCA operation is performed in an arbitrary position of slot N of an UE, the UE can transmit data in slot N+1 (Fig. 3). In this case, in order to minimize or remove the remaining resources between CCA operation position (of slot N) and AGC symbol (of slot N+1), it is necessary to limit the CCA operation position. For example, CCA operation can be performed in last symbol of slot (e.g., gap symbol) (Fig. 4). In this case, in SL-U, the CCA parameters or COT structure should be set in slot or symbol units instead of time units.

Figure 3.

Figure 4.

Proposal 2: In SL-U, CCA operation can be performed in last symbol of slot (e.g., gap symbol).

In SL-U, the channel can be used for the unlicensed spectrum when a channel signal energy measurement in duration of CCA is below a threshold. In this case, the duration of CCA can be set according to the priority of the message to be transmitted. For example, the highest priority message may have the longest duration (slot N of Fig. 5). Similarly, the period of CCA can also be set according to the priority. For example, lower priority message may have CCA operation in every 2 slots and the lowest priority message may have CCA operation in every 4 slots (Fig. 6). These CCA parameters (such as duration, period, etc.) can be provided by the 1st SCI or 2nd SCI.

Figure 5.

Figure 6.

Proposal 3:  The LBT parameters (such as duration, period, etc.) can be provided by SCI.

Conclusion

In this contribution, the following conclusions were made:

Proposal 1: In SL-U, the COT structure can be set (1) until an UE(s) receives data (or HARQ feedback or CSI report) from other UE(s) (2) or whenever each UE(s) transmits data.

Proposal 2: In SL-U, CCA operation can be performed in last symbol of slot (e.g., gap symbol).

Proposal 3:  The LBT parameters (such as duration, period, etc.) can be provided by SCI.

Reference

[1] RP-220300, “WID revision: NR sidelink evolution,” OPPO.