3GPP TSG RAN WG1 Meeting#109-e		R1-2204122

e-Meeting, May 9th – May 20th , 2022

Agenda item:	9.3.1

Source:	KT Corp.

Title:	Discussion on deployment scenario of NR duplex evolution

Document for:	Discussion and Decision

# Introduction

The study item “Evolution of NR duplex operation” [1] was agreed in RAN#94-e to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this contribution, some applicable and relevant deployment scenarios are discussed with some proposals suggested for approval.

# Possible deployment scenario of NR duplex evolution

In the 3GPP 5G-Advanced workshop held during RAN#92e, number of companies have shown possible deployment scenario with initial evaluation results [2-5]. From gNB side it is clear that following interferences need to be considered:

- Self-Interference from intra-gNB

- Cross Link Interference from inter-gNBs from the same operator

- Cross Link Interference from inter-gNBs from the other operators

To handle Self-interference, some known self-interference cancellation techniques such as directional SI suppression, antenna separation and SI cancellation, analogue cancellation, digital cancellation, etc. should be taken into account [6]. Using separate panel for Tx/Rx can help reducing SI where physical separation brings natural isolation which may provide enough suppression at gNB antenna front-end.

Observation 1: Using separate panel for Tx/Rx can help reducing Self-Interference

In Rel-16, feasibility study on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR was done following normative works. Study results showed that for BS to BS interference, no performance degradation was observed in Macro-to-Indoor and Indoor-to-macro scenario [7]. It is also shown that when power control is operated, no performance degradation was observed for Indoor-to-indoor scenario. Based on this results, it is suggested to prioritize specification work of NR duplex evolution based on indoor small cell gNB.

Observation 2: No performance degradation was observed for Macro-to-Indoor and Indoor-to-Macro scenario

Based on the above two observations, KT proposes to consider prioritizing indoor small cell based gNB using separate Tx/Rx panel to support sub-band non-overlapping full duplex. As indoor small cell based gNB is less likely to suffer from inter-gNB CLI from the macro cell, KT believes that indoor small cell based gNB will help to realize the actual deployment of sub-band non-overlapping full duplex gNBs.

Proposal 1: For the deployment scenario, prioritize indoor small cell based gNB for sub-band full duplex gNB

Proposal 2: Consider using separate Tx/Rx panel for the default design of sub-band full duplex gNB

# Conclusion

Based on the discussion in the above section, the following proposal is suggested for approval.

Proposal 1: For the deployment scenario, prioritize indoor small cell based gNB for sub-band full duplex gNB

Proposal 2: Consider using separate Tx/Rx panel for the default design of sub-band full duplex gNB

# Reference

- RP-220633, Revised SID: Study on evolution of NR duplex operation, RAN#95e

- RWS-210026, On NR Full Duplex, Qualcomm, 3GPP RAN Rel-18 Workshop

- RWS-210144, Draft SID on duplex enhancement for NR, China Telecom, 3GPP RAN Rel-18 Workshop

- RWS-210180, XDD for enhanced duplexing operation in 5G Advanced, Samsung, 3GPP RAN Rel-18 Workshop

- RWS-210241, Motivation for new SI: Study on full duplex operation for NR, LG Electronics, 3GPP RAN Rel-18 Workshop

- Full Duplex Techniques for 5G Networks: Self-Interference Cancellation, Protocol Design, and Relay Selection, Z. Zhang, X. Chai, K. Long, A.V. Vasilakos, L. Hanjo. IEEE Communications Magazine. May 2015

- 3GPP TR 38.828 V16.1.0