3GPP TSG RAN WG1 #109-e		                                    		   R1-2204124

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.11.2

Source:	InterDigital, Inc.

Title:	Discussion on XR specific capacity enhancements

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, the contents of the SID for Rel. 18 NR XR Enhancements were agreed [1]. The objectives in the SID for XR-specific capacity improvements are as follows.

Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution, we discuss the potential enhancements and evaluations for capacity improvements for XR applications/services.

# Discussion

During Rel-17 SI [2], the traffic models for different XR applications (AR, VR and cloud gaming) were defined and the performance evaluations for capacity in DL and UL were performed.

From the discussions on traffic models, the XR traffic in DL is characterized by high and variable PDU sizes (e.g. corresponding to I-frames, P/B frames), high number of PDUs per frame, quasi-periodic periodicity (e.g. corresponding to video/media frame rate and jitter), and multiple flows/streams per application. In the case of UL, the XR traffic is characterized by high periodicity (e.g. for pose/control data), high data rate (e.g. for AR video), and multiple streams per application (e.g. video, audio). The XR traffic in both DL and UL requires satisfying stringent QoS in terms of high data rate (e.g. 45Mbps), low latency (e.g. 10ms PDB) and high reliability (e.g. 99% success rate) for ensuring user satisfaction.

From the evaluations performed during Rel-17 [2], achieving high system capacity with XR traffic in UL and DL in different deployment scenarios (e.g. InH, DU, UMa) is extremely challenging. The KPI related to UE satisfaction considered in the Rel-17 evaluations is as follows [2]:

A UE is declared as a satisfied UE if all the considered streams meet their own PER and PDB requirements, i.e., more than a certain percentage of packets are successfully transmitted within a given air interface PDB.

Given the UE satisfaction KPI, the system capacity is defined as the maximum number of users per cell with at least Y % of UEs being satisfied, where Y=90 (baseline) or 95 (optional).

Observation 1: 	Achieving high system capacity with XR traffic in UL and DL in different deployment scenarios (e.g. InH, DU, UMa) is extremely challenging

The scope of Rel-18 SI discussed during RAN#94-e includes the objectives for capacity improvement based on SPS/CG enhancements and dynamic grant (DG) enhancements. However, it is unclear how certain XR-specific traffic characteristics can be accounted in the enhancements for SPS/CG and DG and what is the impact of accounting such traffic characteristics in the enhancements on the capacity improvements. The key traffic characteristics of XR and the corresponding impacts that should be further studied when considering enhancements for capacity are:

- Consideration of PDU sets/frames

- Consideration of multiple streams

Impact of handling of PDU sets/frames

A PDU set can consist of several PDUs that can be associated with a video/media frame. The video encoder at the application server or UE may generate different types of video frames (e.g. I-frame, P/B-frame) at a particular frame rate (e.g. 60fps, 90fps). The video frames generated at a frame rate may consist of variable payload sizes. Effectively, the number of PDUs per frame or size of a PDU set can vary depending on the type of video frame generated.

Given the interdependencies between the PDUs, all PDUs in a PDU set need to be successfully delivered within a PDU set-level latency requirement for determining whether the transmission is successful. If at least one of the PDUs in a PDU set belonging to a critical video frame is not delivered successfully in time (e.g. due to jitter or losses), the user may perceive QoE degradation.

Observation 2: 	Given the interdependencies between the PDUs, all PDUs in a PDU set need to be successfully delivered within a PDU set-level latency requirement for counting towards capacity

When considering scheduling enhancements for improving capacity, how the PDU set can be accommodated in the enhancements for SPS/CG and DG should be studied. This can include consideration of variable size of PDU set (e.g. variable number of PDUs per PDU set), importance of PDU set/frame, and different arrival times of PDUs in PDU set (e.g. due to jitter). For example, it may be beneficial to study how the grant sizes, start time offset, and periodicity for SPS/CG may be configured or adapted based on the PDU set characteristics. Also, how the PDU sets generated at non-integer periodicities (e.g. corresponding to video frame rates) can be accommodated with SPS/CG resources configured with integer periodicities should also be investigated. For DG resources, how to trigger SR/BSR and perform grant adjustments based on PDU set characteristics that would result in flexible, efficient and low latency resource allocation should be studied.

Proposal 1: 	Capture in TR, XR-specific enhancements for capacity that support adaptations based on PDU set characteristics in UL and DL

For the different scheduling enhancements, the impact of handling PDU sets on the capacity improvements achievable should be studied and evaluated. The KPI used for determining UE satisfaction should also be adapted when considering the transmission of PDUs in a PDU set. For example, the latency metric considered for PDU set can be PDU set delay bound (PSDB) and for reliability the metric can be PDU set error rate (PSER).

Impact of handling of multiple streams

During Rel-17 SI [2], performance evaluations for XR applications with multi-streams indicate significant drop in capacity in both DL and UL compared to applications with single stream. This is due to the increase in total traffic load per UE and the need for supporting traffic with diverse characteristics (e.g. high periodicity for pose/control data and high PDU sizes for video data for UL AR).

For XR applications with multiple streams, the application layer may require the PDUs in different streams to be received within a synchronization time window to jointly perform processing and rendering. Such time window can induce some interdependency between the streams. However, since the PDUs in different streams may be transported independently, it is possible that inter-stream jitter may be introduced where the PDUs may arrive at the UE (in DL) or application server (in UL) with different latencies.

To address this issue, in addition to the per-stream QoS (e.g. PER, PDB) that each stream is required to fulfil independently, certain coordination is needed between the streams to ensure that the PDUs in different streams arrive within a maximum inter-stream jitter value to be counted towards capacity. For example, the maximum inter-stream jitter can be represented as a joint QoS requirement corresponding to the synchronization time window that different streams must fulfil jointly during transmission.

Observation 3: 	In multi-stream scenario, the PDUs in different traffic streams need to be received by UE (in DL) or application server (in UL) within a maximum inter-stream jitter value to be counted towards capacity

Regarding enhancements for capacity, whether and how inter-stream coordination can be supported via scheduling for meeting the joint QoS should be studied. For example, it is important to investigate how to enable configuring of SPS and CG resources by aligning the grant sizes, start time offsets and periodicities with the traffic in multiple streams. Similarly, DG enhancements that results in minimizing the inter-stream jitter based on prioritization of PDUs in different associated/correlated streams should be studied.

Proposal 2: 	Capture in TR, XR-specific enhancements for capacity that support transmission of multiple associated/correlated traffic streams in UL and DL

The impact of handling multiple associated streams on capacity should also be studied and evaluated. For multiple streams, a metric related to maximum inter-stream jitter (e.g. maximum difference in latency between arrival of PDUs in different associated streams) can be considered in the evaluations. This metric can be used as the KPI for determining UE satisfaction and capacity in UL and/or DL in multi-stream scenarios.

Performance evaluations for XR-specific capacity improvements

For any of the resource allocation enhancements (e.g. SPS, CG, and DG enhancements), it would be beneficial to investigate how the capacity performance compares with respect to baseline schemes. For example, for studying the impact of handling PDU sets, the capacity performance achievable with an enhancement scheme should be compared with a baseline scheme that is agnostic to handling PDU sets during resource allocation. Similarly, for determining the capacity gains achievable with an enhancement scheme that can handle multiple streams, the capacity performance should be compared with a baseline scheme that is agnostic to handling association of multiple streams.

Proposal 3: 	RAN1 to perform evaluations of XR-specific resource allocation and scheduling enhancement techniques for evaluating the capacity performance

Given that the key system parameters were discussed during Rel-17 SI, we propose to reuse the baseline evaluation assumptions (e.g. deployment scenarios, SLS parameters, traffic models) used in TR 38.838 as the starting point for the SLS performance evaluations for capacity. The SLS parameters and baseline traffic model parameters are provided in Section 5 (Annex).

Proposal 4: 	Reuse the baseline evaluation assumptions (e.g. deployment scenarios, SLS parameters, traffic models) in TR 38.838 when evaluating capacity performance

During Rel-17 SI evaluations, resource allocation on a PDU set or per-frame basis was considered in some of the enhancement schemes, such as delay-aware scheduling (e.g. scheduling based remaining delivery time). Other schemes that may support handling of PDU sets includes multi-TB scheduling and enhanced BSR. Similarly, enhancement schemes such as prioritizing/pre-empting important streams in UL/DL may handle transmission of PDUs in multiple streams. Such enhancement schemes that can leverage on the SPS, CG and DG enhancements can be considered as a starting point for Rel-18 enhancements and evaluations for capacity improvements.

Proposal 5: 	Starting point for potential enhancement techniques for capacity can be those identified during Rel-17 SI (e.g. delay aware scheduling, prioritizing/pre-empting important streams in DL/UL, multi-TB scheduling, enhanced BSR)

# Simulation Results

In this section, we provide system level simulation results on capacity performance for single and multi-stream traffic models in UL and DL. The SLS are performed using baseline evaluation assumptions and parameters in TR 38.383 (provided in Section 5).

3.1 Uplink Capacity Results

The uplink system capacity evaluation for the single stream and multiple-stream traffic models are presented in this section. System capacity in UL is defined as the maximum number of supported UEs out of which at least Y% of UEs are satisfied (e.g. PDB and PER requirements are met for all UL traffic streams), where Y=90 (baseline) is used in the simulations.

### UL Capacity for single stream transmission

#### Indoor Hotspot

UL system capacity for Indoor hotspot (parameters defined in Table 1 and Table 3 in Annex) is evaluated for FR1 (4 GHz) using SU-MIMO with PF scheduling. A UE is declared satisfied if more than 99% of packets are successfully transmitted within PDB values of 30ms and 10ms for AR and CG/VR applications, respectively.

Figure 1: FR1 UL AR results for Indoor Hotspot scenario

Figure 2: FR1 UL CG/VR results for Indoor Hotspot scenario

Figures 1 and 2 show the UL results for system capacity for AR and CG/VR respectively in an InH deployment where a single stream is simulated. It is observed that a large number of UEs/cell (>20) can be supported when only pose/control information is sent in the UL for the CG/VR case. In the AR case, where aggregated video is sent in the UL, the capacity experiences a sharp drop as the number of UEs exceed ~10 UEs/cell. The maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to < 12 UEs/cell. Resource utilization (%) also follows a steeper increase as the number of UEs/cell increases when sending aggregated video in the UL vs only pose/control information.

Dense Urban

Figures 3 and 4 show the results for system capacity for AR and CG/VR, respectively in a dense urban scenario (with parameters as defined in Tables 1 and 3 in Annex) and all other evaluation assumptions kept the same (i.e., PDB values of 30ms for AR application, 10ms for CG/VR). For the AR traffic we utilize 3 scheduling approaches as this is the bottleneck traffic case. The scheduling approaches are PF, FIFO and resource sharing. In FIFO based scheduling, the gNB prioritizes UEs for scheduling based on the arrival times and hence, delay. The packet (and hence UE) with earlier packet arrival will be allocated all resources for that slot. In case of multiple streams, the arrival rate of stream with more stringent PDB is used. For e.g., for UL multi-stream, CG/AR arrivals are used for FIFO. In the resource sharing based scheduling, the gNB ensures that all packets (and hence UEs) are allocated equal resources at every scheduling instant. This guarantees all UEs some resources.

The results for CG/VR shown in Figure 4, we utilized only PF scheduling. We observe that for pose/control traffic, up to 10 UEs can be supported, whereas for uplink traffic of AR 10 Mbps this drops to 3 UEs per cell. In in UL AR, FIFO based scheduling slightly outperforms PF at low load, while PF seems to yield best performance as load increases. Additionally for AR, a number of factors including the high data rate (10Mbps), reduced number of UL slots in a radio frame (DDDSU), and high load (number of UEs) can create a high degree on congestion in the system, thereby limiting any gains that may be achieved from alternate scheduling strategies such as FIFO and resource sharing.

Figure 3: FR1 UL AR results for Dense Urban scenario

Figure 4: FR1 UL CG/VR results for Dense Urban scenario

Observation 4: 	Transitioning from InH to DU deployment scenario has a significant impact on the capacity as shown by a large drop in #UEs/cell that can be supported for all XR applications

Observation 5: 	In DU deployment scenario, FIFO based scheduling slightly outperforms PF at low load, while PF scheduling yields best performance as the load increases

### UL Capacity for multi-stream transmission

The UL capacity for multi-stream traffic is evaluated using a multiplexing-based approach where data from multiple streams are be mapped onto the same PUSCH. In this approach, a form of book-keeping is used to track the transmission and delay budget for each packet in a PUSCH.

#### Indoor Hotspot

The UL system capacity for Indoor hotspot is evaluated for two streams in FR1 (4 GHz) using SU-MIMO with PF scheduling, for a pose/control stream and an aggregated video stream at data rates of 10 Mbps. A UE is declared satisfied if more than 99% of packets are successfully transmitted for each stream with their respective PDB values, i.e., stream 1: [99%, 10ms] and stream 2: [99%, 30ms].

Figure 5: FR1 UL AR results for 2 streams in Indoor Hotspot scenario

Figure 5 shows the UL results for system capacity for multi-stream AR in an InH deployment. It is observed that a smaller number of UEs/cell can be supported compared to single-stream scenarios in UL. The trend mirrors the aggregated video performance displaying a drastic fall-off as the number of UEs per cell increases. In this case, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to about 7 UEs/cell while satisfying the PDB requirements for 2 streams (compared to < 12 UEs/cell for single stream aggregated video transmission).

Dense Urban

The UL system capacity for Dense Urban for two steams, specifically a pose/control stream and an aggregated video stream at data rates of 10 Mbps is evaluated in FR1 (4 GHz) using SU-MIMO with PF, FIFO and resource sharing based scheduling. A UE is declared satisfied if more than 99% of packets are successfully transmitted for each stream with their respective PDB values, i.e., stream 1: [99%, 10ms] and stream 2: [99%, 30ms].

Figure 6: FR1 UL AR results for 2 streams in Dense Urban scenario

Figure 6 shows the UL results for system capacity for multi-stream AR in a Dense Urban deployment. The results show that as cell load increases from 1 to 3 UEs, the resource sharing scheduling performs the best, achieving a capacity that is similar to the single stream capacity. PF and FIFO scheduling are unable to achieve 90% UE satisfaction for 3 UEs. This is because, unlike resource sharing based scheduling, PF and FIFO may end up prioritizing only certain UEs at different times. Additionally, the non-homogenous nature of the traffic (AR + CG/VR) with different arrival rates and PDB, are transparent to the PF algorithm, whereas the FIFO scheduler prioritizes CG/VR (due to the more stringent PDB), which may result in lack of resources for AR which has a high data rate. Both these (PF, FIFO) algorithms may be prone to favoring one stream (or UE) over another, which can impact overall capacity since UE success is determined based on both AR and CG/VR success. Additionally, given the TDD configuration (DDDSU) and the limited number of available uplink slots available in a radio frame, limiting the number of resources to a subset of UEs can adversely impact a packets probability of successful reception, especially for the AR 10 Mbps stream.

Observation 6: 	UL capacity of multi-stream traffic is typically less than that of single-stream traffic when using PF and FIFO based scheduling approaches.

Observation 7: 	Resource sharing based scheduling approach (e.g. allocation of RBs to all UEs in cell) enables UL capacity achieved with multi-stream traffic to be similar with that of single stream traffic

3.2 Downlink results

The downlink system capacity evaluations for the single stream and multi-stream model are presented in this section. System capacity in DL is defined as the maximum number of supported UEs out of which at least Y% of UEs are satisfied (e.g. PDB and PER requirements are met for all DL traffic streams), where Y=90 (baseline) is used in the simulations.

DL Capacity for single stream transmission

#### Indoor Hotspot

DL system capacity for Indoor hotspot (parameters defined in Table 1 in Annex) is evaluated for FR1 (4 GHz) using SU-MIMO with PF, FIFO and resource sharing based scheduling. A UE is declared satisfied if more than 99% of packets are successfully transmitted within thePDB values for CG and AR/VR applications.

Figure 7: FR1 DL CG results for Indoor Hotspot scenario at data rate of 30 Mbps

Figure 7 show the results for InH for CG at 30 Mbps. These results demonstrate that resource sharing yields the best user satisfaction at both low and high load scenarios. The achievable capacity is 9 UEs per cell for this scheme, whereas PF and FIFO can do no better than 7. An additional observation to make here is the drop in UE satisfaction under FIFO at high load scenarios. Since FIFO scheduler always prioritizes packets from UEs that have earliest arrival a bottleneck such as that experienced at high load will result in a lot of packets in the system (across other UEs) failing (due to the PDB expiration), and hence, dramatically impact the overall system capacity.

Figure 8: FR1 DL AR results for Indoor Hotspot scenario at data rate of 30 Mbps

Figure 8 show results for InH for AR at 30 Mbps. As expected, the lower PDB (10ms) for AR traffic results in a drop in the achievable capacity compared to CG with a higher PDB of 15ms. Here too, resource sharing based scheduling yields the best capacity (7 UEs) vs. PF (5 UEs).

Figure 9: FR1 DL AR results for Indoor Hotspot scenario at data rate of 45 Mbps

Figure 9 shows results for InH for AR at 45 Mbps. As expected, the high data rate and stringent PDB requirements results in the lowest capacity of all traffic cases. Even with the high load, we see that resource sharing based scheduling has the best performance of all the scheduling schemes. The achievable capacity however, for all schemes is 3 UEs per cell.

#### Dense Urban

Figure 10: FR1 DL CG results for Dense Urban scenario at data rate of 30 Mbps

Figure 10 show results for DU for CG at 30 Mbps. Resource sharing based scheduling outperforms other scheduling schemes, by supporting at least 8 UEs per cell, while FIFO and PF scheduling cap off at 7 UEs.

Figure 11: FR1 DL AR results for Dense Urban scenario at data rate of 30 Mbps

Figure 11 show results for dense urban with AR @30 Mbps. As expected, the lower PDB (10ms) for AR traffic results in a drop in achievable capacity when compared to CG at 30 Mbps with PDB of 15ms. Here too, resource sharing based scheduling yields the best capacity of all scheduling schemes, :with 8 UEs (for resource sharing) vs. 5 UEs (for PF) and 6 UEs (for FIFO).

Figure 12: FR1 DL AR results for Dense Urban scenario at data rate of 45 Mbps

Figure 12 show results for DU for AR at 45 Mbps. As expected, the high data rate and stringent PDB requirements results in the lowest capacity of all traffic cases. Even with the high load, we see that resource sharing based scheduling has the best performance of all the scheduling schemes. The achievable capacity for both resource sharing and FIFO are 4 UEs, whereas PF can support only 3 UEs.

Observation 8: 	PF-based scheduling is generally suited for maximizing purely throughput-based performance. However, the limiting factors for capacity due to the PDB requirements make PF-based scheduling less suited for XR traffic

Observation 9: 	As enhancement schemes, FIFO and resource sharing based scheduling can be better suited to meet XR-specific traffic requirements such as PDB and high throughput, and can in turn yield improved capacity performance when compared to a purely PF based approach.

DL Capacity results for multi-stream transmission

Figure 13: FR1 DL AR results for Dense Urban scenario at data rate of 45 Mbps

Figure 13 shows the performance of DL multi-stream traffic for slice-based model (Option 1A in Table 4 of Annex) for InH scenario for AR/VR at 30Mbps. The results for capacity for multi-stream case is compared with respect to single stream case, where up to 5 UEs can be supported for multi-stream case when using the slice-based traffic model. Additionally, multi-stream case results in a slightly higher percentage of satisfied UEs compared to single stream case. This can be due to handling of lower per-PDU set size (e.g. for I-frame, P-frame), on average, for the multi-stream case compared to the single stream case which typically consists of a larger packet size.

Observation 10: 	Handling of lower size of per-PDU set (e.g. for I-frame, P-frame), on average, for DL multi-stream traffic case can result in slightly higher percentage of satisfied UEs compared to the single stream case

# Conclusion

In this contribution, the following observation are made:

Observation 1: 	Achieving high system capacity with XR traffic in UL and DL in different deployment scenarios (e.g. InH, DU, UMa) is extremely challenging

Observation 2: 	Given the interdependencies between the PDUs, all PDUs in a PDU set need to be successfully delivered within a PDU set-level latency requirement for counting towards capacity

Observation 3: 	In multi-stream scenario, the PDUs in different traffic streams need to be received by UE (in DL) or application server (in UL) within a maximum inter-stream jitter value to be counted towards capacity

Observation 4: 	Transitioning from InH to DU deployment scenario has a significant impact on the capacity as shown by a large drop in #UEs/cell that can be supported for all XR applications

Observation 5: 	In DU deployment scenario, FIFO based scheduling slightly outperforms PF at low load, while PF scheduling yields best performance as the load increases

Observation 6: 	UL capacity of multi-stream traffic is typically less than that of single-stream traffic when using PF and FIFO based scheduling approaches.

Observation 7: 	Resource sharing based scheduling approach (e.g. allocation of RBs to all UEs in cell) enables UL capacity achieved with multi-stream traffic to be similar with that of single stream traffic

Observation 8: 	PF-based scheduling is generally suited for maximizing purely throughput-based performance. However, the limiting factors for capacity due to the PDB requirements make PF-based scheduling less suited for XR traffic

Observation 9: 	As enhancement schemes, FIFO and resource sharing based scheduling can be better suited to meet XR-specific traffic requirements such as PDB and high throughput, and can in turn yield improved capacity performance when compared to a purely PF based approach.

Observation 10: 	Handling of lower size of per-PDU set (e.g. for I-frame, P-frame), on average, for DL multi-stream traffic case can result in slightly higher percentage of satisfied UEs compared to the single stream case

Based on these observations, the following proposals are made:

Proposal 1: 	Capture in TR, XR-specific enhancements for capacity that support adaptations based on PDU set characteristics in UL and DL

Proposal 2: 	Capture in TR, XR-specific enhancements for capacity that support transmission of multiple associated/correlated traffic streams in UL and DL

Proposal 3: 	RAN1 to perform evaluations of XR-specific resource allocation and scheduling enhancement techniques for evaluating the capacity performance

Proposal 4: 	Reuse the baseline evaluation assumptions (e.g. deployment scenarios, SLS parameters, traffic models) in TR 38.838 when evaluating capacity performance

Proposal 5: 	Starting point for potential enhancement techniques for capacity can be those identified during Rel-17 SI (e.g. delay aware scheduling, prioritizing/pre-empting important streams in DL/UL, multi-TB scheduling, enhanced BSR)

# Annex

The following show the parameters from TR 38.838 [2] used in system level simulations for XR.

Table 1: Assumptions for System-level simulations

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:64 Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,8,2,1,1;4,8), (dH, dV) = (0.5λ, 0.5λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | Reciprocity-based precoding |  |  |
| Scheduling Algorithm | DL: SU-MIMO with PF, FIFO, and resource sharing based scheduling UL: SU-MIMO with PF, FIFO and resource sharing based scheduling FIFO - gNB prioritizes UEs for scheduling based on the arrival times and hence delay. The packet (and hence UE) with earlier packet arrival will be allocated all resources for that slot. In case of multiple streams, the arrival rate of stream with more stringent PDB is used. For e.g., for UL multi-stream, CG/AR arrivals are used for FIFO Resource sharing - gNB ensures that all packets (and hence UEs) are allocated equal resources at every scheduling instant. This guarantees all UEs some resources. |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

Table 2: DL Traffic models for CG/AR/VR evaluations

|  | CG | VR | AR |
| --- | --- | --- | --- |
| Data Rate | 30Mbps (baseline) @60fps | 30Mbps (baseline), 45Mbps @60fps | 30Mbps (baseline), 45Mbps @60fps |
| FPS | 60 fps (baseline) 120 fps (optional) Other values, e.g., 30, 90 fps can be also optionally evaluated. |  |  |
| Packet Arrival Distribution (single video stream) | Periodic (with periodicity = 1/fps) - Each packet k corresponds to set of IP packets belonging to video frame k - Jitter (with random distribution) is added to arrival slot of each packet k |  |  |
| Packet Size Distribution | Truncated Gaussian distribution - Mean: Derived from average data rate and fps as: (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes] - STD: [10.5% of Mean] - Max packet size: [150% of Mean] - Min packet size: [50% of Mean] |  |  |
| Air Interface PDB | 15ms (baseline) | 10ms (baseline) | 10ms (baseline) |
| Jitter (single video stream) | Arrival time of packet k is k/X x 1000 [ms] + J [ms], where X is the given fps value and J is a random variable (drawn from Truncated Gaussian Distribution) - Mean: [0], STD: [2 ms], Range: [[-4, 4]ms] |  |  |

Table 3: UL Traffic models for CG/VR/AR evaluations

|  | Stream 1 (pose/control data) (for CG/VR/AR) | Stream 2 (aggregated video) (for AR) |
| --- | --- | --- |
| Packet Arrival distribution | Periodic: 4ms (no jitter) | Periodic (periodicity: 1/60fps) (no jitter) |
| Data rate | 0.2 Mbps | 10 Mbps |
| Packet Size distribution | 100 bytes | Truncated Gaussian, same parameters as DL (Table 2) |
| Air Interface PDB | 10ms | 30ms |
| Capacity KPI [X, PDB] | [99%,10ms] (baseline) | [99%,30ms] (baseline) |
| Jitter | Periodic: 4ms (no jitter) | Jitter: same model as for DL |

Table 4: Statistical Parameters for Option 1A (Slice-based) Multi-stream DL traffic model

| Parameter | I-stream | P-stream |
| --- | --- | --- |
| Packet modelling | Slice-level |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |
| Number of packets per stream at a time | 1 | N – 1 |
|  | N = 8: The number of slices per frame. |  |
| Average data rate per stream | RI=R*αN-1+α | RP=R*N-1N-1+α |
| Average size ratio between one I-frame/slice and one P-frame/slice (α) | 2 |  |
| Packet size distribution | Truncated Gaussian distribution |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) |
|  | [STD, Max, Min]: [10.5, 150, 50]% of mean packet size |  |

# Reference

[1] RP-220285, New SID on XR Enhancements for NR, RAN#95e, Mar. 2022.

[2] TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17), Dec. 2021.