3GPP TSG RAN WG1 #109-e		                          			   R1-2204127

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.5.1

Source:	InterDigital, Inc.

Title:	Remaining issues for accuracy enhancements for NR positioning

Document for:	Discussion and Decision

# Introduction

In RAN1#108-e, RAN1 made agreements regarding details of enhancement for accuracy improvement [1]. In this contribution, we discuss the remaining issues for accuracy enhancement techniques for NR positioning.

# Remaining details for accuracy enhancement

## Remaining details for TEG reporting for UL based positioning

In RAN2#117e, the following agreement was made [2].

Agreements:

Proposal 3 (modified): The configurable intervals on report of association of TxTEG via RRC are ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240.  Relation to SRS intervals can be further checked in running CR discussion.

reportingAmount can be 1 or infinity.

One remaining issue is whether the UE should perform Tx TEG reporting when there is no change in Tx TEG association in the configured period. According the above agreement in RAN2, “reportingAmount=1” can imply that the UE reports TEG when there is a change in TxTEG.

Observation 1: When the UE reports TxTEG to the gNB with reportingAmount=1, it can be an event-triggered occasion

If there is no change in the Tx TEG association, the UE does not need to report the Tx TEG association information. Instead, the network may use the Tx TEG association from the last TEG reporting to derive the location of the UE. Therefore, by allowing the UE to report Tx TEG association whenever there’s a change in association, signaling overhead can be reduced significantly, especially in the infrequent Tx TEG association change and short configured reporting interval.

Proposal 1: Support the UE to report the association information between UE Tx TEG and SRS resource for UL-TDOA only if there is a change in the Tx TEG association compared to the last reporting.

Event-triggered TEG reporting:

For an accurate positioning algorithm, the network should be aware of the Tx TEG information for every SRS transmission. Otherwise, if the network is not aware of the Tx TEG information for one SRS transmission, the measurement from such SRS transmission may cause error in the positioning accuracy of the UE. Regarding the Tx TEG association reporting, the UE should report the association between Tx TEG and SRS only the association is changed compared to the previous reporting. Such information is known exactly at the UE. Therefore, allowing the UE to report the updates association between Tx TEG and SRS when it changes the association should be supported.

Proposal 2: Support the UE to report the association information between UE Tx TEG and SRS resource whenever the UE determines the previous association information is no longer valid.

## DL-AoD enhancements

In RAN1#107e, the following agreement was made related to DL-AoD [3].

Agreement For UE-assisted DL-AOD positioning method, to enhance the signaling to the UE for the purpose of PRS resource(s) reporting, the LMF may indicate in the assistance data (AD), one or both the following: option 1: subject to UE capability, for each PRS resource, a subset of PRS resources for the purpose of prioritization of DL-AOD reporting: a UE may include the requested PRS measurement for the subset of the PRS in the DL-AoD additional measurements if the requested PRS measurement of the associated PRS is reported The requested PRS measurement can be DL PRS RSRP and/or path PRS RSRP. UE may report PRS measurements only for the subset of PRS resources. Note: The subset associated with a PRS resource can be in a same or different PRS resource set than the PRS resource option 2: subject to UE capability, for each PRS resource, the boresight direction information. Note: Either case does not imply any restriction on UE measurement FFS: prioritization of the PRS resources and resource subsets to be measured

In RAN1#108e, the details of the report sent by the UE were discussed when the UE is configured with an association between a subset of PRS resources and a PRS resource. For clarification, the following proposal was made by the feature lead [4].

Proposal 3.5b: To support reporting of measurement based on the PRS subsets in DL AOD measurement report, the LMF may transmit the following to the UE in a measurement request: - Request the UE to report PRS measurements only based on the PRS associated subsets - Request the UE to report PRS measurements based on the PRS and its associated subset

The above proposal concerns 2-stage beam sweeping where wide beam sweeping is followed by beam sweeping with narrower beamwidth. In the proposal above, the first bullet corresponds to the case when there are two reporting occasions for reporting during 2-stage beam sweeping. Firstly, the UE reports measurements for the first set of sweep with beams with wide beamwidth. After the first sweep, a sweep with the subset of beams, i.e., beams with narrower beamwidth, follows and the UE reports measurements corresponding to the subset.

On the other hand, the second bullet corresponds in the above proposal to the case when the UE includes measurements for both wide and narrow beams (the subset with an associated PRS resource) and include the measurements in the same report. Both bullets in the feature lead’s proposal shall be agreed as both are relevant to 2-stage beam sweeping.

Proposal 3: When PRS is associated with PRS subsets, support reporting from the UE to include measurements related to subsets associated with PRS or PRS and its associated subsets, depending on the request from the LMF.

## Remaining details for multipath reporting

In RAN1#107e, the following agreement related to the granularity of LOS/NLOS indication was made [3].

Agreement Support the following two options of values for LoS/NLoS indicator reporting from UE/TRP: Soft values: [0, 0.1, …, 0.9, 1] (in steps of 0.1) Hard values: [0, 1] The values correspond to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS

According to the above agreement, indication of soft values and hard values is supported. In RAN1#108-e [1], it was discussed whether a UE can support both soft and hard value reporting. From the agreement above, it seems clear that the UE can either report soft values or hard values, depending on whether the UE is capable of reporting soft or hard values, respectively. Since the candidate values for soft values include 0 and 1, motivation for supporting a UE capability which can report both soft and hard value is not clear.

From the discussion in RAN1#108-e [1], the supporting argument for supporting both types of values is that the LMF can request the UE to report either soft or hard value where the UE is capable of reporting both soft and hard values. However, benefits for the network selecting between soft or hard value is not clear. Another supporting argument for supporting both soft or hard value is that higher confidence can be achieved with one indicator (e.g., soft) over another (e.g., hard) indicator. However, reliability associated with a soft or hard indicator is up to the quality of UE implementation. Soft or hard indicator with similar reliability can be supported depending on the implementation. Thus, we make the following proposal :

Proposal 4: A UE capability supporting both soft and hard value reporting for LoS/NLoS indication is not supported

In addition, the following proposal was made in the feature lead’s summary in RAN1#108e [5].

Proposal 4.1-D - Support reporting differential RSRPP for the PRS-RSRPP measurement in DL-TdoA and multi-RTT for at least the additional paths. - Alt. 1: The reference, for calculation of differential RSRPP, for all the additional paths PRS-RSRPPs is the PRS-RSRPP of the strongest path associated with the nr-RSTD-r16 or nr-UE-RxTxTimeDiff-r16 measurement. - Alt. 2: The reference, for calculation of differential RSRPP, for the additional path PRS-RSRPP is the PRS-RSRPP of the strongest path of the same PRS resource as the additional path PRS-RSRPP.

Pros and cons of the above alternatives were discussed in RAN1#108e. While the proposal in Alt. 1 preserves the definition of a reference PRS similar to the reference RSTD, the proposal in Alt. 2 sets the reference within each PRS resource. Under Alt. 1, the reference used for differential RSRPP calculation may have relatively small or large RSRPP compared to other PRS resources. Thus, since the same reference is used for all PRS resources in Alt. 1, range of the differential RSRPP may be larger, compared to the range in Alt. 2, possibly causing saturation in the range for Alt. 1. Thus the following observation is made.

Observation 2: For Alt. 1 for calculation of differential RSRPP, whether differential RSRPP can exceed the range should be investigated.

# Conclusion.

In this contribution, the following proposals and observation are made.

TEG reporting for UL-based positioning

Observation 1: When the UE reports TxTEG to the gNB with reportingAmount=1, it can be an event-triggered occasion

Proposal 1: Support the UE to report the association information between UE Tx TEG and SRS resource for UL-TDOA only if there is a change in the Tx TEG association compared to the last reporting.

Proposal 2: Support the UE to report the association information between UE Tx TEG and SRS resource whenever the UE determines the previous association information is no longer valid.

AoD Enhancements

Proposal 3: When PRS is associated with PRS subsets, support reporting from the UE to include measurements related to subsets associated with PRS or PRS and its associated subsets, depending on the request from the LMF.

Multipath/NLOS mitigation

Proposal 4: A UE capability supporting both soft and hard value reporting for LoS/NLoS indication is not supported

Observation 2: For Alt. 1 for calculation of differential RSRPP, whether differential RSRPP exceeds the range should be investigated.

# Reference

[1] RAN1 Chairman’s notes, RAN1#108-e, March 2022.

[2] 3GPP, Session Chair’s note “Report from session on positioning and sidelink relay,” RAN2#117-e, March 2022.

[3] 3GPP, Session Chair’s note “Report from session on positioning and sidelink relay,” RAN1#107-e, Nov. 2021.

[4] R1-2202605, “FL summary #3 for AI 8.5.3 Accuracy improvements for DL-AoD positioning solutions,” Ericsson, RAN1#108-e, March 2022.

[5] R1-2202559, “Feature Lead Summary #4 for Maintenance of multipath/NLOS mitigation,” Nokia, RAN1#108-e, March 2022.