3GPP TSG-RAN WG1 #109 electronic	                         R1-2204129

e-Meeting, 9th May– 20th May, 2022

Source:	III

Title:	Discussion on XR specific capacity enhancements techniques

Agenda item:	9.11.2

Document for:	Discussion

- Introduction

In 3GPP Rel-18 XR enhancements for NR, “XR-specific capacity improvement enhancements” had been accepted to be the study item. The objects are [1]:

Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics(periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; Dynamic scheduling/grant enhancements.

In TR38.838 [2], “XR-specific capacity improvement enhancement” include:

- Staggering of packet arrivals at gNB among UEs - Delay aware scheduler - Frame level integrated transmission scheduler - Cooperative MIMO/precoding via bi-directional training (BiT) - Network coding (NC)/outer coding (OC) - gNB scheduling awareness UE playout buffer - Impact of carrier aggregation - Prioritizing important stream - Adaptive inter-UE/intra-UE multiplexing techniques - HARQ-ACK enhancement for DG scheduling - Enhanced buffer status reporting for UL transmission - Application data unit (ADU) dropping

## In this contribution, we focus on two important issues, i.e. “Prioritizing important stream” and ”Adaptive inter-UE/intra-UE multiplexing techniques”.

- Discussion

  - Potential XR-specific capacity improvement enhancements

In TR 38.838[1], there are many potential XR-specific capacity improvement enhancements have been proposed.  However, to enhance SPS, CG and DG for XR-specific, the following issues need to be further considered:

- Prioritizing important stream

- Adaptive inter-UE/intra-UE multiplexing techniques

    - Prioritizing important stream

In XR service, there are many types of streams, e.g. I-frame, P-frame, pose/control, video or audio. Each stream may have different priority in the service. Furthermore, legacy URLLC and eMBB traffic needs to consider multiplexing with XR. The existing priory index is only one bit, which is used to differentiate URLLC and eMBB traffics. In order to properly accommodate the XR service, we propose to increase the priority index to be more than one bit.

Proposal#1: In order to properly accommodate the XR service, we propose to increase the priority index to be more than one bit.

    - Adaptive inter-UE/intra-UE multiplexing techniques

Before the R18, there are only two classes of services in 5G network, i.e. URLLC and eMBB. Due to the characteristic of these two services, when there is scheduling conflict, the URLLC service always punctures the resource elements already scheduled for the eMBB service.

In R18, the XR service is added as the third class of services in 5G network, the following issues need to be considered:

- Due to the XR service being delay sensitive, when a XR service frame is preempted by URLLC, the impact for the XR service shall be reduced. For example, the legacy preemption mechanism can only be allocated in time domain. We propose both the time and frequency domains shall be considered to reduce delay impacts, when the XR frame is preempted.

- Because the XR service is delay sensitive, we propose the case of XR service preempting URLLC shall be considered.

- In R15, when the eMBB service being preempted by the URLLC service, the URLLC service is using mini-slot level to scanning PDCCH. In XR service, we propose to consider whether to continue supporting URLLC-like mini-slot scheduling for XR multiplexing and preempting with eMBB.

Proposal#2: We propose both the time and frequency domains shall be considered to reduce delay impacts, when the XR frame is preempted.

Proposal#3: Because the XR service is delay sensitive, we propose the case of XR service preempting URLLC shall be considered.

Observation#1: In XR service, we propose to consider whether to continue supporting URLLC-like mini-slot scheduling for XR multiplexing with eMBB.

- Conclusion

From the perspectives of priority index granularity and multiplexing/preemption for XR, we have the following observations and proposals:

Proposal#1: In order to properly accommodate the XR service, we propose to increase the priority index to be more than one bit.

Proposal#2: We propose both the time and frequency domains shall be considered to reduce delay impacts, when the XR frame is preempted.

Proposal#3: Because the XR service is delay sensitive, we propose the case of XR service preempting URLLC shall be considered.

Observation#1: In XR service, we propose to consider whether to continue supporting URLLC-like mini-slot scheduling for XR multiplexing with eMBB.

References

[1] 	RP-213558, “Study on XR Enhancements for NR,” RAN#94-e.

[2] 	TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17),” V17.0.0.