3GPP TSG RAN WG1 #109-e		                                    		   R1-2204133

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.1

Source:	InterDigital, Inc.

Title:	Integrity for RAT dependent positioning techniques

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, the contents of SID for Rel. 18 were agreed [1]. It was agreed to study integrity for RAT dependent positioning. The descriptions in the SID are as follows.

- Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

Integrity for positioning is defined as follows in TS 38.305 [2]:

“Positioning integrity: A measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts.”

In this contribution, we discuss potential solutions for error sources and integrity for RAT dependent positioning techniques.

# Error sources for determination of integrity

Integrity error sources can refer to any components or events in the network that may impact the positioning measurements and integrity calculations. During Rel-17 SI [3], the errors sources for GNSS integrity were modeled as feared events and grouped under the following categories/subcategories:

- Feared events in the GNSS assistance data

- Feared events during positioning data transmission

- GNSS feared events

  - Satellite feared events

  - Atmospheric feared events

  - Local environment feared events (e.g. multipath, LOS/NLOS, interference)

- UE feared events (e.g. HW and SW error sources)

- LMF feared events

For RAT dependent positioning a similar modeling approach for error sources can be used. In this case, some of the error sources which are common to both RAT dependent positioning and GNSS can be reused and other error sources which are different can be adapted to be specific to RAT-dependent positioning (e.g. feared events in assistance data). For RAT dependent positioning, the local environment feared events such as the following can be used when determining integrity.

- LOS/NLOS indicator : This is an indication of the presence of multipaths in the channel. Both UE-based and UE-assisted LOS/NLOS indication via hard (0,1) or soft value (a value between 0 and 1 at specified granularity) are supported in Rel. 17. The indicator can be generated by the LMF or UE. The indicator can be used to detect potential error event due to the presence of multipaths.

- TEG (Timing Error Group) : This is an indication of the unknown timing offset in measurements. Both Tx and Rx TEG are supported in Rel. 17. If RS resources belong to the same TEG, the measurements corresponding to the resources contain similar amount of timing offset. TEG can be useful for double differential time difference-based positioning to eliminate the unknow timing offset. An indication of TEG can assist the UE or LMF to determine a potential error event due to the unknown timing offset.

- Multipaths information : This is a more explicit and detailed information regarding the multipath channel. The information may contain relative time delay or differential RSRPP which can assist the UE or LMF for determination of integrity.

- Uncertainty/expectation in measurements : Range in measurements, provided in assistance information, can be used to determine uncertainty in measurements. For example, nr-DL-PRS-ExpectedRSTD-Uncertainty or nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedAoD-or-AoA are specified in assistance information for the UE [4].

Proposal 1: For error sources for determining integrity, LOS indicator, TEG, multipath information and uncertain or expectation in measurements should be considered.

# Procedure to determine integrity

## Integrity concepts and metrics

When supporting a positioning service for application or LCS client (MT-LR or MO-LR), the positioning error can be determined as E = |PAL – P|, where PAL is the UE location info with error tolerated by the application within an alert limit (AL) and P is the location info determined using a positioning method (e.g. DL-TDOA, UL-RTOA).

As integrity requirements for automotive, rail and IIoT uses cases were discussed during Rel-17 [2][3], AL can be defined as the maximum tolerable error for an intended application. The other integrity requirements which are related to AL and discussed for GNSS integrity are time-to-alert (TTA) and target integrity risk (TIR) [2][4]. TTA refers to the maximum allowable elapsed time from when the positioning error exceeds the AL until an alert message is transmitted to the application. TIR refers to the probability that the positioning error exceeds AL without an alert message is transmitted within the TTA. Given that the integrity requirements (e.g. AL, TTA, TIR) discussed during Rel-17 are generic and not specific to GNSS, they can be considered for RAT dependent positioning.

Observation 1 : KPI (AL, TTA and TIR) ranges for automotive, rail and IIoT use cases are shown in TR 38.857

Proposal 2: The integrity requirements corresponding to AL, TTA and TIR in TR 38.857 can be considered for RAT dependent positioning

During Rel-17, the protection level (PL) concept was introduced and discussed as a way for calculating the integrity result [3][4]. The PL was defined as a statistical upper-bound of the positioning error that ensures that the probability of the error being greater than AL and for longer than TTA is less than TIR. In this case, the PL can be used for further bounding the positioning error (e.g. E’ = |PPL – P|) when calculating the integrity result as a function of different error sources or feared events including LOS, TEG and uncertainty in measurements.

## UE-based vs. UE-assisted integrity determination

In Rel-17, the support for integrity was specified for GNSS-based positioning, in particular for UE-based mode of GNSS positioning. In Rel-18 similar principles can be applied for supporting integrity for RAT-dependent positioning methods for both UE-based and UE-assisted modes.

Proposal 3: Study both UE-based and UE-assisted modes for a support for integrity for RAT dependent positioning methods

For UE-based mode, the UE calculates its own location and integrity based on the assistance data for integrity received from the network and PRS measurements performed by UE. In UE-assisted mode, UE makes PRS measurements and sends to LMF the measurement reports, based on which the LMF determines UE location and calculates integrity. Within the UE-based and UE-assisted modes, integrity can be supported for both MO-LR and MT-LR service types. In this regard, for both UE-based and UE-assisted modes the assistance data for integrity that is provided to the UE and the measurements/calculations reported by UE for integrity should be further studied.

Proposal 4: Study and identify the assistance data that can be provided to UE for integrity for UE-based and UE-assisted modes

Proposal 5: Study and identify the measurements/calculations that can be reported by UE for integrity for UE-based and UE-assisted modes

## Integrity results reporting

The modes for reporting of the calculated integrity result discussed during Rel-17 are as follows [3][4]:

- Mode 1 of Integrity Result Reporting: PL Reporting

- Mode 2 of Integrity Result Reporting: Integrity Event Flagging

Both modes of integrity result reporting can be beneficial, depending on granularity of integrity information required by the LCS client/application. Both modes of reporting can also be applicable for UE-based and UE-assisted integrity.

For example, Mode 1 can be used in scenarios where the application requires the calculated PL result based on monitoring of error sources/feared events and UE location info. Mode 2 can be useful in scenarios where the application/LCS client requires only information on whether integrity performance is met with respect to the integrity requirements (e.g. AL, TTA, TIR).

For RAT-dependent positioning, Mode 2 can also include reporting of a do not use (DNU) flags for certain beams or TRPs, for example. Alternatively, a binary flag (0 and 1) to indicate whether the positioning system is available or not, i.e., positioning system is still available to offer trustable position estimation [3].

Proposal 6: For integrity result reporting, Mode 1 (PL reporting) and Mode 2 (integrity flag reporting) can be considered for UE-based and UE-assisted integrity

## Integrity alert

From the definition of positioning integrity [3], UE or LMF provides timely alerts to application/LCS client when the associated integrity requirements are not met. In other words, when an integrity related event (e.g. PL > AL) or feared event is detected, the UE (for UE-based) or LMF (for UE-assisted) sends the alert messages to the LCS client, indicating potential failure condition for integrity. Such alerts can also be beneficial and applicable for RAT dependent positioning in UE-based and UE-assisted modes, when considering detection of any feared events (e.g. LOS, TEG) that may potentially impact the integrity result.

Proposal 7: Transmission of integrity alerts can be considered when detecting integrity related events or feared events for UE-based and UE-assisted modes

## Compatibility between GNSS and RAT dependent integrity

For enabling the use of a compatible integrity framework, a mechanism for combining integrity results derived from RAT independent and RAT dependent positioning can be considered. For UE-based mode, the UE location info determined via RAT-independent positioning method (e.g. GNSS positioning) can be used for cross-checking and improving the integrity performance achievable via a RAT-dependent positioning method (e.g. DL-TDOA, UL-RTOA). Similarly, for UE-assisted positioning, the LMF can use the UE location info determined by via GNSS and RAT-dependent positioning methods for validation and improving the confidence level/accuracy of the positioning info and integrity. When the UE reports the location info/measurements/integrity results to the LMF, identifiers/tags can be used to identify the different positioning methods applied for determining the positioning info.

Proposal 8: Study how RAT independent and RAT dependent positioning methods can be made to be compatible with each other and leveraged for improving integrity

# Conclusion.

In this contribution, the following proposals and observation are made.

Observation 1 : KPI (AL, TTA and TIR) ranges for automotive, rail and IIoT use cases are shown in TR 38.857

Proposal 1: For error sources for determining integrity, LOS indicator, TEG, multipath information and uncertain or expectation in measurements should be considered.

Proposal 2: The integrity requirements corresponding to AL, TTA and TIR in TR 38.857 can be considered for RAT dependent positioning

Proposal 3: Study both UE-based and UE-assisted modes for a support for integrity for RAT dependent positioning methods

Proposal 4: Study and identify the assistance data that can be provided to UE for integrity for UE-based and UE-assisted modes

Proposal 5: Study and identify the measurements/calculations that can be reported by UE for integrity for UE-based and UE-assisted modes

Proposal 6: For integrity result reporting, Mode 1 (PL reporting) and Mode 2 (integrity flag reporting) can be considered for UE-based and UE-assisted integrity

Proposal 7: Transmission of integrity alerts can be considered when detecting integrity related events or feared events for UE-based and UE-assisted modes

Proposal 8: Study how RAT independent and RAT dependent positioning methods can be made to be compatible with each other and leveraged for improving integrity

# Reference

[1] 3GPP, “Study on expanded and improved NR positioning,” RP-213588, RAN#94e, Dec. 2021.

[2] 3GPP TS 38.305, “NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17), v17.0.0, Mar 2022

[3] 3GPP TR 38.857, “Study on NR Positioning Enhancements (Release 17), v17.0.0, Mar 2021

[4] 3GPP TS 37.355, “LTE Positioning Protocol (LPP) (Release 17), v17.0.0, Mar 2022