3GPP TSG RAN WG1 #109-e		                          			   R1- 2204134

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.2

Source:	InterDigital, Inc.

Title:	Potential solutions for carrier phase based positioning

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, the contents of SID for Rel. 18 were agreed [1]. It was agreed to study positioning based on NR carrier phase measurements. The descriptions in the SID are as follows.

- Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

In this contribution, we discuss proposals related to potential positioning methods and reference signal designs for phase-based positioning.

# Carrier phase measurements

## Positioning using phase measurements

In GNSS, carrier phase measurements are used for positioning [2,3]. For phase-based positioning, phase difference based positioning can be considered. For example, based on phase difference of one PRS, transmitted from a TRP, between two received antennas, distance between UE and TRP can be estimated using antenna geometry [2]. Alternatively, two TRPs can transmit two PRSs, which can be differentiated by scrambling codes or polarization if they are transmitted using the same time-frequency resource, and the UE can measure phase difference between the received PRSs. Thus, the following proposal is made.

Proposal 1: Study transmission and reception modes (e.g., 2 TX 1RX, 1Tx 2 RX) for phase-difference based positioning

In addition, how phase measurement based positioning can be configured for the UE should be discussed. In NR, both timing and angle-based positioning have been specified. Phase measurements can serve as assisting measurements which means that phase-based positioning is configured jointly with other positioning methods, e.g., DL-TDOA. Thus, whether phase based positioning can be stand-alone or joint positioning method should be discussed in the study.

Proposal 2: Study both stand-alone phase-based positioning and/or joint positioning methods, e.g., joint timing and phase-based measurements

In typical phase-based positioning, difference or double-difference of phase measurements is used for positioning to mitigate unknown phase offset present in the measurements. For this purpose, for the UE to compute phase difference, transmitted PRS resources targeted for phase-difference based positioning, should share the common or similar unknown phase offset characteristics. Thus, a mechanism to include information indicating that PRS resources share similar unknown phase characteristics should be studied.

One possible approach is to specify a phase error group. PRS resources in the same phase error group may share similar unknown phase offset, since, for example, they are transmitted from the same amplifier. Knowing that the PRS resources share similar unknown phase offset characteristics may allow the receive to implement differential or double-differential phase calculation to mitigate residual phase in the measurements.

Proposal 3:  Study a framework that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group.

## Positioning in the presence of multi-path channel

Presence of multiple paths can significantly affect the accuracy of phase-based positioning. Due to mutipath, phase of the signal is corrupted by reflected signals. Mitigation techniques to enhance performance of phase based positioning in the presence of multiple-path channel should be studied.

Proposal 4: Study multi-path mitigation techniques for phase-based positioning

## Reference signal design for phase measurements

In GNSS, carrier signals modulated by pseudo noise sequences are used for positioning. In NR, PRS are design with N-comb patterns and N OFDM symbols are placed in a slot for positioning purpose. In the comb pattern, PRS is not placed continuously in a carrier across OFDM symbols. Thus, consistent carrier phase measurement is not possible.

PTRS is specified in NR for both DL and UL [4]. It is possible to use PTRS for phase measurements. However, since PTRS are multiplexed with data, continuous placement of RS in the time domain is not possible. In addition, configurations of PTRS are associated with DMRS [4] which limits flexibility in PTRS configuration. Thus, there is a need to place PRS in the time domain such that phase of the received signal can be measured accurately. In addition, if more than one PRSs are transmitted at the same time-frequency resource, there should be a mechanism to differentiate the transmitted PRSs by the receiver, i.e., multiplexing via pseudo random code.

Proposal 5: Study new PRS designs for carrier phase measurements for UL and DL

Proposal 6: A method to differentiate multiple PRSs, if they are transmitted in the same time-frequency resource, should be identified

## Measurements

Finally, the UE should include phase difference measurements in the report. The UE needs to include information related to the reference PRS in the report, if it is not configured by the network. In addition, in the presence of integer ambiguity [5] due to transmission sources located beyond one cycle of the wavelength of the carrier, the UE can report estimated number of cycles of the received carrier.

Proposal 7: Study contents of the measurement report for phase-difference based positioning.

# Conclusion.

In this contribution, the following proposals are made.

Proposal 1: Study transmission and reception modes (e.g., 2 TX 1RX, 1Tx 2 RX) for phase-difference based positioning

Proposal 2: Study both stand-alone phase-based positioning and/or joint positioning methods, e.g., joint timing and phase-based measurements

Proposal 3:  Study a framework that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group.

Proposal 4: Study multi-path mitigation techniques for phase-based positioning

Proposal 5: Study new PRS designs for carrier phase measurements for UL and DL

Proposal 6: A method to differentiate multiple PRSs, if they are transmitted in the same time-frequency resource, should be identified

Proposal 7: Study contents of the measurement report for phase-difference based positioning.

# Reference

[1] 3GPP, “Study on expanded and improved NR positioning,” RP-213588, RAN#94e, Dec. 2021.

[2] R. A. Brown, “Instantaneous GPS attitude determination,” IEEE Aerospace and Electronic Systems Magazine, vol. 7, no. 6, June, 1992, pp. 3-8,

[3] P. Axelrad, C. J. Comp and P. F. Macdoran, “SNR-based multipath error correction for GPS differential phase,” IEEE Trans. On Aerospace and Electronic Systems, vol. 32, no. 2., April 1996, pp. 650-660.

[4] TS 38.211, “Physical channels and modulation,” ver. 16.5.0, March, 2021.

[5] F. V. Diggelen, “A-GPS : Assisted GPS, GNSS and SBAS,” Artech House, 2009.