3GPP TSG RAN WG1 #109e		R1-2204135

e-Meeting, May 9th – 20th, 2022

Source:	Panasonic

Title: 	Discussion on evaluation on NR duplex evolution

Agenda Item:		9.3.1

Document for:	Discussion

Introduction

Objectives for “New SI: Study on evolution of NR duplex operation” were agreed as follows [1]. This document provides our view on evaluation assumptions and our current link-level simulation results for the subband non-overlapping full duplex.

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

Discussion

For the deployment scenarios, since one of the motivations for NR duplex evolution is to improve UL coverage, scenarios used for coverage enhancement evaluation can be a starting point (e.g., FR1 with Urban and Rural) [2].

In order to keep the interference to other frequencies in the band and to increase the coverage/throughput of UL, both edges of a band are DL and the center frequency are assigned for UL should be evaluated as the main target. Figure 1 shows the example of the target subband allocation.

If DL and UL transmissions are assumed as in-band (e.g., one BWP includes DL and UL subbands), CLI due to time difference between DL and UL symbols needs to be taken into account. For the evaluation of DL performance caused by UL interference, at the UE point, DL and UL time difference may not be within CP length. Therefore, DL and UL time difference is beyond CP should be evaluated for DL performance caused by UL interference at UE (i.e., time difference beyond CP causes inter-carrier interference due to loss of orthogonality). For the evaluation of UL performance caused by DL interference at gNB, DL and UL time difference would be usually within CP length. Therefore, DL and UL time difference is within CP should be enough for UL performance caused by DL interference at gNB.

Figure 1 Example of target subband allocation

Proposal 1: The following assumptions can be considered for link-level simulation.

- Scenarios used for coverage enhancement evaluation can be a starting point.

- Typical subband allocation can be that edges of a band are for DL and the center of a band is for UL.

- For DL evaluation, CLI due to time difference between DL and UL symbols needs to be taken into account if DL and UL are assumed as in-band.

We evaluated DL performance caused by UL interference when DL and UL are in-band. The aggressor UE transmits UL and the victim UE receives DL in adjacent subband. DL performance of victim UE is evaluated. Figure 2 shows an example of RB allocation ((a) shows no guard band case, (b) shows 1 RB guard band case). DL performance depending on CLI intensity is evaluated by “DL/UL power ratio”, which means that the ratio of DL reception power to UL reception power at victim UE (e.g., “0 dB” intends that DL and UL reception power are the same). Simulation parameters used for this evaluation are shown in Table 1 in Appendix.

Figure 2 Example of RB allocation

Figure 3 and Figure 4 show simulation results (MCS 0, 10 respectively). Each graph in the figure corresponds to results for different guard band size (0, 1, 2, 4, 10 RBs).

For Figure 3, the left-top graph shows simulation results for MCS 0 without guard band. “DL only” means that there is no interference. “D/U=-20dB” means that DL/UL power ratio is -20 dB. When DL/UL power ratio is 10 to 20 dB, performance based on 10% BLER is not degraded even if there is no guard band. When DL/UL power ratio is 0 dB, performance is not degraded if guard band is 1 RB (see next graph). When DL/UL power ratio is -10 dB, performance is not degraded if guard band is 2 RB (see next graph). When DL/UL power ratio is -20 dB, performance is not degraded if guard band is 10 RB (see last graph). According to these results, guard band can mitigate interference from UL transmission in adjacent subband. The suitable guard band size is different due to DL/UL power ratio (i.e., CLI intensity). Thus, CLI measurement/repot from UE can be useful for the adjustment of guard band.

For MCS 10 (Figure 4), performance improvement by guard band can be observed as well. Therefore, we made following observations.

Observation 1: Guard band can mitigate inter-UE CLI from adjacent subband.

Observation 2: CLI measurement/repot from UE would be useful for the adjustment of guard band.

Figure 3 Simulation results (MCS 0)

Figure 4 Simulation results (MCS 10)

Conclusion

In this contribution, we made the following proposal and observations.

Proposal 1: The following assumptions can be considered for link-level simulation.

- Scenarios used for coverage enhancement evaluation can be a starting point.

- Typical subband allocation can be that edges of a band are for DL and the center of a band is for UL.

- For DL evaluation, CLI due to time difference between DL and UL symbols needs to be taken into account if DL and UL are assumed as in-band.

Observation 1: Guard band can mitigate inter-UE CLI from adjacent subband.

Observation 2: CLI measurement/repot from UE would be useful for the adjustment of guard band.

- Reference

- RP-220633, “Revised SID: Study on evolution of NR duplex operation”, CMCC

- 3GPP TR 38.830 V17.0.0 (2020-12)

- Appendix: Link-level simulation parameters

Table 1 Link-level simulation parameters

| Parameter | Values |
| --- | --- |
| Carrier frequency | 4 GHz |
| System bandwidth | 20 MHz |
| Subcarrier spacing | 15 kHz |
| UL waveform | CP-OFDM |
| Channel model | TDL-C |
| Delay spread | 300 ns |
| UE speed | 3 km/h |
| PRB allocation | DL: 1 PRB, UL: 1 PRB |
| Symbol allocation | 14 symbols |
| Number of layers | 1 |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 |
| DMRS configuration | Type 1 with single DMRS symbol |
| MCS | 0 (QPSK, code rate: 0.117), 10 (16QAM, code rate: 0.332) |
| DL/UL power ratio at victim UE | -20, -10, 0, 10, 20 dB |
| Time difference between DL and UL at victim UE | 13 μs |
| Guard band size | 0, 1, 2, 4, 10 RBs |