3GPP TSG RAN WG1 #109-e		R1-2204136

e-Meeting, May 9th – 20th, 2022

Source:	OPPO

Title:	Discussion on UE features for FR2-2

Agenda Item:	8.16.2

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the UE features for FR2-2. This discussion is based on the summary of UE features for supporting NR from 52.6 GHz to 71 GHz [1].

Discussion

In the last RAN1#108-e meeting, a great progress was made on the UE feature for FR2-2, where the FGs were agreed in the most details. In this section, we further discuss some of the remaining points to further polish this list to a final version.

FG 24-1d, FG24-1f

In the last version, separate FGs of multi-PDSCH scheduling by single DCI for 120kHz SCS are defined for FR2-2 and FR2-1 respectively. In our view, type 1 and type 2 HARQ codebook are introduced mainly for multi-PDSCH scheduling case, and these two features should be bundled. We propose to remove the brackets for FG 24-1d and FG24-1f.

Proposal 1: for FG24-1d and FG24-1f, removing the corresponding brackets.

FG 24-8, FG 24-9

Considering the capability requirements of supporting 32 HARQ processes are different in single carrier case and CA case, more CCs require high UE cost if the capability is not differentiated per CC, so we think it is beneficial report this capability per FSPC instead of per UE or per band, to leave more flexibility for UE implementation and cost consideration.

Proposal 2: for FG 24-8 and FG 24-9, supporting ‘per FSPC’ for 32 HARQ processes.

Conclusion

In this document, we provide our views on some of the feature groups based on conclusion in last RAN1 meeting for supporting NR from 52.6 GHz to 71 GHz. Our proposals are made as follows:

Proposal 1: for FG24-1d and FG24-1f, removing the corresponding brackets.

Proposal 2: for FG 24-8 and FG 24-9, supporting ‘per FSPC’ for 32 HARQ processes.

Reference

- R1-2202851, Summary of UE features for supporting NR from 52.6 GHz to 71 GHz, Moderator (AT&T).