3GPP TSG RAN WG1 #109-e	   	 	  	                	      R1-2204138

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.1.2

Source:	LG Electronics

Title:	Text proposal on multi-TRP deployment

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss a remaining issue on beam management for multi-TRP.

Discussion

Beam failure recovery for multi-TRP

In RAN1#108-e, RRC + MAC CE based explicit BFD-RS configuration/update was agreed to be supported, as below.

Agreement Support to configure/update explicit BFD -RS set by RRC signaling and MAC CE signaling

The agreement and the related specification has an ambiguity on the behavior when the number of BFD-RS(s) in a set is equal to or less than NBFD. To our understanding, there is no need to activate BFD-RS(s) by MAC-CE signaling in this case.

Proposal #1: Clarify that no MAC-CE activation is required for explicit BFD-RS when the number of RRC configured BFD-RS(s) in a set is equal to or less than NBFD. Adopt the following TP for Rel-17 M-TRP beam management.

- Reason for change: Ambiguous behavior when the number of BFD-RS(s) in a set is equal to or less than NBFD.

- Summary of change: UE considers that RRC configured BFD-RS(s) is activated when the number of BFD-RS(s) in a set is equal to or less than NBFD.

- Consequences if not approved: UE cannot monitor/detect the RRC configured BFD-RS(s) until activation MAC CE is transmitted, when the number of BFD-RS(s) in a set is equal to or less than NBFD.

- Proposed TP for TS 38.213:

TS 38.213 (section 6) # 6 Link recovery procedures A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. If the number of resources provided in each set q0,0 and q0,1 is equal to or less than NBFD, UE considers that the resources in each set q0,0 and q0,1 are activated. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. - unchanged part is omitted -

Conclusion

In this contribution, we share our view on the remaining issue for Rel-17 enhancements on beam management for multi-TRP, and propose the following based on the discussion.

Proposal #1: Clarify that no MAC-CE activation is required for explicit BFD-RS when the number of RRC configured BFD-RS(s) in a set is equal to or less than NBFD. Adopt the following TP for Rel-17 M-TRP beam management.

- Reason for change: Ambiguous behavior when the number of BFD-RS(s) in a set is equal to or less than NBFD.

- Summary of change: UE considers that RRC configured BFD-RS(s) is activated when the number of BFD-RS(s) in a set is equal to or less than NBFD.

- Consequences if not approved: UE cannot monitor/detect the RRC configured BFD-RS(s) until activation MAC CE is transmitted, when the number of BFD-RS(s) in a set is equal to or less than NBFD.

- Proposed TP for TS 38.213:

TS 38.213 (section 6) # 6 Link recovery procedures A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. If the number of resources provided in each set q0,0 and q0,1 is equal to or less than NBFD, UE considers that the resources in each set q0,0 and q0,1 are activated. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. - unchanged part is omitted -

References

[1] TS 38.213 V17.1.0, NR; Physical layer procedures for control