3GPP TSG RAN WG1 #109-e				                            R1-2204141

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.1.1

Source:	LG Electronics

Title:	Unified TCI framework extension for multi-TRP/panel

Document for:	Discussion and Decision

Introduction

In RAN#94 meeting, the objectives for further enhancements on MIMO for NR were approved [1], which includes the following objectives related to multi-beam (MB) operations for multi-TRP/panel operation:

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss potential enhancements on MB operations for multi-TRP/panel based on the Rel-17 TCI framework for Rel-18.

- Discussion

In this section, we discuss three WI objectives related to BM, i.e., Rel-17 TCI framework extension, BM for simultaneous multi-panel UL transmission, and UL power control enhancement for mTRP in each subsection, respectively.

Extension for indication of multiple DL/UL TCI states

In Rel-17 unified TCI framework, dynamic beam indication/update operation using joint TCI and separate DL/UL TCI state have been introduced and gNB utilizes DCI and/or MAC-CE to indicate a TCI state for applying the common beam to the target channels/RSs. Related to the common beam applicability for the indicated TCI state(s), the variables M and N were defined with relevant terms such as joint/DL/UL TCI as captured below, but unfortunately, only M=1 and N=1 cases were specified in Rel-17 due to the limited discussion time.

Conclusion (in RAN1#104-e) On Rel.17 unified TCI framework, based on the agreements in RAN1#102-e and 103-e, the following terms are defined as follows (at least for the purpose of discussion and reaching agreements). For M=1: - DL TCI: The source reference signal(s) (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC For N=1: - UL TCI: The source reference signal in the UL TCI provides a reference for determining UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC For M=N=1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. - Separate DL/UL TCI: The DL TCI and UL TCI are distinct (therefore, separate). For M>1: - DL TCI: Each of the M source reference signals (or 2M, if qcl_Type2 is configured in addition to qcl_Type1) in the M DL TCIs provides QCL information at least for one of the M beam pair links for UE-dedicated receptions on PDSCH and/or subset of CORESETs in a CC For N>1: - UL TCI: Each of the N source reference signals in the N UL TCIs provide a reference for determining UL TX spatial filter at least for one of the N beam pair links associated with dynamic-grant(s)/configured-grant(s) based PUSCH, and/or subset of dedicated PUCCH resources in a CC For M>1 and/or N>1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. In this case, M=N. - Separate DL/UL TCI: The M DL TCIs and N UL TCIs are distinct (therefore, separate). Note: Other TCI types/terms such as “common TCI” are not used.

According to the above, M/N>1 leads multiple TCI states for DL/UL and each TCI state may affect different target channels/RSs, e.g. different subset of CORESETs.

Rel-16/17 MTRP operation is for improving throughput and/or reliability. MTRP transmission schemes are categorized into two, i.e. 1) multiple DCI (M-DCI) based mTRP transmission schemes in which each TRP transmits a different DCI and 2) single DCI (S-DCI) based mTRP transmission schemes.

For M-DCI based MTRP transmission, it is operated based on CORESET pool index where each TRP independently schedules PDCCH/PUSCH/PUCCH by DCI associated with its CORESET pool index. Hence, each TRP/panel can be identified based on its CORESET pool index so that each TCI state for M=2 and/or N=2 can be associated to each CORESET pool for PDCCH. For channels/RSs other than PDCCH, target channels/RSs applying each TCI state can be differently configured by RRC.

Proposal #1: For M-DCI mTRP with M/N=2, each DL, UL or joint TCI state between the two is associated with CORESET pool index.

  - Each TCI state is indicated/updated via MAC-CE/DCI from the associated CORESET pool

  - For channels/RSs other than PDCCH, target channels for each TCI state can be configured differently by RRC

For S-DCI based mTRP transmission, by configuring M and/or N=2, the beam indication DCI with/without DL assignment and MAC-CE needs to indicate one or two TCI state(s). For example, on the MTRP repetition schemes, the indicated beams are adopted on the corresponding repetition in FDM/TDM manner, respectively. For SFN MTRP scheme, the indicated beams are simultaneously adopted on SFN PDCCH/PDSCH.

Proposal #2: For S-DCI mTRP with M/N=2, beam indication MAC-CE/DCI may update one or two TCI state(s).

  - For mTRP PDCCH/PDSCH/PUSCH/PUCCH repetition, each TCI state can correspond to transmission occasion(s) for one TRP/panel

  - For mTRP SFN PDCCH/PDSCH, each TCI state can correspond to one of the two TCI states to be applied for the PDCCH/PDSCH

In Rel-17, the common beam applicability for M=N=1 is to follow the indicated TCI state for DL/UL data and control channels (i.e. PDSCH/PDCCH/PUSCH/PUCCH). When M/N>1 is applied for S-DCI based MTRP operation in Rel-18, it is essential to clearly distinguish which channel/RS or transmission occasion would follow which TCI state among the multiple TCI states. For example, when M=N=2 is configured, one TCI state may only be applied to a specific DG/CG PUSCH transmission or a specific PUCCH resource group. In addition, it should also be possible that some DL/UL transmission is for mTRP, e.g. mTRP repetition, SFN, but some other transmission is for sTRP. Thus, the applicability of each or both TCI state(s) should be configurable.

Proposal #3: For M/N>1, applicability of each or both TCI state(s) should be configurable.

BM for simultaneous UL multi-panel transmission

As many operators concerned during Rel-18 MIMO scoping discussion in RAN plenary, major bottle-neck of commercialized NR network is UL performance. In order to obtain higher UL throughput, simultaneous UL multi-panel transmission (STxMP) can be regarded as a useful tool with the introduction of features for UL panel selection in Rel-17. As shown in [2], a significant cell edge throughput gain from STxMP can be observed compared to the UL transmission with 1-panel selection. In addition, according to LS from 5GAA [3], the STxMP is also related to vehicular distributed antenna systems considering several antenna configuration/location.

For the STxMP, UL beam should be determined on per panel basis. Rel-17 BM introduced an enhanced beam reporting mode in which UE reports CRI/SSBRI with UE capability value set ID but it cannot distinguish panels with the same number of antenna ports, i.e. homogeneous panels. Thus, gNB cannot acquire two UL spatial relation RSs where each spatial relation RS correspond to different UE panel. Another aspect for enhancement is the beam indication where only one TCI/TPMI/TRI/SRI is supported for PUSCH/PUCCH/SRS transmission currently. The design for N>1 TCI states discussed in the previous section could also be used for simultaneous UL multi-panel transmission where each UL/joint TCI state corresponds to each UE panel.

Proposal #4: Enhanced beam management for simultaneous UL multi-panel transmission should be supported.

  - Enhanced beam reporting via extending the usage of UE capability value set to homogeneous panels

  - Enhanced beam indication based on N>1 UL/joint TCI states, where each UL/joint TCI state corresponds to each UE panel

Regarding transmission schemes for STxMP, two schemes can be considered for S-DCI PUSCH. One is SLxMP (same-layer transmission from multi-panel) where same layer(s) is transmitted from multiple UE panels, and the other is DLxMP (different-layer transmission from multi-panel) where different layer is transmitted from different UE panel, where details of the schemes can be found in our companion paper [2]. For S-DCI PUCCH, only SLxMP would be applicable. For S-DCI SRS, both SLxMP and DLxMP can be considered, where different port can be transmitted from different panel.

Proposal #5: For S-DCI STxMP with N=2, the two TCI states can be used for SLxMP (same-layer transmission from multi-panel) and/or DLxMP (different-layer transmission from multi-panel) for PUSCH/PUCCH/SRS transmission.

In Rel-16, for M-DCI based MTRP PUSCH/PUCCH transmission TRP independently schedules its PUSCH/PUCCH in different symbol by DCI which is associated with its CORESET pool index. This legacy signaling framework can be simply extended to support M-DCI based simultaneous UL multi-panel transmission. The key difference would be to allow overlapping of PUCCH/PUSCH from different CORESET pool. Then, UL channels for each TRP is transmitted in TDM manner. For STxMP transmission, for example, one DCI associated with CORESET pool index 0 can be used to schedule PUSCH transmission for one UL panel and the other DCI associated with CORESET pool index 1 can be used to schedule PUSCH for the other panel with the respective TCI state indication.

Proposal #6: For M-DCI STxMP with N=2, multiple UL resources can be overlapped and each TCI state is indicated/updated on per CORESET pool basis.

Power control for UL S-DCI based MTRP

For power control (PC) aspects, the PC parameters including P0, alpha, closed-loop index, and path-loss reference RS (PL-RS) can be included in the TCI state where the PC parameters excluding PL-RS are channel/RS-specific and PL-RS is channel/RS-common. Considering N>1 in Rel-18, TRP/panel-specific UL PC parameters can be indicated by the common beam which can include PC parameters naturally.

Observation #1: For N>1, TRP/panel-specific PC parameters can be naturally supported by the indicated TCI states, where each TCI state includes respective PC parameter if we simply extend Rel-17 design.

For example, PC parameters included in the UL or joint TCI states can be naturally applied for S-DCI MTRP PUSCH/PUCCH transmission case. When N=2, the first and second TCI state in S-DCI can be associated with the specific PUSCH/PUCCH repetition transmission occasion, respectively. Meanwhile, for STxMP transmission, each TCI state including PC parameters can be associated with a certain UE Tx panel and UE transmits PUSCH/PUCCH by using two panels at the same time. As a result, the total transmit power through multiple UE panels can exceed UE Pc,max value. Hence, it should be further studied with transmit power management for STxMP transmission. To this end, consider per-panel power control for STxMP transmission by taking maximum Tx power of UE and panel-wise maximum power into account. Note that the maximum Tx power may be different depending on power amplifier configuration of each panel or maximum permissible exposure (MPE) issue on human body. Hence, power head room report (PHR) to gNB for the current power budget of UE seems to be more preferable by including virtual PH value and per panel Pc,max together.

Proposal #7: Consider per panel power control for STxMP transmission by taking maximum Tx power of UE and panel-wise maximum power into account.

Conclusion

In this contribution, we have discussed the potential enhancements on MTRP operation based on the extension of unified TCI framework. Based on the discussions above, following observation and proposals are given as:

Observation #1: For N>1, TRP/panel-specific PC parameters can be naturally supported by the indicated TCI states, where each TCI state includes respective PC parameter if we simply extend Rel-17 design.

Proposal #1: For M-DCI mTRP with M/N=2, each DL, UL or joint TCI state between the two is associated with CORESET pool index.

  - Each TCI state is indicated/updated via MAC-CE/DCI from the associated CORESET pool

  - For channels/RSs other than PDCCH, target channels for each TCI state can be configured differently by RRC

Proposal #2: For S-DCI mTRP with M/N=2, beam indication MAC-CE/DCI may update one or two TCI state(s).

  - For mTRP PDCCH/PDSCH/PUSCH/PUCCH repetition, each TCI state can correspond to transmission occasion(s) for one TRP/panel

  - For mTRP SFN PDCCH/PDSCH, each TCI state can correspond to one of the two TCI states to be applied for the PDCCH/PDSCH

Proposal #3: For M/N>1, applicability of each or both TCI state(s) should be configurable.

Proposal #4: Enhanced beam management for simultaneous UL multi-panel transmission should be supported.

  - Enhanced beam reporting via extending the usage of UE capability value set to homogeneous panels

  - Enhanced beam indication based on N>1 UL/joint TCI states, where each UL/joint TCI state corresponds to each UE panel

Proposal #5: For S-DCI STxMP with N=2, the two TCI states can be used for SLxMP (same-layer transmission from multi-panel) and/or DLxMP (different-layer transmission from multi-panel) for PUSCH/PUCCH/SRS transmission.

Proposal #6: For M-DCI STxMP with N=2, multiple UL resources can be overlapped and each TCI state is indicated/updated on per CORESET pool basis.

Proposal #7: Consider per panel power control for STxMP transmission by taking maximum Tx power of UE and panel-wise maximum power into account.

- References

[1] RP-213598	New WID: MIMO Evolution for Downlink and Uplink		Samsung

[2] R1-2204146	UL precoding indication for multi-panel transmission			LG Electronics

[3] R1-2205116	LS on Rel-18 WI related to vehicular distributed antenna systems	LG Electronics