3GPP TSG RAN WG1 #109-e	   	 	  	                	      R1-2204142

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.1.2

Source:	LG Electronics

Title:	Two TAs for multi-TRP/panel

Document for:	Discussion and Decision

Introduction

The approved WID [1] includes enhancements on MIMO for Rel-18, including multi-TRP enhancement, CSI enhancement, reference signal enhancement, and uplink transmission enhancement. Specifically for multi-TRP enhancement, there are for sub-topics, regarding unified TCI frame work extension, simultaneous multi-panel UL transmission, power control for UL single DCI, and two TAs for multi DCI, respectively.

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we discuss the enhancement on two TAs for multi-DCI based M-TRP operation.

Discussion

Use cases of two TAs

When it comes to different TA per TRP/panel, there are three factors to be considered.

- Difference of propagation delay between a UE and each TRP

- DL reference timing offset between TRPs

- Inter-UE-panel delay

Especially, UE distributed antenna system (DAS) implementation is being attractive in automotive industry. As being requested by LS from 5GAA [2], two TA feature would be useful for vehicle DAS UE. For vehicle DAS UE, each TA value could be managed by each distributed antenna group or panel, and this could be used for S-TRP deployment scenario as well as M-TRP since ‘TRP’ can be transparent to UE.

Proposal #1. Enhancements for two TA should take vehicle DAS UE implementation into account, where each TA value is managed by each distributed antenna group or panel.

Analysis on TRP/panel-specific UL timing synchronization

In numerous results, it has been shown that UL performance degradation can be significant if UL timing error exceeds 0.1us (i.e., about 22% of the CP length) [4][5]. It is obvious that if it exceeds the CP length, UL reception will not be completed successfully. Based on the fact, we show simulation results related to Rx timing offset between two TRPs for MTRP scheme in this contribution.

The following Table 1 shows a generalized formulation of Rx timing offset between two TRPs for the case that the same TA value is used for both TRPs, where τasync represents relative timing offset of TRP2 and τIPD represents relative inter-panel timing offset of the UE where each UE panel is assumed to be associated to each TRP. Based on RAN4 requirements, the possible largest value of τasync is 3us and that of τIPD is 130ns [6][7].

Table 1. Rx timing at each TRP with Tx timing asynchronization between TRPs and inter-panel delay of MPUE

|  | TRP1 | TRP2 |
| --- | --- | --- |
| TRP reference timing | T1 | T1+τasync |
| Propagation delay | RTT12 | RTT22 |
| UE Rx timing & TA reference timing | T1+RTT12 | T1+τasync+RTT22+τIPD |
| TRP Rx timing | TRx,TRP1=T1+RTT1-TA | TRx,TRP2=T1+τasync+RTT2+τIPD-TA |
| Rx timing offset between TRPs | Toffset=TARx,TRP1-TARx,TRP2=RTT1-RTT2-τasync-τIPD=2cd1-d2-c2τasync+τISD |  |

In Table 1, d1, d1 and c represent distance between UE and TRP1/2, and the speed of light, respectively. As described in Table 1, Rx timing offset between TRPs is determined by the difference in the distance between UE and each TRP, Tx timing offset between TRPs, and inter-panel delay of MPUE. If UE has to transmit UL signal based on one TA value, performance would be degraded due to such Rx timing offset between TRPs. Based on this formulation, we observed the ratio of UEs of which Rx timing offset Toffset exceeds the CP length via system level simulation. Simulation parameters are shown in Table 2. The parameters are set based on [8][9].

Table 2. Simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario | UMa, UMi - street canyon, InH-office |
| Carrier frequency | 2GHz, 30GHz, 60GHz |
| Channel model | TR38.901 |
| TRP antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2), (dH, dV) = (0.5, 0.8)λ for UMa, UMi 2GHz 2 ports: (8,8,2,1,1), (dH, dV) = (0.5, 0.8)λ for UMa 30GHz 2 ports: (8,4,2,1,1), (dH, dV) = (0.5, 0.8)λ for UMi 30GHz 2 ports: (4,8,2,1,1), (dH, dV) = (0.5, 0.5)λ for InH 60GHz |
| UE antenna configuration | 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for 2GHz 2 ports: (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ, Ω0,1=Ω0,0+180 for 30GHz 2 ports: (4, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ, Ω0,1=Ω0,0+180 for 60GHz |

Table 3-1. Portion of MTRP UEs exceeding CP length (UMa, 2GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 kHz | 4.68 us | 1.8 % (3.7 %) | 1.9 % (3.9 %) | 2.9 % (5.9 %) |
| 1 | 30 kHz | 2.34 us | 7.8 % (16.1 %) | 7.8 % (16.1 %) | 15.7 % (32.4 %) |
| 2 | 60 kHz | 1.17 us | 13.2 % (27.2 %) | 13.2 % (27.2 %) | 31.7 % (65.4 %) |

Table 3-2. Portion of MTRP UEs exceeding CP length (UMa, 30GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 2 | 60 kHz | 1.17 us | 13.0 % (38.4 %) | 13.0 % (38.4 %) | 22.4 % (66.2 %) |
| 3 | 120 kHz | 585.93 ns | 18.8 % (55.6 %) | 18.9 % (55.9%) | 28.1 % (83.1 %) |

Table 3-3. Portion of MTRP UEs exceeding CP length (UMa, 30GHz, with blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 2 | 60 kHz | 1.17 us | 20.5 % (46.3 %) | 20.5 % (46.3 %) | 29.3 % (66.2%) |
| 3 | 120 kHz | 585.93 ns | 27.3 % (61.7 %) | 27.4 % (61.9 %) | 36.7 % (83.0%) |

Table 3-1, 3-2, and 3-3 show simulation results in UMa scenario. In the simulation, we assume that MTRP transmission is done only for the UEs that the RSRP difference between two TRPs having the best RSRP is smaller than 6dB for the case without blockage or 10dB for the case with blockage, where blockage model A in TR38.901 is used, respectively. For other UEs, STRP transmission is assumed. We observed the portion of MTRP UEs of which timing offset exceeds the CP length among all UEs (x%) or among all MTRP UEs (y%) in the deployment, where both x and y are present in the tables as ‘x % (y %)’. In the tables, the impact of timing offset/error of TRP or UE side is also observed, where the term τasync + τIPD in Table 1 is randomly generated with uniform distribution in [-τMAX, τMAX]. Three values of τMAX were tested, i.e. ideal timing offset (τMAX =0), inter-panel delay at UE side (τMAX =130ns), and inter-TRP asynch scenario (τMAX =3us).

In Table 3-1, the ratio of MTRP UEs was 48.4%, meaning that 51.6% are STRP UEs. From the table, we can observe that timing offset exceeds the CP length for 7.8% of UEs and 13.2% of UEs in 30kHz and 60kHz numerology in the case of ideal timing offset(τMAX =0), respectively, which can also be interpreted that 16.1% and 27.2% of UEs among MTRP UEs. And, these values are more than doubled for inter-TRP asynch scenario (τMAX =3us).

Observation #1: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs is < 6dB and Toffset>CP length is 7.8% and 13.2%, respectively, in the 30kHz and 60kHz numerology in 2GHz UMa scenario when no timing offset is assumed between TRPs and between UE panels. In case of τMAX = 3us, these ratios increase to 15.7% and 31.7% at 30kHz and 60kHz, respectively.

In Table 3-2, the ratio of MTRP UEs was 33.8%. From the table, we can observe that timing offset exceeds the CP length for 13.0% of UEs and 18.8% of UEs in 60kHz and 120kHz numerology in the case of ideal timing offset(τMAX =0), respectively. In Table 3-3, considering the blockage, i.e., up to 10dB RSRP difference, the ratio of MTRP UEs increases to 44.2%. And, we can observe that timing offset exceeds the CP length for 20.5% of UEs and 27.3% of UEs in 60kHz and 120kHz numerology in the case of ideal timing offset(τMAX =0), respectively.

Observation #2: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs < 6dB and Toffset>CP length is 13.0% and 18.8%, respectively, in the 60kHz and 120kHz numerology in 30GHz UMa scenario when no timing offset is assumed between TRPs and between UE panels. If blockage and RSRP difference of up to 10dB is considered, the ratio of UEs increases to 20.5% and 27.3% at 60kHz and 120kHz, respectively.

Table 4-1. Portion of MTRP UEs exceeding CP length (UMi, 2GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 kHz | 4.68 us | 0 % (0 %) | 0 % (0 %) | 0.2 % (0.3 %) |
| 1 | 30 kHz | 2.34 us | 0.5 % (0.9 %) | 0.6 % (0.9 %) | 14.1 % (25.5 %) |
| 2 | 60 kHz | 1.17 us | 9.0 % (16.3 %) | 9.0 % (16.3 %) | 33.9 % (61.4 %) |

Table 4-2. Portion of MTRP UEs exceeding CP length (UMi, 30GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 2 | 60 kHz | 1.17 us | 7.9 % (26.6 %) | 7.9 % (26.6 %) | 18.1 % (61.1 %) |
| 3 | 120 kHz | 585.93 ns | 15.1 % (51.0 %) | 15.1 % (51.0 %) | 23.7 % (80.0 %) |

Table 4-3. Portion of MTRP UEs exceeding CP length (UMi, 30GHz, with blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 2 | 60 kHz | 1.17 us | 11.2 % (27.9 %) | 11.2 % (27.9 %) | 24.5 % (61.0 %) |
| 3 | 120 kHz | 585.93 ns | 20.9 % (52.1 %) | 20.9 % (52.1 %) | 32.3 % (80.5 %) |

Table 4-1, 4-2, and 4-3 show simulation results in UMi scenario. In Table 4-1, the ratio of MTRP UEs was 55.2%. From the table, we can observe that timing offset exceeds the CP length for 9.0% of UEs in 60kHz numerology in the case of ideal timing offset(τMAX =0), which can also be interpreted that 16.3% of UEs among MTRP UEs. And, this value increases more than three times for inter-TRP asynch scenario (τMAX =3us).

Observation #3: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs is < 6dB and Toffset>CP length is 9.0% in the 60kHz numerology in 2GHz UMi scenario when no timing offset is assumed between TRPs and between UE panels. In case of τMAX = 3us, this ratio increases to 33.9% at 60kHz.

In Table 4-2, the ratio of MTRP UEs was 29.6%. From the table, we can observe that timing offset exceeds the CP length for 7.9% of UEs and 15.1% of UEs in 60kHz and 120kHz numerology in the case of ideal timing offset(τMAX =0), respectively. In Table 4-3, considering the blockage, i.e., up to 10dB RSRP difference, the ratio of MTRP UEs increases to 40.1%. And, we can observe that timing offset exceeds the CP length for 11.2% of UEs and 20.9% of UEs in 60kHz and 120kHz numerology in the case of ideal timing offset(τMAX =0), respectively.

Observation #4: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs < 6dB and Toffset>CP length is 7.9% and 15.1%, respectively, in the 60kHz and 120kHz numerology in 30GHz UMi scenario when no timing offset is assumed between TRPs and between UE panels. If blockage and RSRP difference of up to 10dB is considered, the ratio of UEs increases to 11.2% and 20.9% at 60kHz and 120kHz, respectively.

Table 5-1. Portion of MTRP UEs exceeding CP length (InH, 60GHz, without blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 5 | 480 kHz | 146.48 ns | 5.8 % (19.7 %) | 8.6 % (29.2 %) | 28.1 % (95.5 %) |
| 6 | 960 kHz | 73.24 ns | 14.9 % (50.6 %) | 17.9 % (60.8 %) | 28.8 % (97.9%) |

Table 5-2. Portion of MTRP UEs exceeding CP length (InH, 60GHz, with blockage)

| μ | SCS | CP length | τMAX=0ns | τMAX=130ns | τMAX=3us |
| --- | --- | --- | --- | --- | --- |
| 5 | 480 kHz | 146.48 ns | 9.3 % (23.9 %) | 12.6 % (32.3 %) | 37.2 % (95.6 %) |
| 6 | 960 kHz | 73.24 ns | 21.1 % (54.2 %) | 24.0 % (61.6 %) | 38.0 % (97.6 %) |

Table 5-1 and 5-2 show simulation results in InH scenario. In Table 5-1, the ratio of MTRP UEs was 29.4%. From the table, we can observe that timing offset exceeds the CP length for 5.8% of UEs and 14.9% of UEs in 480kHz and 960kHz numerology in the case of ideal timing offset(τMAX =0), respectively, which can also be interpreted that 19.7% and 50.6% of UEs among MTRP UEs. And, these values increase to 8.6% and 17.9% if inter-panel delay at UE side is considered (τMAX =130ns). In Table 5-2, considering the blockage, i.e., up to 10dB RSRP difference, the ratio of MTRP UEs increases to 38.9%. And, we can observe that timing offset exceeds the CP length for 9.3% of UEs and 21.1% of UEs in 480kHz and 960kHz numerology in the case of ideal timing offset(τMAX =0), respectively. And, these values increase to 12.6% and 24.0% if inter-panel delay at UE side is considered (τMAX =130ns).

Observation #5: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs is < 6dB and Toffset>CP length is 5.8% and 14.9%, respectively, in the 480kHz and 960kHz numerology in 60GHz InH scenario when no timing offset is assumed between TRPs and between UE panels. In case of τMAX = 130ns, these ratios increase to 8.6% and 17.9% at 480kHz and 960kHz, respectively. If blockage and RSRP difference of up to 10dB is considered, the ratio of UEs increases to 9.3% and 21.1% at 480kHz and 960kHz, respectively.

Based on the above simulation results, the ratio of UEs of which Rx timing offset Toffset exceeds the CP length is not negligible in various network deployment scenarios. The ratio becomes larger for higher carrier frequency, higher inter-TRP backhaul delay, and/or higher inter-UE-panel delay. It should also be noted that the ratio would become much larger if we observe the ratio of UEs of which Rx timing offset Toffset exceeds 20% of the CP length, where UL performance would be highly degraded based on analysis of many different literatures.

Proposal #2: Two TAs should be supported for multi-TRP/panel use cases considering inter-TRP/panel timing offset in FR1 as well as FR2-1/2-2.

Configuration of two TAs for multi-DCI mTRP

In Rel-17 beam management enhancement, unified TCI framework for DL/UL beam indication had been specified. In unified TCI, dynamic common beam indication with M/N=1 for DL/UL channel/RSs is supported, wherein power control parameters including P0, alpha, closed-loop index, and pathloss reference RS can be included in the TCI state. In Rel-18 extension of unified TCI discussion, M/N>1 considering M-TRP DL/UL transmission is to be introduced in order to support TRP/panel-specific DL/UL common beam indication. When N>1 is supported, TRP/panel-specific UL PC parameters can be naturally indicated by TRP/panel-specific common beam which can include power control parameters. Moreover, as discussed in Rel-17 beam management enhancement, if two TAs are supported, TRP/panel-specific TA value can also be included in or associated with UL/joint TCI state to support TRP/panel-specific DL/UL common beam indication with different TA. To be specific, for N=2, each UL/joint TCI state can be associated with a specific CORESET pool index, so each TA value can be managed per TRP or UE panel.

Observation #6: For enhanced TCI framework with N>1, TRP/panel-specific PC parameters can be naturally supported since each TCI state can include respective UL PC parameters such as P0, Alpha, closed-loop index, and PL-RS.

Proposal #3: Support UL/joint TCI state to include UL TA value related parameter similarly as UL PC parameters, in order to support TRP/panel-specific PC/TA configuration.

Proposal #4: Each TCI of M/N>1 can be associated with CORESET pool index in case of M-DCI based M-TRP operation.

If two TAs for M-DCI are supported, it should be investigated on how to measure the two TAs for M-TRP uplink transmission. In legacy behavior, PRACH is generally used for UL TA measurement and RAR MAC CE and/or TA command MAC CE can be used to configure/update UL TA per TAG. First, PRACH could be enhanced to support TRP/panel-specific PRACH transmission. As mentioned above, each of two TAs can be associated with a specific CORESET pool index. Then, if association between CORESET pool index and PRACH is introduced, PRACH transmission with a TRP/panel specific TA related with the specific CORESET pool index can be supported for gNB to measure and configure TRP/panel-specific TA. After all, RAR MAC CE and/or TA command MAC CE can be enhanced to configure TRP/panel-specific TA (per TAG).

Proposal #5: Support RACH enhancement for measuring timing offset per TRP/panel and for indicating TA value per TRP/panel.

Conclusion

In this contribution, we have discussed the enhancement on two TAs for multi-DCI based M-TRP operation, and provided the following observations and proposals.

Proposal #1. Enhancements for two TA should take vehicle DAS UE implementation into account, where each TA value is managed by each distributed antenna group or panel.

Observation #1: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs is < 6dB and Toffset>CP length is 7.8% and 13.2%, respectively, in the 30kHz and 60kHz numerology in 2GHz UMa scenario when no timing offset is assumed between TRPs and between UE panels. In case of τMAX = 3us, these ratios increase to 15.7% and 31.7% at 30kHz and 60kHz, respectively.

Observation #2: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs < 6dB and Toffset>CP length is 13.0% and 18.8%, respectively, in the 60kHz and 120kHz numerology in 30GHz UMa scenario when no timing offset is assumed between TRPs and between UE panels. If blockage and RSRP difference of up to 10dB is considered, the ratio of UEs increases to 20.5% and 27.3% at 60kHz and 120kHz, respectively.

Observation #3: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs is < 6dB and Toffset>CP length is 9.0% in the 60kHz numerology in 2GHz UMi scenario when no timing offset is assumed between TRPs and between UE panels. In case of τMAX = 3us, this ratio increases to 33.9% at 60kHz.

Observation #4: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs < 6dB and Toffset>CP length is 7.9% and 15.1%, respectively, in the 60kHz and 120kHz numerology in 30GHz UMi scenario when no timing offset is assumed between TRPs and between UE panels. If blockage and RSRP difference of up to 10dB is considered, the ratio of UEs increases to 11.2% and 20.9% at 60kHz and 120kHz, respectively.

Observation #5: The ratio of UEs corresponding to the case that the RSRP difference of the best two TRPs is < 6dB and Toffset>CP length is 5.8% and 14.9%, respectively, in the 480kHz and 960kHz numerology in 60GHz InH scenario when no timing offset is assumed between TRPs and between UE panels. In case of τMAX = 130ns, these ratios increase to 8.6% and 17.9% at 480kHz and 960kHz, respectively. If blockage and RSRP difference of up to 10dB is considered, the ratio of UEs increases to 9.3% and 21.1% at 480kHz and 960kHz, respectively.

Proposal #2: Two TAs should be supported for multi-TRP/panel use cases considering inter-TRP/panel timing offset in FR1 as well as FR2-1/2-2.

Observation #6: For enhanced TCI framework with N>1, TRP/panel-specific PC parameters can be naturally supported since each TCI state can include respective UL PC parameters such as P0, Alpha, closed-loop index, and PL-RS.

Proposal #3: Support UL/joint TCI state to include UL TA value related parameter similarly as UL PC parameters, in order to support TRP/panel-specific PC/TA configuration.

Proposal #4: Each TCI of M/N>1 can be associated with CORESET pool index in case of M-DCI based M-TRP operation.

Proposal #5: Support RACH enhancement for measuring timing offset per TRP/panel and for indicating TA value per TRP/panel.

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink

[2] R1-2205116(S-220026), LS on Rel-18 WI related to vehicular distributed antenna systems, 5GAA WG4, LG Electronics

[3] R1-2103511, Analysis on TRP- and/or UE panel-specific UL timing synchronization, LG Electronics

[4] R1-062834, UL Timing Control Accuracy and Update Rate, Nokia

[5] R1-071479, Simulation of Uplink Timing Error Impact on PUSCH, Texas Instruments

[6] TS 38.133, Requirements for support of radio resource management

[7] TS 38.101-1, User Equipment (UE) radio transmission and reception

[8] “3GPP RAN1 #94bis Chairman’s Notes”, October 8th – 12th, 2018

[9] TR 38.808, Study on supporting NR from 52.6 GHz to 71 GHz