3GPP TSG RAN WG1 #109-e                                     R1-2204145

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.1.3.2

Source:		LG Electronics

Title:			SRS enhancement targeting TDD CJT and 8 TX operation

Document for:	Discussion and Decision

Introduction

In the scope of Rel-18 MIMO enhancement, two aspects for SRS enhancement are included as follows [1].

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

The first topic is about SRS capacity and/or interference randomization enhancement targeting TDD CJT and the other is about 8 TX operation. In this contribution, we provide our view on the two topics for SRS enhancement.

Discussion

Enhancement for TDD CJT

In TDD system, gNB may acquire the precoding matrix for DL transmission based on SRS from a UE. For this purpose, gNB may configure the UE with SRS resource set in which usage is configured as antennSwitching. From the UL measurement from SRS, gNB may determine DL precoder and can transmit precoded CSI-RS(s) for  RI/CQI report from the UE, e.g., based on CSI-ReportConfig in which reportQuantity is configured as cri-RI-CQI. Consequently, gNB can perform the appropriate data scheduling for the UE based on the precoding matrix and the reported CSI. Above procedure can well be used for the acquisition of the CSI acquisition in single or multi-TRP scenario. From SRS transmission perspective from UE side, ‘TRP’ would be transparent. It only requires information related to UL power control and spatial relation RS (for BM in FR2) for the SRS and doesn’t need to know which TRP would receive which SRS. In this regard, it is unclear which enhancement is exactly needed for CJT inter-TRP cross-SRS interference mitigation, management or measurement.

Proposal #1: For CJT mTRP cross-SRS interference, the work scope needs to be further clarified, e.g. whether it is for SRS interference mitigation/randomization, SRS interference management/coordination, or inter-TRP interference measurement by SRS.

For the first aspect, i.e. SRS interference mitigation/randomization, current specification supports quasi-orthogonal SRS by configurable sequence initialization (i.e. virtual cell ID) per SRS resource. So, it should be clarified which aspect needs to be enhanced and why current specification is not sufficient.

Proposal #2: For SRS interference mitigation/randomization, any proposed scheme needs to be compared with interference mitigation/randomization methods supported by current specification, e.g. virtual cell ID based quasi-orthogonal SRS sequence.

For the second aspect, i.e. SRS interference management/coordination, it is about SRS capacity enhancement to our understanding because CJT can only be performed in ideal backhaul case, where tight coordination in SRS resource allocation between TRPs is possible. If specification provides sufficient number of orthogonal SRS resources, each TRP can assign orthogonal SRS for CJT. Thus, the only aspect that might cause a problem for UL/DL channel estimation from SRS is the deficiency of the number of orthogonal SRS resources. During the Rel-17 SRS enhancement discussion, many schemes for SRS capacity enhancement were discussed. In Rel-17, enhanced schemes such as larger comb value and partial frequency sounding were introduced as a result of the discussion. If a UE supports Rel-17 SRS transmission, it can be used for single TRP or multi-TRP transmission in TDD system in a UE transparent manner. So, the need of any further capacity enhancement is not clear.

Proposal #3: For SRS interference management/coordination, ideal backhaul between TRPs should be assumed and Rel-17 SRS should be a baseline for performance comparison.

For the third aspect, i.e. inter-TRP interference measurement by SRS, it is possible for network to configure/trigger different two SRS resource sets with usage of “codebook” or “nonCodebook” for UL channel acquisition, one for each TRP with corresponding UL PC, spatial relation RS configured, by current specification. In addition, it is also possible to receive a same SRS resource by multiple TRPs by network implementation. Another aspect to consider is that DL interference channel and UL interference channel cannot be assumed identical even in TDD due to different DL and UL transmission from same/different TRP(s) and cell(s).

Proposal #4: For inter-TRP interference measurement by SRS, different DL and UL interference channel condition needs to be taken into account.

Enhancement for 8 TX operation

Based on the current specification, 1, 2, or 4 port SRS transmission is possible. As a result, maximum 4 layer can be supported in UL transmission. Considering CPE/FWA/vehicle/industrial devices which can equip more than 4 antenna ports and are relatively less critical to power consumption issue compared to handheld devices, 8 port SRS transmission should be supported for more than 4 layer UL transmission which can improve UL performance significantly, which was the key motivation of Rel-18 MIMO enhancement.

Proposal #5: Support 8 port SRS transmission.

- Codebook based UL transmission

For SRS resources in a SRS resource set in which usage is configured as codebook, two multiplexing approaches can be considered. One is to use time domain multiplexing such as TDM or TD-OCC, and the other is to use frequency domain multiplexing such as CS with FDM.

| Approach | Pros & Cons |
| --- | --- |
| T-domain (TDM or TD-OCC) | Pros: Higher port density than F-domain approaches Cons: Vulnerable to the impairment such as phase coherency or phase noise as well as large Doppler spread channel |
| F-domain (CS with FDM) | Pros: Resistant to the impairment such as phase coherency or phase noise as well as large Doppler spread channel Cons: Lower port density, vulnerable to the large delay spread channel. |

In addition to above, the performance of port multiplexing based on TDM or TD-OCC could be more sensitive to channel variation between adjacent symbols compared to F-domain approaches so this would be related to target channel environment of 8Tx operation. For TD-OCC, multi-symbol based sequence/group hopping may be needed to prevent hopping between two symbols of same port. Based on the above analysis, we provide the following proposal.

Proposal #6: Consider TDM/TD-OCC and/or CS with FDM for 8 port SRS transmission for SRS with usage = codebook.

- Non-codebook based UL transmission

In a SRS resource set in which usage is configured as nonCodebook, maximum 4 SRS resources can be configured in the current specification. And maximum 4 SRS resources can be indicated for 4 layer UL transmission. Several schemes to support more than 4 layers and details of SRI indication for non-codebook based UL transmission are described in our companion contribution [2]. For enabling 8 TX non-codebook based UL transmission, our proposal is as follows.

Proposal #7: Consider following alternatives for enabling 8 TX non-codebook based UL transmission.

- Alt1. Increase the number of SRS resources from 4 to 8

- Alt2. Allow maximum two ports per SRS resource

- Alt3. Reuse Rel-17 S-DCI based M-TRP PUSCH mechanism

- Antenna switching

In a SRS resource set in which usage is configured as antennSwitching, the following antenna configuration are supported, i.e., "1T=1R", "1T2R", "1T4R", "1T6R", "1T8R", "2T=2R", "2T4R", "2T6R", "2T8R", "4T=4R", and "4T8R". So, at least 8T8R configuration should be supported for 8 TX operation. For this purpose, a SRS resource set with one SRS resource with 8 port can be configured for 8T8R configuration.

Proposal #8: Support 8T8R configuration for SRS with usage = antennaSwitching.

Conclusion

In this contribution, we discuss about SRS enhancement targeting TDD CJT and 8 TX operation. We provide the following proposals.

Proposal #1: For CJT mTRP cross-SRS interference, the work scope needs to be further clarified, e.g. whether it is for SRS interference mitigation/randomization, SRS interference management/coordination, or inter-TRP interference measurement by SRS.

Proposal #2: For SRS interference mitigation/randomization, any proposed scheme needs to be compared with interference mitigation/randomization methods supported by current specification, e.g. virtual cell ID based quasi-orthogonal SRS sequence.

Proposal #3: For SRS interference management/coordination, ideal backhaul between TRPs should be assumed and Rel-17 SRS should be a baseline for performance comparison.

Proposal #4: For inter-TRP interference measurement by SRS, different DL and UL interference channel condition needs to be taken into account.

Proposal #5: Support 8 port SRS transmission.

Proposal #6: Consider TDM/TD-OCC and/or CS with FDM for 8 port SRS transmission for SRS with usage = codebook.

Proposal #7: Consider following alternatives for enabling 8 TX non-codebook based UL transmission.

- Alt1. Increase the number of SRS resources from 4 to 8

- Alt2. Allow maximum two ports per SRS resource

- Alt3. Reuse Rel-17 S-DCI based M-TRP PUSCH mechanism

Proposal #8: Support 8T8R configuration for SRS with usage = antennaSwitching.

Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung

[2] R1-2204147, SRI/TPMI enhancement for enabling 8 TX UL transmission, LG Electronics