3GPP TSG RAN WG1 #109-e	                          			      R1-2204146

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.4.1

Source:	LG Electronics

Title:	UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss UL precoding indication for multi-panel transmission based on S-DCI mTRP PUSCH framework and based on M-DCI mTRP PUSCH framework, separately. Specifically, we discuss how to reuse S-DCI/M-DCI based mTRP PUSCH framework to schedule STxMP PUSCH, the limitation of the legacy framework, and potential candidates of STxMP transmission scheme.

Discussion

According to LS from 5GAA [1], STxMP enhancement and multiple TA enhancement for mTRP operation among Rel-18 MIMO WI objectives are relevant to vehicular distributed antenna systems and 5GAA WG4 asks 3GPP RAN WG1 to consider 5GAA technical report on vehicular distributed antenna systems in Rel-18 MIMO WI. Given that FR 1 providing better coverage has more popular industrial demand for vehicle communication and antenna units are distributed in several meters, unlike conventional mobile device, these aspects for vehicle communication should be taken into account as well in MIMO WI in order to take advantage of the relative MIMO enhancement in vehicular DAS.

Proposal 1: Considering vehicular DAS use case, STxMP enhancement should be supported in both FR 1 and FR 2, and supported for widely distributed panels as well.

STxMP based on S-DCI mTRP PUSCH framework

In Rel-17, S-DCI based mTRP PUSCH repetition is introduced. In order to support this feature, many scheduling fields and parameters such as SRI, TPMI, TPC, PTRS, SRS resource set, and PUSCH power control parameters are extended in TRP specific manner and SRS resource set indicator field is also introduced. Even though STxMP and mTRP PUSCH repetition are different transmission schemes, the precoder indication mechanism designed for mTRP PUSCH can be a starting point for STxMP precoder indication because two features have similarity from signaling perspective. Therefore, these fields and parameters for mTRP PUSCH repetition can be reused or slightly modified to support STxMP without starting from the scratch.

For example, two TPMI fields are already supported in a single DCI so each TPMI field can be used to indicate RI/PMI for each panel. In Rel-17, in order to minimize TPMI field size increase, second TPMI field is designed without rank information, which is reasonable given that same rank is applied for the PUSCH repetition. However, when it comes to STxMP, this restriction is not needed and it may reduce the performance of STxMP transmission. So, we can consider to use two TPMI fields without dependency each other unlike Rel-17.

Observation 1: Many scheduling fields and parameters to support Rel-17 mTRP PUSCH repetition such as 2 SRI fields, 2 TPMI fields, 2 TPC fields, 2 PTRS fields, 2 SRS resource sets, 2 PUSCH power control parameters, and SRS resource set indicator field can be reused or slightly modified to support STxMP.

Observation 2: Two TPMI fields are already supported for single DCI mTRP PUSCH repetition but second TPMI field does not include rank information due to the same rank restriction.

STxMP is simultaneous transmission across multi-panel by definition but there are multiple transmission schemes that can be considered with STxMP. Depending on whether transmission layer from multi-panel is the same or different, two different STxMP transmission schemes can be considered; SLxMP (Same Layer across Multi-Panel) and DLxMP (Different Layer across Multi-Panel), respectively.

In SLxMP case, the same layers are transmitted from multi-panel. It can be considered as PUSCH repetition but, unlike Rel-17 using different TO, two repetitions are transmitted at the same time but from different panel with potentially different Tx beams. Since each PUSCH repetition is transmitted toward different TRP, macro diversity gain can be achieved even in blockage channel environment and latency can be more reduced compared to conventional mTRP PUSCH repetition. In addition, it may also be considered to indicate a co-phasing parameter for coherent SLxMP transmission to improve combining performance further.

On the other hand, in DLxMP case, different layers of PUSCH are transmitted from multi-panel. It can be considered as an uplink version of S-DCI based DL NCJT. For example, the first layer of rank 2 PUSCH is transmitted from panel 0 to TRP 0 and the second layer is transmitted from panel 1 to TRP 1 at the same time. Given that total transmission rank increases by using two different UL channels, throughput gain can be expected.

In our view, the usage of SLxMP and DLxMP are different and both are very useful.

Proposal 2: Support SLxMP (Same Layer across Multi-Panel) for URLLC and for cell edge UE performance enhancement and DLxMP (Different Layer across Multi-Panel) for eMBB as S-DCI based STxMP transmission schemes.

STxMP based on M-DCI mTRP PUSCH framework

In Rel-16, M-DCI based mTRP PUSCH transmission is introduced. Specifically, each TRP independently schedules its own PUSCH in different symbol with DCI associated with its CORESET pool index. Since gNB should avoid time domain overlapping between different TRP’s PUSCH/PUCCH, different TRP’s UL channels are transmitted in TDM manner. Even if multiple TRPs use separate DCI, the same UL scheduling parameters are shared such as SRS resource set, PUSCH power control parameters and codebook configuration without TRP specific extension. As a result, it has a limitation on TRP specific scheduling flexibility.

For STxMP transmission, M-DCI mTRP PUSCH framework also can be reused or slightly modified. For example, one DCI associated with CORESET pool index 0 can be used to schedule PUSCH transmission for panel 0 and another DCI associated with CORESET pool index 1 can be used to schedule PUSCH transmission for panel 1.

Observation 3: M-DCI mTRP PUSCH framework, in which DCI can be separated based on associated CORESET pool index, can be reused or slightly modified to support STxMP.

However, in order to support simultaneous transmission, unlike Rel-16 M-DCI mTRP PUSCH, scheduling restriction to avoid time domain overlapping should be removed. In this way, two PUSCH transmitted from different panels can be time domain overlapped or not. One issue related to the time domain overlapping is that, if the two PUSCH are partial overlapped in time, symbol level PUSCH power control may be needed, which increases UE complexity. For example, if symbol 0 and 1 are allocated for PUSCH from panel 0 and symbol 1 and 2 are allocated for PUSCH from panel 1, PUSCH transmission power can be changed symbol by symbol. Meanwhile, in case of fully overlapped PUSCHs, there is no such issue.

Observation 4: If STxMP PUSCHs are partially overlapped in time domain, symbol level PUSCH power control is needed, which increases UE complexity.

Proposal 3: In order to avoid symbol level PUSCH power control, STxMP PUSCHs should be fully overlapped in time domain.

Even though different TPMI can be indicated for different panel by using separate DCI, different codebook configuration, such as max rank, codebook subset restriction, full power mode and so on, cannot be applied for different panel according to Rel-16 M-DCI based mTRP PUSCH framework since CORESET pools share the same PUSCH-Config IE. However, in practice, two panels can be implemented with different antenna configuration, antenna coherency characteristic and PA combination. Taking this panel specific implementation into account, separate codebook configuration should be supported for multi-panel.

Proposal 4: Support separate codebook configuration for different Tx panels.

Regarding STxMP transmission scheme based on M-DCI mTRP PUSCH framework, each panel transmits separate PUSCH containing different TB, unlike S-DCI based STxMP which transmits the same TB as we described in Section 2.1. Given that M-DCI based mTRP transmission is introduced mainly for non-ideal backhaul network, in which dynamic coordination between TRPs are not available, different TB transmission for multi-panel seems a natural candidate STxMP transmission scheme under M-DCI framework.

Proposal 5: Consider DTxMP (Different TB across Multi-Panel) for eMBB as a potential candidate of M-DCI based STxMP transmission scheme.

Evaluation results for SLxMP

With evaluation assumption in Table 2, Figure 1 shows RSRP difference between the strongest and the second strongest UE panels and between the strongest and the third strongest UE panels, respectively, when UE has 3 panels. From the figure, RSRP difference between the strongest and the second strongest UE panels is less than 4.2dB in urban macro and 8.8dB in indoor hotspot scenarios, respectively, for half of UEs. It means that, for majority UEs, the UL channel quality from the second strongest UE panel is not significantly degraded compared to the strongest UE panel so that using STxMP based on two strong panels can provide a potential gain.

Figure 1. RSRP difference between multi-panel in dense urban macro and indoor hotspot.

Observation 5: RSRP difference between the strongest and the second strongest UE panels are less than 4.2dB in urban macro and 8.8dB in indoor hotspot scenarios, respectively, for half of UEs

We also provide SLS evaluation results on the SLxMP in Table 1 and evaluation assumptions are summarized in Table 2. For 2-panel STxMP, two strongest panels among three UE Tx panels are selected. Table 1 shows system level throughput results and the gains (cell edge, average) of UL 2-panel STxMP over UL 1-panel selection transmission. Prominent cell edge throughput gain is observed from system level evaluation, which means that STxMP brings proper improvement on link gain aspect especially for poor channel quality UE. We should note that this gain is expected to be increased for 4 panel UE since the RSRP difference of two best panels selected from 4 panels would become smaller than the three panel case.

Table 1. System level throughput results in dense urban macro scenario

|  | Cell edge UE throughput [Mbps] | Average UE throughput [Mbps] |
| --- | --- | --- |
| 1-panel selection (baseline) | 0.6 (100%) | 25.9 (100%) |
| 2-panel STxMP (w/ strongest two panels) | 1.3 (198.0%) | 26.7 (103.1%) |

Observation 6: In SLS evaluation, significant cell edge throughput gain (98%) of 2-panel SLxMP over the best 1-panel selection based UL transmission (as baseline) is observed for 3 panel UE.

Table 2. Evaluation assumptions

Conclusion

In this contribution, we discuss we discuss UL precoding indication for multi-panel transmission based on S-DCI mTRP PUSCH framework and based on M-DCI mTRP PUSCH framework, separately, and propose the followings:

Proposal 1: Considering vehicular DAS use case, STxMP enhancement should be supported in both FR 1 and FR 2, and supported for widely distributed panels as well.

STxMP based on S-DCI mTRP PUSCH framework:

Observation 1: Many scheduling fields and parameters to support Rel-17 mTRP PUSCH repetition such as 2 SRI fields, 2 TPMI fields, 2 TPC fields, 2 PTRS fields, 2 SRS resource sets, 2 PUSCH power control parameters, and SRS resource set indicator field can be reused or slightly modified to support STxMP.

Observation 2: Two TPMI fields are already supported for single DCI mTRP PUSCH repetition but second TPMI field does not include rank information due to the same rank restriction.

Proposal 2: Support SLxMP (Same Layer across Multi-Panel) for URLLC and for cell edge UE performance enhancement and DLxMP (Different Layer across Multi-Panel) for eMBB as S-DCI based STxMP transmission schemes.

STxMP based on M-DCI mTRP PUSCH framework:

Observation 3: M-DCI mTRP PUSCH framework, in which DCI can be separated based on associated CORESET pool index, can be reused or slightly modified to support STxMP.

Observation 4: If STxMP PUSCHs are partially overlapped in time domain, symbol level PUSCH power control is needed, which increases UE complexity.

Proposal 3: In order to avoid symbol level PUSCH power control, STxMP PUSCHs should be fully overlapped in time domain.

Proposal 4: Support separate codebook configuration for different Tx panels.

Proposal 5: Consider DTxMP (Different TB across Multi-Panel) for eMBB as a potential candidate of M-DCI based STxMP transmission scheme.

Observations from initial evaluation for same layer STxMP:

Observation 5: RSRP difference between the strongest and the second strongest UE panels are less than 4.2dB in urban macro and 8.8dB in indoor hotspot scenarios, respectively, for half of UEs

Observation 6: In SLS evaluation, significant cell edge throughput gain (98%) of 2-panel SLxMP over the best 1-panel selection based UL transmission (as baseline) is observed for 3 panel UE.

# Reference

[1] R1-2205116	LS on Rel-18 WI related to vehicular distributed antenna systems	LG Electronics