3GPP TSG RAN WG1 #109-e                                     R1-2204151

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.2.3.1

Source:		LG Electronics

Title:			Evaluation on AI/ML for beam management

Document for:	Discussion and Decision

Introduction

In the SID [1], following use cases are introduced for AI/ML for NR air interface. Especially for beam management, sub-use cases are described as beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement.

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, we discuss on evaluation methodology and KPI for AI/ML based beam management.

Discussions on evaluation methodology and KPI

Beam prediction in time domain for overhead and latency reduction

From RAN1 perspective, beam prediction in time domain can be an appropriate sub-use case especially when UE mobility is predictable from gNB side, which can save UE beam reporting overhead and reduce the delay of beam measurement and reporting at UE side. In this sub-use case, less configured candidate of DL/UL beam resources (e.g., SSB or CSI-RS) is used for the beam measurement and the AI/ML can help to predict the full granularity of beams and determine proper DL/UL beams. Therefore, it can save overhead of beam resources from both gNB and UE side.

For DL/UL beam prediction, gNB can use its own measurement as training/validation/testing dataset for AI/ML function. In this case, it can be considered as Cat 0 collaboration level (i.e., no collaboration between gNB side AI/ML and UE side AI/ML). Another possibility is to use beam related UE reporting quantity as one of input parameters for gNB side AI/ML in addition to gNB’s own measurement data. For example, legacy beam reporting and/or CSI reporting can be assistant data for gNB side AI/ML to predict future DL beams. Also, positioning related UE reporting can also be used for DL beam prediction, since UE position is highly related with DL beam to be served for the UE. It could be beneficial when additional UE reporting rather than legacy beam related reporting is supported for input data of gNB side AI/ML for improving the DL beam prediction performance. For example, UE data attained from sensors such as velocity, orientation, rotation, etc can be useful input for gNB side AI/ML based beam prediction. Also, more refined or detailed beam related reporting, e.g., tendency/variance of best N beam(s) and/or past/present best N beam(s) per time stamp, can be considered, which can also be used as input data for gNB side AI/ML. Furthermore, In case of UE side AI/ML, DL beam prediction in time domain can be considered as a sub-use case on AI/ML for beam management. From UE perspective, UE can collect input data for beam prediction from UE sensors, DL measurements, etc. So, if UE side AI/ML works well for beam prediction with its own data, predicted DL beam information can be reported to gNB.

Proposal #1: Consider following information as input data for AI/ML based DL Tx beam prediction in time domain.

- For gNB side AI/ML, gNB measurement (e.g., UE position/trajectory, UL measurement) and UE beam related reporting.

- For UE side AI/ML, UE own measurement (e.g., UE position/velocity, UE orientation/rotation, DL beam/CSI measurement).

In the following, we provide our view on evaluation methodology and KPI for DL beam prediction from gNB side AI/ML and UE side AI/ML as below.

- Evaluation methodology

Beam management related evaluation assumption had been set up in every release of NR. We think that conventional evaluation methodology except AI/ML function for beam prediction can be reused for LLS/SLS simulation of AI/ML for beam management. Specifically, agreed simulation assumptions in Rel-17 beam management enhancement (R1-2007151) can be reused which also includes the assumption on multi-panel UE antenna configuration.

Proposal #2: Reuse agreed simulation assumptions in Rel-17 BM (R1-2007151) except AI/ML related function for Rel-18 evaluation on AI/ML for beam management.

In [2], researches on beam prediction are introduced using convolutional neural network (CNN) and long short-term memory (LSTM). In case of gNB side AI/ML and UE side AI/ML, generally UE position and previous DL beam measurements can be used for input data as training/validation/testing dataset for AI/ML function. Also, deep Q-learning model can also be considerable for DL beam prediction, with using effective throughput reward which takes both the beam quality and the beam measurement and reporting overhead into account. Output data of gNB side AI/ML can be directly configured/indicated for a UE as a DL serving beam, and output data of UE side AI/ML can be reported to gNB for subsequent DL transmission/reception.

- KPIs/metrics on DL beam prediction

For evaluation on DL beam prediction, system/link-level throughput can be one of KPIs as general evaluation on beam management. Discussion point is that, how to compare overhead and latency of AI/ML based beam management with those of legacy beam management procedure. Also, it should be decided before evaluation whether the procedure for training/update/management of AI/ML model in gNB or UE side including data collection, model training, model validation, and model test can be considered as overhead/latency of beam management or not, and how to quantify the overhead/latency of AI/ML model.

Proposal #3: Study whether the procedure for training/update/management of AI/ML model can be considered as overhead/latency of beam management or not, and how to quantify the overhead/latency of AI/ML model.

Conclusion

In this contribution, we provided our view on the evaluation methodology and KPI on AI/ML for beam management and the followings are observed and proposed.

Proposal #1: Consider following information as input data for AI/ML based DL Tx beam prediction in time domain.

- For gNB side AI/ML, gNB measurement (e.g., UE position/trajectory, UL measurement) and UE beam related reporting.

- For UE side AI/ML, UE own measurement (e.g., UE position/velocity, UE orientation/rotation, DL beam/CSI measurement).

Proposal #2: Reuse agreed simulation assumptions in Rel-17 BM (R1-2007151) except AI/ML related function for Rel-18 evaluation on AI/ML for beam management.

Proposal #3: Study whether the procedure for training/update/management of AI/ML model can be considered as overhead and latency of beam management or not, and how to quantify the overhead/latency of AI/ML model.

Reference

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm

[2] Ma, Ke et al. “Deep Learning for Beam-Management: State-of-the-Art, Opportunities and Challenges.” (2021)