3GPP TSG RAN WG1 #109-e                                     R1-2204152

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.2.3.2

Source:		LG Electronics

Title:			Other aspects on AI/ML for beam management

Document for:	Discussion and Decision

Introduction

In the SID [1], following use cases are introduced for AI/ML for NR air interface. Especially for beam management, sub-use cases are described as beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement.

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, we discuss the potential use cases and specification impact on AI/ML for beam management.

Discussion

Beam prediction in time domain for overhead and latency reduction

Recently, AI/ML for wireless communication has been increasingly researched not only in academic area, but also in commercial area. In [2], many researches regarding AI/ML for beam management are introduced such as super-resolution beam prediction, beam tracking and predictive beam switching. From RAN1 perspective, beam prediction in time domain can be an appropriate sub-use case especially when UE mobility is predictable from gNB side, which can save UE beam reporting overhead and reduce the delay of beam measurement and reporting at UE side. In addition, beam prediction in spatial domain can also be a possible sub-use case, since it can save overhead of beam resources from both gNB and UE side considering DL/UL beam determination with less configured candidate of DL/UL beam resources. In order to investigate these potential sub-use cases on AI/ML for beam management, it can be classified as below and relevant potential spec impact can be considered.

- AI/ML at gNB side

In case of gNB side AI/ML, DL beam prediction in time domain can be considered as a sub-use case on AI/ML for beam management. For DL/UL beam prediction, gNB can use its own measurement as training/validation/testing dataset for AI/ML function. In this case, it can be considered as Cat 0 collaboration level (i.e., no collaboration between gNB side AI/ML and UE side AI/ML). Another possibility is to use beam related UE reporting quantity as one of input parameters for gNB side AI/ML in addition to gNB’s own measurement data. For example, legacy beam reporting and/or CSI reporting can be assistant data for gNB side AI/ML to predict future DL beams. Also, positioning related UE reporting can also be used for DL beam prediction, since UE position is highly related with DL beam to be served for the UE. It could be beneficial when additional UE reporting rather than legacy beam related reporting is supported for input data of gNB side AI/ML for improving the DL beam prediction performance. For example, UE data attained from sensors such as velocity, orientation, rotation, etc can be useful input for gNB side AI/ML based beam prediction. Also, more refined or detailed beam related reporting, e.g., tendency/variance of best N beam(s) and/or past/present best N beam(s) per time stamp, can be considered, which can also be used as input data for gNB side AI/ML. These enhanced and/or new type of UE reporting could be the specification impact of AI/ML for beam management. Additionally, UE UL Tx beam prediction in time domain from gNB side AI/ML can also be potential sub-use case. For non-beam correspondence UE, SRS for beam management can be used for gNB to determine gNB Rx beam (U2 procedure) and UE Tx beam with UL beam sweeping (U3 procedure). UL Tx beam prediction from gNB side can be beneficial, since predicted UE Tx beam without UL beam management process (e.g., U2/U3) may be directly configured/indicated for subsequent UL transmission for the UE, which can achieve overhead/latency reduction.

- AI/ML at UE side

In case of UE side AI/ML, DL beam prediction in time domain can be considered as a sub-use case on AI/ML for beam management. From UE perspective, UE can collect input data for beam prediction from UE sensors, DL measurements, etc. So, if UE side AI/ML works well for beam prediction with its own data, predicted DL beam information can be reported to gNB. In this case, a new reporting mode needs to be defined in specification since reporting quantity of the predicted DL beam may be different from legacy beam reporting and this reporting also should be differentiated from legacy beam reporting for gNB to understand whether the reported information is based on current beam or future beam.

Proposal #1: Consider DL Tx beam prediction in time domain with priority for sub-use cases of AI/ML for beam management.

Proposal #2: Potential specification impact for DL Tx beam prediction can be additional UE reporting which can include assistant information for gNB side AI/ML or predicted beam information from UE side AI/ML.

Conclusion

In this contribution, we provided our view on the potential use cases and specification impact on AI/ML for beam management and the followings are observed and proposed.

Proposal #1: Consider DL Tx beam prediction in time domain with priority for sub-use cases of AI/ML for beam management.

Proposal #2: Potential specification impact for DL Tx beam prediction can be additional UE reporting which can include assistant information for gNB side AI/ML or predicted beam information from UE side AI/ML.

Reference

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm

[2] Ma, Ke et al. “Deep Learning for Beam-Management: State-of-the-Art, Opportunities and Challenges.” (2021)