3GPP TSG RAN WG1 #109-e                                     R1-2204154

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.2.4.2

Source:		LG Electronics

Title:			Other aspects on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

Introduction

New SI on artificial intelligence (AI)/machine learning (ML) for NR air interface was approved and three use cases were captured in the SID as follows [1].

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

In this contribution, we discuss on other aspects on AI/ML for positioning accuracy enhancements such as representative sub use-cases that are worth considering and related potential specification impacts in terms of RAN1 point of view.

Discussions on potential sub use cases and their specification impact

Accuracy improvements

In Rel-17 positioning enhancements, reporting of path specific measurement and providing LoS/NLos indicator for first arrival path were agreed since LoS/NLos identification is critical factor for accuracy performance. In reality, the path identified as a first path may not be equal to actual first path in the multi path and heavy NLOS condition. Based on AI/ML algorithm at UE as shown in the figure 1, the FAP (e.g. first arrival path) can be selected more accurately, so that reliable value to identify LoS/NLos for the path also can be obtained.

Figure 1 An illustrative example of utilization of AI/ML for Los/NLos identification

Observation #1:

- In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE)

  - Through AI/ML algorithm, FAP (e.g. first arrival path) can be selected more accurately and then the reliable value of LoS/NLos for the path also can be obtained.

Another example of utilization of AI/ML for accuracy improvement might be for using it for configuration of muting pattern. In case of muting pattern, it was generally introduced to resolve the hear-ability problem. It seems that adaptive configuration of muting pattern based on AI/ML also can be useful for accuracy improvement. In this case, details on indication of muting pattern can be further discussed.

Figure 2 An illustrative example of utilization of AI/ML for Muting pattern

Observation #2:

- Adaptive configuration of muting pattern based on AI/ML also can be useful for accuracy improvement.

Furthermore, in the Rel-17 positioning item, the PRU (e.g. positioning reference unit) was introduced and discussed for accuracy improvement. According to common understanding, both UE and gNB can play a role as the ‘PRU’. However, there is limitation in using UE as PRU since the UE may have some mobility, but not for gNB. If the LMF can predict which UE can be used as PRU, the LMF can use the other UE dynamically as PRU to calculate the position of target UE. AI/ML also can be a one of ways to achieve it. If this sub usecase is supported, the potential specification impact will be details signalling related with measurement report or information about decision on the PRU.

Observation #3:

- If the LMF can predict which UE can be used as PRU, the LMF can use the other UE dynamically as PRU to calculate the position of target UE.

  - AI/ML can be a one of ways to achieve it.

Efficiency and power saving

From point of efficiency and power saving for positioning, AI/ML also can be considered. For example, if LMF can configure more adequate resource(s) (or set/TRP) of PRS/SRS for UE in accordance with current channel condition or other many factors that impact on positioning for target UE, there are some resource like as the time/frequency resources that are not used for positioning and it brings out advantage of resource efficiency. Furthermore, if more adequate search window such as expected RSTD/UL-AoA/DL-AoD (including each uncertainty) can be configured based on AI/ML, it also brings out overhead reduction for beamforming and provides a resource saving for measurement report of unnecessary information. From this perspective, RAN1 can use cases for positioning based on AI/ML in terms of efficiency and power saving. However, such efficiency and power saving can also applied not only positioning but also other area such as beam management, the clear scope seems necessary.

Observation #4:

- AI/ML can be used in terms of efficiency and power saving.

So far, various sub-use cases for AI/ML based positioning enhancement are discussed. According to sub use cases, AI/ML model and its evaluation can be diverge, so in order to reduce work load, prioritization of sub use cases is preferred. Among the sub use cases, accuracy improvement with clear work scope can be prioritized.

Proposal #1:

- Regarding AI/ML for positioning enhancements, accuracy improvement with clear work scope can be prioritized.

Conclusion

In this contribution, we discussed about several aspects on AI/ML for positioning enhancement, and our observations and proposals are summarized below:

Observation #1:

- In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE)

  - Through AI/ML algorithm, FAP (e.g. first arrival path) can be selected more accurately and then the reliable value of LoS/NLos for the path also can be obtained.

Observation #2:

- Adaptive configuration of muting pattern based on AI/ML also can be useful for accuracy improvement.

Observation #3:

- If the LMF can predict which UE can be used as PRU, the LMF can use the other UE dynamically as PRU to calculate the position of target UE.

  - AI/ML can be a one of ways to achieve it.

Observation #4:

- AI/ML can be used in terms of efficiency and power saving.

Proposal #1:

- Regarding AI/ML for positioning enhancements, accuracy improvement with clear work scope can be prioritized.

Reference

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm