3GPP TSG RAN WG1 #109e		R1-2204156

e-Meeting, May 9th – 20th, 2022

Source:	Panasonic

Title: 	Discussion on subband non-overlapping full duplex

Agenda Item:		9.3.2

Document for:	Discussion

Introduction

Objectives for “New SI: Study on evolution of NR duplex operation” were agreed as follows [1]. This document provides our view on subband non-overlapping full duplex.

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

Discussion

Subband allocation

In order to achieve subband non-overlapping full duplex, one discussion point would be how to allocate subband and guard band in system bandwidth (in other words, how to allocate DL/UL boundary). In the current specification, there are some schemes to split frequency domain resources, such as BWP and RB set. Thus, existing schemes can be a starting point to discuss subband allocation. At least three options i.e. BWP, RB set and PRB assignment, can be considered. It is summarized in Table 1.

BWP based

In this option, BWP is used for the subband boundary, i.e., one BWP corresponds to one subband. Assuming that only one active BWP is supported as in the specification, only one subband is supported at the same time from UE perspective. If UE needs to transmit/receive at a different subband, BWP switching is needed. Figure 1 (a) illustrates an example that only one subband is allocated for a UE. In this example, there are three subbands but only subband#0 can be used for the UE as active BWP. In this option, RF/analogue filter can be applied based on BWP as in legacy since UL and DL transmission will not be occurred at the same time in the BWP. It can suppress interference between subbands without increasing UE complexity.

RB set based

In this option, RB set is used for the subband boundary, i.e., one RB set corresponds to one subband. RB set was introduced for Rel.16 NR-U in order to split frequency resource to LBT bandwidth. The number of RBs for each RB set can be semi-statically configured. Guard bands between RB sets can also be configured to mitigate interference to adjacent RB set. These functionalities can be reused for subband non-overlapping full duplex. If multiple RB sets (subbands) are configured for a UE, scheduling flexibility can be improved compared to BWP based (e.g., resources can be assigned across multiple RB sets). Figure 1 (b) shows the case where multiple subbands can be allocated for a UE (either Tx or Rx are active).  In this example, three RB sets can be used for the UE (subband in the figure corresponds to RB set). In this option, since both UL and DL transmissions can be occurred in BWP, enhancement of CLI mitigation could be needed (e.g., RF/analogue filter when transmitting/receiving other than assigned RB sets). In addition, enhancement of slot format indication (SFI) could be needed for higher layer configured transmission/reception for each RB sets. See our accompany paper [3] for the discussion on enhancement of SFI.

PRB assignment based

In this option, PRB assignment is used for the subband boundary. In this option, UE just follows PRB assignment from gNB. UE does not know subband allocation in the cell (e.g., even if subbands and BWP are allocated like Figure 1 (b), UE does not know boundaries between subbands). Since subband allocation is not configured to UE, gNB can dynamically change subband allocation (e.g., the number of RBs for subbands and guard bands can be changed based on traffic and/or CLI among UEs). On the other hand, it would be difficult to apply RF/analogue filter for assigned PRBs. How to manage higher layer configured transmission/reception for each subband would be also a potential issue.

We propose to have further discussion on above three methods.

Proposal 1: The subband boundary method should be compared at least among BWP, RB set and PRB assignment.

For RB set and PRB assignment based option, multiple subbands can be allocated for a UE like Figure 1 (b). However, allocating only one subband would be still beneficial for low capability UE, coexistence with legacy UEs. Therefore, these aspects should also be taken into account.

Proposal 2: Further discuss the case one subband for a UE and the case multiple subbands for a UE.

Table 1 Possible schemes for subband allocation

|  | BWP based | RB set based | PRB assignment based |
| --- | --- | --- | --- |
| The boundary of subbands | Dynamically selected from UE-specific semi-static configurations | Semi-statically configured by UE-specific configuration | Dynamically indicated by PRB or PRG (UE might not be aware of boundaries) |
| Whether multiple subbands can be allocated for a UE | No | Yes | Yes |
| How dynamically indicated | Delayed by BWP switching (in the current spec) | Every resource assignment | Every resource assignment |
| RF/analogue filter when transmitting/receiving other than assigned frequency | Yes | (Needs to be clarified) | No |

Figure 1  Example of subbands and BWP

CLI handling

Inter-UE CLI handling needs to be considered as per objectives. The following possibilities can be considered.

Guard band can mitigate inter-UE CLI from adjacent subband [2]. Since the required guard band size can be different depending on CLI intensity, how to configure guard bands needs to be discussed. If subband is realized by RB set, guard band size can be configured taking into account the strongest CLI which can be caused by UEs. In this case, resources might not be used efficiently because large guard band is not always needed (e.g., if adjacent subband is the same transmission direction like DL-DL, guard band is not needed). In this sense, dynamic allocation is more efficient. For dynamic allocation, if subband is realized by PRB assignment, guard band can also be allocated by PRB assignment (i.e., unused resources can be guard band). Note that even if RB set is used, guard band can be allocated only when transmission direction of adjacent subband is different. Such operation may be seen that guard band size is dynamically indicated by gNB.

CLI measurement/repot from UE would be useful for gNB’s efficient scheduling. According to our simulation results, even if transmission direction of adjacent subband is different, guard band is not needed in some cases [2] (i.e., where reception power of desired signal is sufficiently higher than reception power of interference of adjacent subband). Thus, in order to use resources efficiently, it would be beneficial to take into account the CLI among UEs for scheduling. In the current specification, CLI-RSSI and SRS-RSPR can be used for CLI measurement/repot. However, since these are L3 report, we think to introduce L1 report of CLI would be useful. More discussion on CLI handling can be found in our accompany paper [3].

The limitation of UL transmission power should be considered. Assuming the case where DL transmission is interfered by UL transmission of adjacent subband, UL transmission power would influence CLI intensity. Thus, when UL transmission is assigned on subband boundary, to limit UL transmission power can be considered to improve DL performance.

Proposal 3: Possible schemes for inter-UE CLI handling needs to be discussed (e.g., guard band configuration/indication, CLI measurement/report, UL transmission power limitation).

Conclusion

In this contribution, we made the following proposals.

Proposal 1: The subband boundary method should be compared at least among BWP, RB set and PRB assignment.

Proposal 2: Further discuss the case one subband for a UE and the case multiple subbands for a UE.

Proposal 3: Possible schemes for inter-UE CLI handling needs to be discussed (e.g., guard band configuration/indication, CLI measurement/report, UL transmission power limitation).

- Reference

- RP-220633, “Revised SID: Study on evolution of NR duplex operation”, CMCC

- R1-2204135, “Discussion on evaluation on NR duplex evolution”, Panasonic

- R1-2204076, “Potential enhancements on dynamic/flexible TDD for subband full duplex”, Panasonic