3GPP TSG RAN WG1#109-e                                              	            R1- 2204167

e-Meeting, May 9th –20th, 2022

Agenda Item:	9.1.4.1

Source:	Lenovo

Title:	UL precoding indication for multi-panel transmission

Document for:	Discussion

Introduction

One of Rel-18 MIMO evolution WID scops is to support simultaneous multi-panel UL transmission as highlighted in the approved WID[1]:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we provide our views on the UL precoding indication mechanism for multi-panel transmission.

Discussion

Multi-panel UL transmission was firstly discussed in Rel-16 but without any agreement. In Rel-17, panel-selective based UL transmission for multi-panel UE was discussed and it was agreed that each UE can report multiple capability value sets corresponding to multiple panels that can be used for UL transmission. For some advanced device, especially for the non-power limited device, multiple panels can be simultaneously used for UL transmission when each panel has independent RF chains. As stated in the WID, up to 2 panels and 2 TRPs can be used for multi-panel UL transmission in FR2. Similar to multi-TRP based PDSCH, both multi-DCI based and single-DCI based multi-panel PUSCH transmission need to be supported targeting non-ideal and ideal backhaul assumption between two TRPs.

Both multi-DCI and single-DCI based multi-panel/TRP PUSCH need to be supported for simultaneous multi-panel UL transmission.

Two PUSCH schemes including codebook and non-codebook based PUSCH schemes are specified in Rel-15 for single-panel/TRP scenario. In Rel-17, multi-TRP based PUSCH repetition schemes were supported including both codebook and non-codebook PUSCH schemes. Therefore, both codebook and non-codebook based PUSCH schemes should be supported for simultaneous multi-panel PUSCH transmission.

Both codebook and non-codebook based PUSCH should be supported for simultaneous multi-panel PUSCH transmission.

Since the antenna port(s) for PUSCH transmission are tightly coupled to the SRS antenna port(s), enhancements on the SRS configuration to support multi-panel simultaneous operation are required. In Rel-17, in order to support multi-TRP based PUSCH repetition, two SRS resource sets for CB/nCB can be configured for a UE in a BWP of a cell, where each SRS resource set corresponds to a UE-TRP link. As indicated by the WID, up to 2 TRP and 2 panels can be supported for simultaneous multi-panel PUSCH transmission. It means that two UE panel-TRP links can be supported for simultaneous multi-panel UL transmission. Therefore, the SRS configuration for Rel-17 PUSCH repetition can be the starting point.

Enhancements on SRS for simultaneous multi-panel PUSCH transmission is needed and SRS configuration for Rel-17 multi-TRP PUSCH repetition can be the starting point.

Single-DCI based multi-panel UL transmission

Figure 1 Illustration of multi-panel PUSCH

A multi-panel multi-TRP UL scenario is illustrated in Figure 1, where two panels are equipped for the UE for simultaneous UL transmission and each panel is assumed to have four antenna ports, i.e., antenna port 1000, 1001, 1002, 1003 of one panel. A single-DCI based multi-panel/TRP PUSCH scheme is provided in Figure 1, a PUSCH with 3 layers is scheduled to be transmitted by both panel#0 and panel#1, where the first and the second PUSCH layers are transmitted by PUSCH antenna port 1000, 1001, 1002, 1003 of panel#0 to TRP#0 according to the first TCI state, and the third PUSCH layer is transmitted by PUSCH antenna port 1000, 1001, 1002, 1003 of panel#1 to TRP#1 according to the second TCI state. To better support the codebook based PUSCH transmission, two panels should have the same number of antenna ports and the same coherent capability.

The same number of antenna ports and the coherent capability for each panel is assumed for single-DCI simultaneous multi-panel UL transmission.

In Rel-17, only TDM based PUSCH schemes are supported for multi-TRP based PUSCH repetition since only one panel can be used in a given slot for UL transmission. However, for simultaneous multi-panel UL, it’s possible to support SDM and/or FDM based PUSCH schemes with multi-panel simultaneous transmission capability similar to the multi-TRP PDSCH scheme specified in Rel-16. SDM based multi-panel PUSCH can provide better UL throughput gain by exploiting the spatial domain diversity, and FDM based multi-panel PUSCH can enhance the UL reliability by exploiting both spatial domain diversity and frequency domain diversity.

Both SDM based and FDM based simultaneous multi-panel PUSCH shcemes should be supported.

Usually, the channel condition of different panel-TRP links are independent. Therefore, independent precoding procedure is preferred for each panel according to

z0P0i⋮z0Pρ-1i=W0y00i⋮y0ν-1i  , z1P0i⋮z1Pρ-1i=W1y10i⋮y1ν-1i

where

- the block of vectory00i⋯y0ν-1iT is the modulated data that will be transmitted by the first panel, and the block of vectory10i⋯y1ν-1iT is the modulated data that will be transmitted by the second panel

- W0 and W1 are the precoding matrices applied to the first and the second block of vector for codebook based PUSCH transmission, respectively.

- the block of vector z0P0i⋯z0Pρ-1iT  is the coded data to be transmitted by the antenna ports of the first panel by applying a first UL TCI state, and the block of vector z1P0i⋯z1Pρ-1iT  is the coded data to be transmitted by the antenna ports of the second panel by applying a second UL TCI state.

The precoding information for codebook based PUSCH is indicated by the TPMI field indicating a precoding matrix, while the precoding information for non-codebook based PUSCH is indicated by the SRI indicator field indicating one or more SRS resources. To support independent precoding, separate TPMI and SRI indication are required. For example, two TPMI/SRI fields can be contained in the scheduling DCI and each TPMI/SRI field corresponds to a panel.

Per-panel independent precoding information indication is required for the simultaneous multi-panel PUSCH transmission.

Another issue is the codeword-to-layer mapping rule for this scenario. In Rel-15/16/17, only one codeword is supported, since each PUSCH transmission occasion is transmitted by a panel to a TRP. However, at least for SDM based multi-panel PUSCH transmission as illustrated in Figure 1, different PUSCH layers are transmitted by different panels to different TRPs. Given that the WID restricts the total number of PUSCH layers transmitted from both panelsis no more than 4, two alternatives exist for the codeword-to-layer mapping, as follows:

Alt-1. All PUSCH layers across both panels are mapped to the same codeword.

This alternative is aligned with Rel. 15 PUSCH transmission chain with up to 4 layers mapped to the same codeword. One issue that needs further discussion under this alternative is whether/how restrictions are imposed on the number of layer pairs transmitted from both panels.

Alt-2. Two codewords with one-to-one codeword to panel mapping.

One other alternative is map the PUSCH layers from each panel to a same codeword, where each codeword has independent MCS conformed to the respective panel-to-gNB channel condition for better performance. Similarly, whether/how restrictions are imposed on the allowed rank pair values across both codewords should be discussed.

For SDM based multi-panel PUSCH transmission, down select between the following alternatives:

- Alt-1. All PUSCH layers transmitted from both panels are mapped to the same codeword. FFS: whether/how restrictions are imposed on the number of layer pairs corresponding to transmission from both panels

- Alt-2. PUSCH layers corresponding to each panel are mapped to a distinct codeword. FFS: whether/how restrictions are imposed on the rank pair values corresponding to the two codewords

As stated in WID, Rel-18 MIMO enhancement should be discussed based on unified TCI framework. One issue is how to support PUSCH transmission scheduled by DCI format 0_0 for the UE supporting simultaneous multi-panel UL transmission. For the PUSCH scheduled by DCI format 0_0 in the scenario illustrated in Figure 1, when only one UL TCI state is indicated, the PUSCH scheduled by DCI format 0_0 is transmitted by single port by applying the UL TCI state, the panel used for the PUSCH transmission is dependent on the applied UL TCI state. For example, if the indicated UL TCI state is associated with panel#1, the UE shall transmit the PUSCH scheduled by DCI format 0_0 based single antenna port of panel#1.

When two UL TCI states are indicated, the PUSCH scheduled by DCI format 0_0 is transmitted based on single antenna port with the first UL TCI state as default.

PUSCH scheduled by DCI format 0_0 is transmitted based on single antenna port with the first indicated UL TCI state when two UL TCI states are indicated.

PTRS is used for phase noise estimation in FR2 and independent phase noise estimation for different panel is expect at least for the case that the two panels used for PUSCH transmission are non-coherent (between panels). In this situation, when SDM based multi-panel PUSCH transmission is scheduled, two set of PTRS ports should be configured. For example, for the case that each panel has fully-coherent antenna ports while the two panels are mutually non-coherent, two PTRS ports can be configured, where each PTRS port is shared by the antenna ports within a panel.

Study the enhancements on PTRS to support SDM based multi-panel PUSCH transmission.

Multi-DCI based multi-panel UL transmission

For multi-DCI based multi-panel PUSCH scheduling with simultaneous multi-panel UL transmission, two PUSCH transmitted by different panels can be scheduled in a slot with overlapped resources. Different coresetPoolIndex values are configured for the CORESET in different TRPs. Each TRP is associated with a panel in FR2, as illustrated in Figure 2, TRP#0 is associated with panel#0 and TRP#1 is associated with panel#1. Therefore, each corestPoolIndex value is associated with a UE panel implicitly or explictly. When a UE supports simultaneous multi-panel UL transmission and different coresetPoolIndex values are configured for the CORESETs in the BWP of the cell, then the UE can receive two PDCCH from CORESETs configured with different coresetPoolIndex value scheduling two fully/partial/non-overlapped PUSCH transmissions or PUCCH transmissions in a slot as in Figure 2.

Figure 2 Illustration of multi-DCI based multi-panel/TRP UL transmission

When the UE is scheduled with full/partially/non-overlapped PUSCHs in time and frequency domain, the full scheduling information for transmitting a PUSCH is indicated and carried only by the corresponding PDCCH, and the UE is expected to be scheduled with the same active BWP and the same SCS. From the UE capability point of view, when the UE is scheduled with full/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. Depending on the channel condition, the two PUSCH sent from the two panels can have same or different ranks and are signaled with independent TPMIs/SRIs, although some common constraints can be applied to both of them, such as the maximal total rank.

When a UE reports a capability to support simultaneous multi-panel UL transmission and different coresetPoolIndex values are configured for the CORESETs in the BWP of the cell, the UE can receive two PDCCH from CORESETs configured with different coresetPoolIndex value scheduling two fully/partial/non-overlapped PUSCH transmissions or PUCCH transmissions in a slot.

The same active BWP and the same SCS are expected for multiple PUSCH and PUCCH transmission from multiple panels.

The two PUSCHs sent from the two panels can have same or different ranks, and are signaled with independent TPMIs/SRIs.

Conclusion

Proposal 1	Both multi-DCI and single-DCI based multi-panel/TRP PUSCH need to be supported for simultaneous multi-panel UL transmission.

Proposal 2	Both codebook and non-codebook based PUSCH should be supported for simultaneous multi-panel PUSCH transmission.

Proposal 3	Enhancements on SRS for simultaneous multi-panel PUSCH transmission is needed and SRS configuration for Rel-17 multi-TRP PUSCH repetition can be the starting point.

Proposal 4	The same number of antenna ports and the coherent capability for each panel is assumed for single-DCI simultaneous multi-panel UL transmission.

Proposal 5	Both SDM based and FDM based simultaneous multi-panel PUSCH shcemes should be supported.

Proposal 6	Per-panel independent precoding information indication is required for the simultaneous multi-panel PUSCH transmission.

Proposal 7	For SDM based multi-panel PUSCH transmission, down select between the following alternatives:

−	Alt-1. All PUSCH layers transmitted from both panels are mapped to the same codeword. FFS: whether/how restrictions are imposed on the number of layer pairs corresponding to transmission from both panels

−	Alt-2. PUSCH layers corresponding to each panel are mapped to a distinct codeword. FFS: whether/how restrictions are imposed on the rank pair values corresponding to the two codewords

Proposal 8	PUSCH scheduled by DCI format 0_0 is transmitted based on single antenna port with the first indicated UL TCI state when two UL TCI states are indicated.

Proposal 9	Study the enhancements on PTRS to support SDM based multi-panel PUSCH transmission.

Proposal 10	When a UE reports a capability to support simultaneous multi-panel UL transmission and different coresetPoolIndex values are configured for the CORESETs in the BWP of the cell, the UE can receive two PDCCH from CORESETs configured with different coresetPoolIndex value scheduling two fully/partial/non-overlapped PUSCH transmissions or PUCCH transmissions in a slot.

Proposal 11	The same active BWP and the same SCS are expected for multiple PUSCH and PUCCH transmission from multiple panels.

Proposal 12	The two PUSCHs sent from the two panels can have same or different ranks, and are signaled with independent TPMIs/SRIs.

References

- RP-213598 New WID: MIMO Evolution for Downlink and Uplink