3GPP TSG RAN WG1#109-e                                              	            R1- 2204171

e-Meeting, May 9th –20th, 2022

Agenda Item:	8.1.3

Source:	Lenovo

Title:	Remaining issues on SRS enhancements

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remaining issues on SRS enhancements.

Discussion

Collision for flexible SRS triggering

Figure 1 Collision for multiple aperiodic SRS resource sets triggered by a same DCI

In the example is given in Figure 1, a UE receives a DCI containing an SRS request field with value ’10’ and a value t of ‘2’. Three aperiodic SRS resource sets including SRS resource set#1, SRS resource set#2 and SRS resource set#3 are associated with the SRS request codepoint ‘10’. The slot offset are configured as k=0, 1, 2 for each SRS resource set. It requires that SRS resource set#1 to be transmitted in the 3rd available slot counted from slot n, SRS resource set#2 to be transmitted in the 3rd available slot counted from slot n+1 and SRS resource set#1 to be transmitted in the 3rd available slot counted from slot n+2. If the slot n+1, n+6 and n+7 are not available, SRS resource set#1 and SRS resource set#2 are transmitted in slot n+4 and SRS resource set#3 is transmitted in slot n+5. If the time location for the SRS resources in SRS resource set#1 and the SRS resources in SRS resource set#2 are overlapped in a same OFDM symbol, a dropping rule is required to the UE to select one SRS resource to transmit.

Another example for the collision for SRS resource sets triggered by different DCIs are illustrated in  Figure 2. SRS resource set#1, SRS resource set#2 and SRS resource set#3 are triggered by DCI2, and SRS resource set#4 and SRS resource set#5 are triggered by DCI1. SRS resource set#5 and SRS resource set#3 are determined to be transmitted in the same slot n+5. If the SRS resources in SRS resource set#5 and SRS resources in SRS resource set#3 are determined to be transmitted on a same symbol, a dropping rule is required as well for this case.

Figure 2 Collision for multiple aperiodic SRS resource sets triggered by different DCIs

Different solutions were proposed in the last several meetings, such as the following offline FL proposal

FL Proposal 2-1: Introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs.

- Adopt the following priority rules (with priority level from high to low)

  - Usage > CC ID > Set ID

    - For usages, priority order is AS > CB > NCB > BM

    - For CC ID/set ID, lower ID has higher priority than higher ID

- The new dropping rule is a UE optional feature

  - UE will take collision as error case if UE does not support this feature

For the proposal, it’s unclear why one usage has higher priority than another. And SRS resource set for CB and SRS resource set for nCB cannot be configured for a BWP at the same time. For simplify, the priority can be determined only based on the CC ID and the Set ID.

Proposal: Introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs.

- Adopt the following priority rules (with priority level from high to low)

  - CC ID > Set ID

    - For CC ID/set ID, lower ID has higher priority than higher ID

- The new dropping rule is a UE optional feature

  - UE will take collision as error case if UE does not support this feature

TP for partial frequency hopping

Reason for the proposes TP: It has been agreed that the candidate value of PF including 2 and 4, however, the candidate values are not well captured in TS38.211 as the other parameters.

Proposed TP for TS38.211:

-----------------------------------------------<Unchanged parts are omitted>----------------------------------------------- - 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF∈{2,4} is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6. The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise noffsetFH=b=0BSRSmSRS,bNscRBnb noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF and - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0. -----------------------------------------------<Unchanged parts are omitted>-----------------------------------------------

Conclusion

Proposal: Introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs.

- Adopt the following priority rules (with priority level from high to low)

  - CC ID > Set ID

    - For CC ID/set ID, lower ID has higher priority than higher ID

- The new dropping rule is a UE optional feature

  - UE will take collision as error case if UE does not support this feature

Proposed TP for TS38.211:

-----------------------------------------------<Unchanged parts are omitted>----------------------------------------------- - 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by Msc,bSRS=mSRS,bNscRBKTCPF where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF∈{2,4} is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6. The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise noffsetFH=b=0BSRSmSRS,bNscRBnb noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF and - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0. -----------------------------------------------<Unchanged parts are omitted>-----------------------------------------------

References

- Chairman Notes of RAN1#108e

- TS38.211 v17.1.0