3GPP TSG-RAN WG1 Meeting #109-e	R1-2204172

# e-Meeting, May 9th - 20th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Configuration Index field in DCI format 3_0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS38.214, it is specified that the "Configuration index" field of the DCI format 3_0, if provided and not reserved, indicates the index of the sidelink configured type 2. According to TS38.212, if the DCI format 3_0 with CRC scrambled by SL-CS-RNTI schedules a retransmission of SL CG, i.e. NDI=1, the “Configuration index” field is provided and not reserved. According to clause 5.22.1.1 of TS38.321, it is specified that determination of the SL grant as retransmission of SL CG is based on the idenfified HARQ process ID in DCI format 3_0 that has been set for an activated SL CG, rather than based on the “Configuration index” field. Furthermore, it is ambiguous whether the scheduled resource(s) indicates the recurring resources of the SL CG indicated by the “Configuration index” field or the resources indicated by TDRA/FDRA in the DCI format 3_0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that for DCI format 3_0 with CRC scrambled by SL-CS-RNTI wherein NDI field is set to ‘1’, the “Configuration index” field is reserved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the UE determines the SL resources for retransmission of SL CG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

#### 7.3.1.4	DCI formats for scheduling of sidelink

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Resource pool index –log2I  bits, where I is the number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling.

-	Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214]

-	HARQ process number – 4 bits.

-	New data indicator – 1 bit.

-	Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause 8.1.2.2 of [6, TS 38.214]

-	SCI format 1-A fields according to clause 8.3.1.1:

-	Frequency resource assignment.

-	Time resource assignment.

-	PSFCH-to-HARQ feedback timing indicator –log2Nfb_timing bits, where Nfb_timing is the number of entries in the higher layer parameter sl-PSFCH-ToPUCCH, as defined in clause 16.5 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213].

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI and for DCI format 3_0 with CRC scrambled by SL-CS-RNTI wherein the New data indicator is set to ‘1’.

-	Counter sidelink assignment index – 2 bits

-	2 bits as defined in clause 16.5.2 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic

-	2 bits as defined in clause 16.5.1 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static

-	Padding bits, if required

If multiple transmit resource pools are provided in sl-TxPoolScheduling, zeros shall be appended to the DCI format 3_0 until the payload size is equal to the size of a DCI format 3_0 given by a configuration of the transmit resource pool resulting in the largest number of information bits for DCI format 3_0.

If the UE is configured to monitor DCI format 3_1 and the number of information bits in DCI format 3_0 is less than the payload of DCI format 3_1, zeros shall be appended to DCI format 3_0 until the payload size equals that of DCI format 3_1.

<Unchanged parts are omitted>